Delivering Sustainable Public Transport: The Case of the Better Bus Area Fund

Workshop 6. Delivering Sustainable Public Transport

Yena Song*, Adrian Hickford and John Preston.

Faculty of Engineering and the Environment, University of Southampton, UK.

* Corresponding author: Transportation Research Group, Civil, Maritime and Environmental Engineering and Science, Faculty of Engineering and the Environment, University of Southampton, Highfield, Southampton, SO17 1BJ, UK.

Current address: Global Cooperation Division, Green Technology Center, 173 Toegye-ro, Jung-gu, Seoul, 100-705, South Korea. Tel: +82 (0)2 3393 3923. Email: yena@gtck.re.kr

Keywords

Buses

Sustainability

Technology

Behaviour Change

JEL codes: R40, R48, R53

Abstract

Transport for South Hampshire, a joint board responsible for transport planning for a conurbation of over one million people, successfully bid for the UK’s Better Bus Area Fund. This will result in investments of £7.4 million over the period 2012 to 2014, of which £4.5 million will come from central government. Investments will include installation of Wi-Fi on vehicles, Near Field Communication (NFC) tags and Next Stop Displays and Announcement systems, along with marketing initiatives, the development of mobile apps and the refurbishment of buses. These investments will be complemented by other initiatives in the conurbation, not least some £27 million of investments from the Local Sustainable Transport Fund over the period 2012 to 2015, including the development of key interchange points and bus corridors.

The University of Southampton has been commissioned to monitor and evaluate these investments. This paper will outline the methodology that has been developed and will detail the baseline surveys that have been undertaken, the anticipated impacts on attitudes to and usage of bus services and the results of the initial implementation of the investment programme. The extent to which such investments can revitalise bus services in an area where there has been a long run historic decline will be assessed.

- Introduction

The aim of this paper is to examine the scope for technological solutions to revive the local bus market in an area that has seen a long term decline in passengers. The area in question is South Hampshire, the largest conurbation in the South of England, with a population of over 1 million and an area of 1,848 km2. The main centres are Southampton in the west (population 236,900) and Portsmouth in the east (population 205,100).  The conurbation also includes Eastleigh (125,200), Havant (120,700), Fareham (111,600) and Gosport (82,600) Districts and part of the East Hampshire, New Forest, Test Valley and Winchester Districts (combined population in South Hampshire 167,000) (see also Figure 1).

Transport is managed by a Joint Committee of the three Unitary Authorities (Hampshire, Southampton and Portsmouth) and the eight Districts, called Transport for South Hampshire (TfSH). Over 3.2 million person trips start and/or finish in the TfSH area each day, of which 2.8 million are wholly contained within the TfSH area. The modal shares are car 70%, cycling and walking 25% and public transport only 5% (TfSH, 2012).

In order to increase this low bus market share and to reduce the use of private cars, TfSH successfully bid to the UK’s Better Bus Area Fund. The details of this bid are given in section 2.  In section 3, trends in bus use in South Hampshire will be outlined. In section 4, details of some primary data collection will be given, with a particular emphasis on focus groups with non-bus and with bus users. Lastly, in section 5 some conclusions will be drawn and the implications for further research examined.

Figure 1: The Transport for South Hampshire Area.

- The Better Bus Area Fund (BBAF)

A bid was made to the BBAF in February 2012 to cover funding for the period from April 2012 to March 2014. This resulted in an investment of £7.4 million, of which £4.5 million will come from central government. Investments will include installation of Wi-Fi (£1.2 million) and Next Stop Displays and Announcements on 500 vehicles (£0.6 million) and Near Field Communication (NFC) tags at 4,500 bus stops. In addition, there will be marketing initiatives based around the My Journey brand (£1.5 million), the development of mobile apps (building on work undertaken by University of Southampton Computer Science students), the refurbishment of buses (LED lighting on 500 buses and internal and external refurbishment of 140 buses - £1.8 million) and a bus priority scheme in Gosport (Rowner Roundabout - £1.1 million). There is also some budget for staff training and for monitoring.

These investments will be complemented by other initiatives in the conurbation, not least some £27 million of investments from the Local Sustainable Transport Fund (LSTF) over the period 2012 to 2015, including the development of key interchange points and bus corridors, along with the introduction of Smartcards (in autumn 2014) and the roll-out of Personalised Travel Planning and Workplace Travel Plans. Similar measures are being implemented in the Southampton and Portsmouth LSTF programmes over the same period (with further investments of almost £9 million).

However, there is relatively little evidence on how such technological improvements impact on bus demand and on attitudes to bus use. Balcombe et al. (2004, Chapter 8) give valuations in terms of pence or in-vehicle time minutes per trip of measures such as CCTV and real time information provision. Currie and Wallis (2008, Table 2), using evidence from Australia, inferred that Next Stop Displays could increase patronage by 0.34%, whereas on-vehicle CCTV and related security measures could increase patronage by 1.19%. The TfSH submission refers to the experience of First Group in Tavistock who found Next Stop Displays increased patronage by 10%. It also makes use of a report by the 10 Percent Club (2006) which details the demand impacts of bus improvements in nine locations, although many of the upgrades are more comprehensive than those proposed for South Hampshire. Currie and Wallis (op cit., Tables 4 and 5) cite examples of bus improvements in Australia which, excluding free fares schemes, have increased route patronage by between 10% and 70%, and by between 2% and 53% in Europe, including a 5% increase in Southampton related to real time information (although the system on which this was based, Romanse, has been withdrawn, and replaced by Citywatch).

TfSH forecast that these measures will lead to an 8% increase in bus patronage and 5.6% increase in public transport’s modal share. Assuming, the volumes of all other modes are kept constant and a base public transport mode share of 5%, this suggests a bus modal share of 3.5% in the base.  The BBAF package is forecast to increase annual bus boardings in 2014 from a do-nothing reference case of 37.8 million to 40.8 million.

- Trends in Bus Use in South Hampshire

The bus market in South Hampshire is dominated by three of the ‘big five’. First Group, who acquired the former Municipal operators in Portsmouth and Southampton and the former National Bus Company subsidiary in Fareham and Gosport provides 48% of service. The Go Ahead Group, which operates Blue Star and Unilink services, developed from a combination of new starts and the acquisition of the National Bus Company depot in Eastleigh, has 31% market share. Stagecoach acquired two National Bus Group depots in Portsmouth and Winchester and has an 11% market share. An independent operator, Velvet, based in Eastleigh, has a 4% market share, with other operators having a 6% share. The four largest operators have formed the South Hampshire Bus Operators Association (SHBOA).

Data on bus usage by Unitary Authority level is shown by Table 1.  Hampshire has a 2011 population of 1,317,700, of which 607,100 (46%) live in South Hampshire. It should be noted that since April 2006 a free fare concession for bus use has existed in England for the over 60s and eligible disabled people. This statutory concession operates between 9:30am and 11:00pm Monday to Friday and all day on Saturdays and Sundays and originally covered travel within a Travel Concessions Authority (TCA).  In April 2008, a national scheme was introduced which extended free travel for concessionaires to any journey on a local bus in England (Baker and White, 2010).

| Year | 04/05 | 05/06 | 06/07 | 07/08 | 08/09 | 09/10 | 10/11 | 11/12 | 12/13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Southampton | 18.9 | 18.4 | 19.3 | 19.7 | 19.8 | 18.6* | 18.0 | 18.2 | 17.8 |
| Portsmouth | 11.4 | 11.0 | 11.3 | 11.7 | 11.9 | 10.5 | 10.8 | 10.9 | 10.1 |
| Hampshire | 27.1 | 26.7 | 28.2 | 29.8 | 30.5 | 29.9 | 30.9 | 31.8 | 32.1 |
| Total | 57.4 | 56.1 | 58.8 | 61.2 | 62.2 | 59.0 | 59.7 | 60.9 | 60.0 |

*Difference between 08/09 and 09/10 in Southampton is in part due to the change of data collection methodology.

Table 1: Passenger journeys on local bus services (in millions)

Data source: 2013 Bus statistics, DfT

It can be seen that overall bus usage has fluctuated, with growth between 2005/6 and 2008/9 of 11%, associated primarily with the extension of concessionary fares schemes, but with some retrenchment since then, particularly in Southampton.

The implied bus trips per capita in 2012 is 74.0 per annum in Southampton, compared to 48.8 in Portsmouth and 24.1 in Hampshire, which is much more rural than the other two areas. Portsmouth is relatively densely populated with relatively flat topography and therefore active travel, i.e. walking and cycling, has a relatively high market share.

Figure 2: Annual average number of bus journeys per person

Extensive market research carried out in 2011 and 2012, confirms that the frequency of bus use in Portsmouth and Southampton is much higher than the rest of South Hampshire (Figure 3).

Figure 3: Frequency of bus use by area

Data source: Telephone survey data (MRUK for SCC, ICM for Portsmouth and TfSH) Sample sizes 1,500 for the MRUK survey and 2,300 for ICM.

Figure 4, which is based on runs of the Transport for South Hampshire Sub Regional Transport Model (SRTM) indicates that, under the do-nothing reference case, bus boardings are expected to fluctuate, passenger kms on bus are expected to decline over the period. There appear to be two exceptions. The Shirley corridor in west Southampton is forecast to have demand growth, in part reflecting the current high level of service (57 buses an hour in the peak) due to competition between First and Go-Ahead, and the Gosport – Fareham corridor where a Bus Rapid Transit (BRT) system entitled Eclipse has been in operation since April 2012. In its first year of operation Eclipse has increased public transport operation in Fareham and Gosport by 11%, with usage on the two bus services most affected increasing by 64%.

Detailed data on modal splits are difficult to obtain. Southampton City Council undertake an annual central area cordon count during the AM peak time, which shows that bus’s share of passenger trips was 17.1% in 2006, but had declined to 15.9% by 2012. By contrast, rail had increased from 4.9% to 6.5% over the same period. The Census has data on the Journey to Work where the bus’s share for Southampton was 11.4% in 2001, declining to 9.1% in 2011. For Portsmouth, the decline was from 8.6% in 2001 to 7.3% in 2011.  In both cities, rail’s share had increased  - from 2.0 to 2.8% in Southampton and from 2.4 to 3.5% in Portsmouth.

The iConnect project has collected detailed travel diary data in Southampton (Song et al., 2013). A longitudinal panel of 362 respondents in 2010, 2011 and 2012 indicated a reduction in bus share of weekly passenger kms from 7.3% to 2.8%, whilst rail’s share increased from 13.6% to 14.7%. The relatively high rail share represents the impact of long distance commuting to London. The sharp decline in bus age may reflect a cohort ageing effect, along with targeted improvements in walking and cycling facilities.

Figure 4 Forecast of Bus Usage in South Hampshire 2010 to 2026.

Source: TfSH, 2012.

Bus patronage figures have been provided by the three main bus operating companies: Bluestar, First Group and Stagecoach. These are commercially sensitive data, but these analyses do not identify individual routes or bus companies. The data is aggregated by the nine corridors that TfSH have targeted for improvements using the LTSF, as shown in Table 2.  The nine corridors are shown in Figure 5 and the preliminary analysis of the patronage is shown in Figure 6.

| Corridor | First Soton | First G+H | First P'mouth** | Bluestar | Stagecoach P+H |
| --- | --- | --- | --- | --- | --- |
| W of Soton* (1, 2, 3) | 1, 2, 2A, 10, 11, X2, S1 |  |  | 4, 8, 9, 11, 12, 18 |  |
| N&E of Soton (4, 5, 6) | 4, 4A, 6, 7, X4 |  |  | 1, 2, 3 |  |
| Gosport & Portsmouth (7, 8, 9) |  | 9, 9A, 10, 11, E1, E2 | 3, X4, 7C, Zip8, X8, X9, 18, X40, X41 |  | 20, 21, 23, 700 |

Table 2: Bus services travelling along each corridor group

Note: Time-series data is currently only available for Bluestar and Stagecoach (First Group have provided data for 2012 only).

* Soton is the abbreviation of Southampton.

** P’mouth is the abbreviation of Portsmouth.

Figure 5: The Nine LSTF Intervention Corridors in South Hampshire.

Source: TfSH, 2011.

Figure 6: Bus patronage trends

Data source: bus operators

The corridors from the west into Southampton have seen a steady growth in passenger numbers since 2008, while those from the northeast and east saw growth in 2009 and 2010, and have recovered further growth in 2012 after a slight drop in passenger numbers in 2011.

Note that data from First Group is not yet incorporated into these figures – so some of these changes will reflect shifts between operators. However, it does indicate that growth is possible for some operators on some corridors.

- Case Studies in South Hampshire

4.1 Quantitative Study

The University of Southampton has been commissioned to monitor and evaluate the impact of the BBAF investments in South Hampshire. This has involved a systematic series of surveys based at key locations. Emphasis has been placed on four MOSAIC groups, two that attitudinal and behavioural surveys indicated exhibit a high propensity to change transport modes (e.g. Group H: Couples and Young Singles in Smaller Modern Starter Homes, Group L: Active Elderly in Pleasant Retirement Locations) and two with a low propensity to change (e.g. Group F: Couples with Young Children in Comfortable Modern Housing, Group M: Elderly People on State Support). The location of these groups is shown by Figure 7.

Based upon the spatial distribution of the target groups, level of bus use and consideration of diversity in study sites, four sites were selected for bus surveys. These areas are Shirley High Street in Southampton, London Road in Portsmouth, Eastleigh Bus Station and Fareham Bus Station.

Figure 7: Key MOSAIC Groups and Survey Sites in South Hampshire.

Initially, due to limited budgets, the surveys were to be undertaken online, but subsequently effort has switched to postal self-completion surveys. At each bus station/stop, two or three researchers distributed survey cards to those who were waiting for the bus as the pilot study indicated that passengers alighted from a bus tended not to respond to the invitation. Criteria of selecting target passengers were 1) being adults (over 16 years old); 2) understanding the rationale behind the study; and 3) communicating in English. Those who refused to take the cards or who were unable to consent for themselves were excluded. Based on those criteria, the researchers tried to hand out as many survey cards as possible.

Two waves of surveys were planned for the quantitative study and surveys in 2013 formed the baseline during which only a small part of BBAF schemes were implemented, i.e. 70% of free on-board internet installation, 3% of internal bus refurbishment, 3% of internal LED lighting, real-time bus and test of real-time display system. At the similar time in 2014 a follow-up survey will be carried out to find the impacts of the BBAF schemes on passenger behaviours and perspectives on the local buses. The market research undertaken in the baseline is given in Table 3.

| No | Date | Time | Location | Sample Size | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | Monday 26/11/2012 | 08:00 - 15:00 | Southampton: Highfield Interchange | 400 invitation cards | Online survey (A pilot testing online survey tool) |
| 2 | Monday 04/02/2013 | 08:00 - 15:00 | Fareham : Fareham Bus Station | 130 invitation cards | Online survey |
| 3 | Thursday 14/02/2013 | 09:30 – 12:30 | Southampton: Shirley High Street | 50 invitation cards | Online survey |
| 4 | Monday 25/03/2013 | - | LSTF Facebook and Twitter | - | Online survey site promotion |
| 5 | Friday 10/05/2013 | 08:00 - 16:00 | Eastleigh: Eastleigh Bus Station | 110 survey cards | Online and card versions |
| 6 | Monday 03/06/2013 | 08:30 - 15:30 | Fareham: Fareham Bus Station | 200 survey cards | Online and card versions |
| 7 | Friday 21/06/2013 | 09:30 - 16:00 | Portsmouth: London Road Gunwharf Quay Bus Station | 92 survey cards | Online and card versions |
| 8 | Thursday 18/07/2013 | 09:30 - 13:00 | Southampton: Shirley High Street | 200 survey cards | Online and card versions |

Table 3: BBAF Monitoring – Surveys to date.

Excluding the pilot, 180 contacts only elicited 17 online responses for the first round of surveys from 4th February 2013 until mid-March 2013, representing a response rate of 9.4%. By contrast, 602 contacts have elicited 239 postal responses between 10th May 2013 and 29th July 2013, representing a response rate of 40%. Along with the low response rate, the online survey-only approach significantly limited the sample size as many bus users do not have access to the Internet so the invitation could not be made to them. So we provided two formats for survey from May onwards giving online and hard copy options to the potential respondents.

From card and online surveys we collected usable data from 268 bus users. Due to the small sample size we did not apply a calibration weight or impute the missing values. Compared to the sample profiles of Passenger Focus surveys (Table 4), our respondents showed similar gender and age distribution to Hampshire’s survey results in 2010 but different from Southampton’s, which reflects our survey targets across the South Hampshire area. It should be noted that people who are 60 years old or over can travel for free by local buses.

|  | Passenger Focus* |  | BBAF Survey |
| --- | --- | --- | --- |
|  | Southampton (2009) | Hampshire (2010) | Card survey (2013) |
| Age groups |  |  |  |
| 16-25 | 18.0 | 12.0 | 10.1 |
| 26-34 | 14 | 8 | 7.5 |
| 35-44 | 9 | 7 | 7.8 |
| 45-54 | 11.0 | 11.0 | 7.5 |
| 55-59 | 7 | 7 | 6.8 |
| 60 + | 39 | 52 | 57.5 |
| Missing | 2 | 0 | 1.1 |
| Gender |  |  |  |
| Male | 32 | 31 | 31.0 |
| Female | 67 | 65 | 67.9 |
| Missing | 2 | 4 | 1.1 |

Table 4: Age Groups of Survey Respondents

* Figures from Passenger Focus reports were weighted.

The majority of respondents used buses for shopping and personal business accounting for about 60% of journey purposes on the day of survey.  Journeys with time constraints such as commuting for work and school or business journeys, accounted for about 33% of bus journeys (Figure 8). This partly reflects the bus market: concessionary passholders who are less likely to work or study due to their age or health, account for greatest share of the market. Most of the respondents (96%) used the bus more than once a week and 30% used it almost every day. The average number of bus journeys made by the respondents was 6.72 a week (average figure from card data only due to different format of questions) and this is over one third of the average number of trips per week by all modes according to the National Travel Survey 2012 figure.

Figure 8: Journey purposes

Convenience was the most frequently noted reason for using local buses and long distance for walking and cycling followed behind. No other available options was the third most frequently reported reason for choosing local buses. Though environmental concern and health are often emphasised in promotion of bus uses by local authorities and researchers, only 3.7% and 7.1% of respondents identified these as their reasons to choose local buses for travel.

Bus passengers were asked to indicate how much they agree with eight statements regarding the local bus services in South Hampshire. These are to measure the level of customer satisfaction in eight distinctive areas, i.e. ①accessibility, ②routes, ③reliability, ④multimodal connectivity, ⑤convenience, ⑥journey time, ⑦fare and ⑧access to bus information (Figure 9).

Overall over 60% of respondents strongly or somewhat agreed with the statements shown in the survey suggesting that they were generally satisfied with the services. Most satisfactory was the location of bus stops, while the accessibility to local amenities was ranked second. The least satisfactory area was the bus fare and reliability, punctuality and multimodal connectivity followed after it. Our satisfaction statistics appear lower than Passenger Focus in 2010 (overall 90%, Punctuality 74%, Value for Money 55%) although the same wordings or questions were not used in both instruments.

Figure 9: Service satisfaction levels

Additionally various suggestions were made by the respondents. The most frequently commented areas for further improvement were 1) frequent bus services 2) lower ticket prices 3) reliability and punctuality and 4) provision of convenient routes.

  - Qualitative Study

In addition, three focus groups, two for non-bus users and one for bus users, were convened in Southampton in summer 2013. Participants were recruited through advertisements on local papers, local governments’ social networking sites, University website and email contacts by the city council. We targeted the general adult population who had interest in the local bus services and were willing to talk about their views and thoughts on it. Our call resulted in 141 people registered for the study and among them, 12 people were invited to each focus group meeting assuming 30% drop-out. Participants were selected to ensure diversity. The selection criteria were mainly age and gender and any further information, such as workplace or home address, provided voluntarily was also considered in the selection process.  In the non-bus users meetings, seven and ten participants joined and seven bus using participants also joined the meeting, suggesting greater drop-outs than anticipated at the last moment.

    - Non-Bus Users’ Focus Group

The non-bus users generally had some experience of using buses, with their last trip being between three weeks and 15 years ago. The main barriers to use related to inadequacies of service and expensive fares. The non-bus users were aware that there had been recent service reductions, as a result of funding cuts (Southampton City Council had recently cut annual bus subsidy by £400K), and some had switched away from bus when direct services had been withdrawn. There were also concerns about the lack of service after 6pm. Buses were perceived to be slow, due to indirect routings and lack of priority so that buses were never faster than the car. Fares were perceived to be expensive, particularly where the journey required a change of buses and hence two tickets.  Buses were also perceived to perform poorly in terms of reliability, comfort, cleanliness and staff courtesy, with one operator attracting particular criticism. There was one positive comment regarding the improvement in bus layouts so that they can now accommodate wheelchairs and pushchairs, although we also received a detailed written submission regarding the difficulties that remain for wheelchair users of bus services.

Non-bus users went to extraordinary lengths to avoid using buses – I plan my whole life around not using buses. Although many non-bus users were motorists, a significant number were also walkers and cyclists – with the latter group being particularly committed – I prefer the bike even in the rain to being dry on the bus.  Many non-bus users did use the train for longer journeys and found trains generally easier to use than buses in terms of knowing where to get on and off. Internet shopping was being used to reduce the need to make shopping trips by bus.

There was resistance to making other modes less attractive to encourage people to use the bus – parking was already seen as expensive. There were suggestions about how buses could be improved, related to developing integrated services (including better coordination with ferries, trains and school opening times) and fare systems. There was knowledge of public transport in London, continental Europe and in Hong Kong, with support for Oyster/Octopus Smart Cards, time delimited tickets and Group discounts. There was familiarity with bus based park and ride in nearby cities (Salisbury, Winchester) and it was felt that such a service should be provided in Southampton. There was some support for express bus services on the main corridors and feeder services to link into these services. There was an awareness that real time information systems had existed in Southampton but had been withdrawn and there was scepticism surrounding the veracity of the system that was deployed. Some saw a conflict between the public service role and the private provision of buses but there were disagreements as to whether the Council run services (privatised in 1993) were better than what was currently offered.

Non-bus users saw the BBAF measures as gimmicks. They were particularly sceptical about the on-board WiFi which was seen as ridiculous in the age of Smart Phones, and there was a reluctance to use laptops on buses, echoing the findings of Russell et al. (2011) in New Zealand who found that less than 1% of bus users used a computer during their journey. Smartcards were thought to be useful if they gave you the best price, as was Real Time Information if it was accurate. Text messaging services were not thought to be useful, although there was some support for a Transport for London style App.

    - Bus Users Focus Group

Bus users, all of whom used service at least 2 to 3 time a week, with many using buses 5 or more days per week had similar viewpoints to the non-bus users, but often from a more informed position. There was concern about various aspects of service deterioration, including withdrawal of services  - one corridor (Hill Lane) was mentioned where services had gone down from four to no buses an hour, with services being rerouted down an already heavily bussed corridor (Shirley High Street). There were also issues about the fares, related to the withdrawal of transfer and discounted tickets, the restrictions imposed by operator-specific ticketing and inconsistencies in the fare scales, and to unreliability and bus bunching.

Few of the participants were captive to bus – as some were also motorists and cyclists, most walked and used the train for some journeys. There was not support for penalising car users and some concerns about proposals to introduce parking charges after 6pm.

In terms of improving services the key was thought to be journey time – when you use a bus, everything takes a day. Bus priority was seen to be important, with reallocation of road space suggested on key routes to key destinations (e.g. Southampton General Hospital). The improvements related to low floor buses and dropped kerbs had brought to people with restricted mobility was acknowledged. Bus fares were acknowledged to vary between companies but were perceived as expensive (as were rail fares) but, due to concessions, it was noted that very few people paid fares outside the rush hours. There was a recognition that some bus stops (but by no means all) had been improved and that we do not have a bus station any more.

A number of bus users indicated that they made use of WiFi on buses and had used mobile phone Apps and web based travel planners. The biggest difference between bus users and non-bus users was with respect to the BBAF measures. These were generally supported by bus users as all being very good and what we have been talking about. There was particular support for Next Stop Displays and Smartcards (particularly if they were multi-functional as in Hong Kong), with NFC tags seen as least useful. Apps were seen as useful, although those currently available were not seen as user friendly, taking several minutes to load, and interactive maps were seen as difficult to use. It was noted that a current operator-specific Smartcard, the Key, took 48 hours for payments to be uploaded if you paid on-line.

- Conclusions and Future Prospects

The prospects for bus transport in South Hampshire appear unpromising. Volumes have returned to long term decline, following a brief increase between 2005 and 2008 as result of the extension of concessionary fares. Market shares have consistently declined.

The Focus Groups indicated that the bus industry in Southampton was behind internationally and characterised by poor service levels, high fares and a lack of integration. This has resulted in a fragmented network, without any focal points (see also Figure 10).

Figure 10: Southampton Bus Network

Non-bus users were sceptical as to whether technological improvements would make a difference. Bus users were more encouraging – but they only constitute an estimated 23% to 37% of the adult population (based on Table 2, defining bus users who use buses at least once a week).

Both bus users and non-bus users recognised that there were weaknesses in the current bus offering in terms of frequency, speed, cost and reliability. In written correspondence, on bus user summarised this as regularity, relevance and reliability. There seemed to be a consensus that for bus services to be used more frequently improvements to these ‘hard’ factors are required. Bus priority measures such as those introduced in Gosport and changes to the fare system and structure stimulated by Smartcards might offer sufficient conditions for growth, but there are institutional barriers to such developments. The ‘softer’ technological and marketing measures might be seen as necessary, but not on their own sufficient, conditions for growth. Young bus users would appreciate technological improvements but such soft measures on their own were highly unlikely to attract non-users to use local buses. Interestingly one Focus group speculated that airline style alliances between operators might lead to a more integrated service.

Future work will monitor the impact of the BBAF and the related LSTF measures to determine whether the interventions can change attitudes and behaviour towards buses.

Acknowledgement

This work has been funded by Transport for South Hampshire and has also benefitted from involvement in the Centre for Sustainable Travel Choices, a partnership between Southampton City Council, Sustrans and the University of Southampton. However, the views expressed in this paper are solely those of the authors.

References

Baker, S. and White, P. (2010) Impacts of Free Concessionary Travel: Case Study of an English Rural Region. Transport Policy, 17, 1, 20-26.

Balcombe, R. (Ed) et al. (2004) The Demand for Public Transport: A Practical Guide. TRL Report 593. TRL, Crowthorne.

Currie, G. and Wallis, I. (2008) Effective Ways to Grow Urban Bus Markets – A Synthesis of Evidence. Journal of Transport Geography, 16, 6, 419-429.

DfT (2013a) Local area walking and cycling in Englad, 2011/12, Department for Transport. https://www.gov.uk/government/publications/local-area-walking-and-cycling-in-england-2011-12

DfT (2013b) National Travel Survey 2012, Department for Transport

Passengerfocus (2009) Bus Passenger Survey – Southampton – Total, Pilot Study – April/May 2009.

Passengerfocus (2010) Bus Passenger Survey Full Report. July. Passengerfocus, London. http://www.passengerfocus.org.uk/research/publications/bus-passenger-survey-full-report-july-2010

Passengerfocus (2013) Bus Passenger Survey. March. Passengerfocus, London. http://www.passengerfocus.org.uk/research/publications/bus-passenger-survey-full-report-march-2013

Russell, M., Price, R., Signal, L. and Stanley, J (2011) What Do Passengers Do During Travel Time? Structured Observations on Buses and Trains. Journal of Public Transportation, 14, 3, 123 -146.

Song, Y., Brand, C and Preston, J. (2013) What explains active travel behaviour? Evidence from case studies in the UK.  Environment and Planning A, forthcoming.

TfSH (2011) A Better Connected South Hampshire. Volume 1: The Five Business Cases. DfT Local Sustainable Transport Fund Bid. December. TfSH, Winchester.

TfSH (2012) Your Journey: Making Travel Time Your Time. DfT Better Bus Area Fund Bid. February. TfSH, Winchester.

The 10 Percent Club (2006) Routes to Revenue Growth: the message from nine successful bus services. Landor Books, London.

In 2013 TfSH was joined by the Isle of Wight but for the purposes this paper we will refer only to the original TfSH.

See, for example, http://www.myjourneysouthampton.com/

According to the DfT’s local area walking and cycling statistics in 2010/11 (DfT, 2013a), Southampton had a lower proportion of people who cycle regularly than Portsmouth. In Southampton 14% of residents answered that they cycled at least once a month and 91% walked at least once a month whilst Portsmouth’s figures were 20% for cycling and 93% for walking.

http://www.firstgroup.com/ukbus/hampshire/travel_news/news_initiatives/?item=10411&conf=0

Researchers observed that young males were found to be the most difficult group to penetrate as they constitute a small proportion of bus passengers and were more likely to refuse to take the survey cards. On the other hand, elderly females were more likely to respond to our invitation positively: they often finished the cards during their wait and discussed issues with our researchers.

Hampshire’s bus passenger survey in 2010 (Passengerfocus, 2010) showed that just over 50% of bus users were fare payers and more recently the survey on the Gosport-Fareham corridor in 2013 (Passengerfocus, 2013) showed only 31% of the Bus Rapid Transit users were paying the fare.

According to the National Travel Survey 2012 (DfT, 2013b), people make 954 trips per person per year in the U.K., and this indicates roughly 18.3 trips per person per week.

http://www.dailyecho.co.uk/news/10137835.Bus_passengers_face_cuts_to_services/