1

- Global coastal wetland change under sea-level rise and related

- stresses: the DIVA Wetland Change Model

4

- Thomas Spencera*, Mark Schürchb, Robert J. Nichollsc, Jochen Hinkeld, Daniel Lincked, A.T. Vafeidisb,

- Ruth Reefa, Loraine McFaddene, and Sally Brownc

7

- aCambridge Coastal Research Unit, Department of

- Geography, University of Cambridge, Downing Place,

- Cambridge CB2 3EN, UK

11

- bGeographisches Institut, Ludewig-Meyn-Str. 14

- 24098 Kiel, Germany

14

- cFaculty of Engineering and the Environment,

- University of Southampton, Highfield,

- Southampton SO17 1BJ, UK

18

- dGlobal Climate Forum e.V. (GCF)

- Neue Promenade 6

- 10178 Berlin, Germany

22

- eFlood Hazard Research Centre, Middlesex University,

- The Burroughs Hendon London NW4 4BT, UK

25

26

27

28

29	*Corresponding author 30	Tel: 0044 (0)1233 333399 31	Fax: 0044 (0)1223 333392

32	Email: ts111@cam.ac.uk (T. Spencer)

33

34	Abstract

35

- The Dynamic Interactive Vulnerability Assessment Wetland Change Model (DIVA_WCM) comprises

- a dataset of contemporary global coastal wetland stocks (estimated at 756 x103  km2  (in 2011)),

- mapped to a one-dimensional global database, and a model of the macro-scale controls on wetland

- response to sea-level rise. Three key drivers of wetland response to sea-level rise are considered: 1)

- rate of sea-level rise relative to tidal range; 2) lateral accommodation space; and 3) sediment

- supply. The model is tuned by expert knowledge, parameterised with quantitative data where

- possible, and validated against mapping associated with two large-scale mangrove and saltmarsh

- vulnerability studies. It is applied across 12,148 coastal segments (mean length 85 km) to the year

- 2100. The model provides better-informed macro-scale projections of likely patterns of future

- coastal wetland losses across a range of sea-level rise scenarios and varying assumptions about the

- construction of coastal dikes to prevent sea flooding (as dikes limit lateral accommodation space

- and cause coastal squeeze). With 50 cm of sea-level rise by 2100, the model predicts a loss of 46 –

- 59% of global coastal wetland stocks. A global coastal wetland loss of 78% is estimated under high

- sea-level rise (110 cm by 2100) accompanied by maximum dike construction. The primary driver for

- high vulnerability of coastal wetlands to sea-level rise is coastal squeeze, a consequence of long-

- term coastal protection strategies. Under low sea-level rise (29 cm by 2100) losses do not exceed

- ca. 50% of the total stock, even for the same adverse dike construction assumptions.  The model

- results confirm that the widespread paradigm that wetlands subject to a micro-tidal regime are

- likely to be more vulnerable to loss than macro-tidal environments. Countering these potential

- losses  will  require  both  climate  mitigation  (a  global  response)  to  minimise  sea-level  rise  and

- maximisation of accommodation space and sediment supply (a regional response) on low-lying

- coasts.

58

- Keywords: tidal wetlands, wetland vulnerability, wetland transitions, wetland loss, accommodation

- space, sea-level rise

61

62

63

# 1. Introduction

- Millennial,  centennial  and  decadal  records  of  changing  patterns  of  coastal  wetlands,

- including mangrove forests, saltmarshes, mudflats and associated habitats, show that they are

- particularly sensitive to environmental change (e.g. Morris et al., 2002; French, 2006; Fitzgerald et

- al., 2008; Mudd et al., 2009). More recent system changes also reflect the impacts of human

- activities superimposed on these natural dynamics, such as drainage and conversion to agriculture

- (e.g. Gedan et al., 2009) and aquaculture (e.g. Murdiyarso et al., 2015). There is concern, therefore,

- as to how near-future global environmental change will further modify these systems (e.g. Alongi,

- 2008; Kirwan et al., 2010; Fagherazzi et al., 2012).

- On contemporary timescales, tidal wetlands are biologically productive ecosystems of high

- biodiversity supplying multiple ecosystem services. At the same time they are subject to significant,

- and accelerating, rates of global coastal wetland loss due to natural and anthropogenic drivers (e.g.

- Adam, 2002; Millennium Ecosystem Assessment, 2005; Barbier et al., 2011; Nicholls et al., 2011).

- Davidson  (2014)  estimates  that  natural  coastal  wetlands  have  declined  by  46–50%  since  the

- beginning of the 18th century and by 62–63% over the course of  the 20th century and Leadley et

- al.’s (2014) Wetland Global Extent Index estimates an almost 50% decline between 1970 and 2008.

- Ecosystem  services  and  loss  rates  have  become  linked  over  the  last  decade  with  the

- recognition of the role of low-lying wetlands in natural coastal protection (e.g. Shepard et al., 2012),

- following the interactions between mangrove ecosystems and the wave fields of the 2004 Asian

- tsunami (e.g. McIvor et al., 2012) and between coastal marshes and 2005 Hurricanes Katrina and

- Rita on the Gulf coast, USA (e.g. Barbier et al., 2013) amongst others. Much remains to be done,

- however, on identifying the exact linkages between mosaics of coastal habitat area and habitat

- fragmentation and the maintenance of a coastal protection function (e.g. Barbier et al., 2008; Koch

- et al., 2009; Loder et al., 2009; Gedan et al., 2011). Furthermore, these debates are embedded in a

- context where the knowledge of the general spatial distribution of coastal wetland ecosystems is

- currently poor, particularly for saltmarshes (e.g. Rebelo et al., 2009, Saintilan et al., 2009; Chmura,

- 2011). There are serious gaps in the information base and much of the data that has been collected

- has come from different sources and different time periods and at a range of scales (Friess and

- Webb, 2014). Indeed, Friess et al. (2012) goes as far as to argue that the under-reporting of

- saltmarsh from the tropics underpins the presumption that mangrove replaces saltmarsh in the

- tropical intertidal zone. These shortcomings have hampered the assessment of the extent and

- condition of wetlands and proper estimations of the rate of loss. Thus one review concludes ‘a

- number of prognostications have been made regarding the future of the world’s mangrove forests

- in the face of climate change with local, regional, and global forecasts ranging from extinction to no

- or little change in areal coverage’ (Alongi, 2008, 8).

- Accelerated sea-level rise is a major threat to wetland futures at regional to global scales.

100

However, most detailed studies on wetland vulnerability to accelerated sea-level rise have been

101

over small spatial scales and short timescales and most concentrate on the likelihood of vertical

102

drowning (Webb et al., 2013), when sediment accumulation on the platform cannot keep vertical

103

pace with sea-level rise. There has been less emphasis on rates of horizontal retreat, associated

104

with  wave-induced  marsh  boundary erosion (Mariotti and Carr, 2014). Thus, for example, the

105

Surface Elevation Table – Marker Horizon (SET-MH) methodology has the necessary precision to

106

allow annual surface elevation change to be related to annual rates of sea level change (Cahoon et

107

al., 2002) although inter-site and inter-annual variations in surface response characteristics are high

108

(Cahoon et al., 2006). Historically, the SET-MH global network of sites has been patchy and not

109

focussed on those areas where wetland loss rates are thought to be particularly high (Webb et al.,

110

2013). However, there has been an expansion of sites globally in the last few years and it is

111

becoming possible to use this network to model larger scale patterns in wetland vulnerability, as

112

has been shown for Indo-Pacific mangrove SET sites (Lovelock et al., 2015). Furthermore, SET

113

datasets have been used as calibration datasets in other models of wetland change, most notably

114

the SLAMM model; we return to this usage below.

115

The  generic  problems  of  large-scale  analysis  have  been  addressed  in  part  by  the

116

development of macro-scale landscape models. These models vary in structure, complexity and the

117

ease with which they can be applied. The more sophisticated landscape models use geomorphic

118

and hydrologic sub-models to distribute fluxes of water, sediments and nutrients across a raster

119

grid  (e.g.  CELSS  model:  Sklar  et  al.,  1985)  to  calculate  likely  changes  in  wetland  type  extent.

120

However,  the  data  and  computational  requirements  of  such  an  approach  largely  preclude  its

121

application as a broad-scale tool for wetland analysis (Martin et al., 2002; Reyes, 2009; Couvillion

122

and Beck, 2013). Simpler models, such as cellular automata (Ross et al., 2009), capture the key

123

characteristics of wetland dynamics empirically, require fewer data, and are easily applied, but the

124

ability to deal with low frequency, high magnitude impacts and the recognition of the interaction

125

and	feedback	of	geo-morphological   and	ecological	processes	are	missing   (Kirwan	and

126

Guntenspergen, 2009). Nevertheless, these approaches are useful for calibration purposes, as we

127

demonstrate below. Of this suite of large-scale models, the one that has been most widely applied

128

is the ‘Sea Level Affecting Marshes Model’ (SLAMM) (Clough et al., 2010). SLAMM is open source

129

and has a user-friendly interface for implementation; is based on empirical calculations so that

130

computation times are substantially less than those required for complex numerical models; and

131

implementation has low data demands.

132

The pioneering Global Vulnerability Assessment (GVA), and its subsequent revision, is a

133

macro-scale model which provided the first worldwide estimates of the impacts of accelerated sea-

134

level rise on coastal systems (Hoozemans et al., 1993; Nicholls et al., 1999). This included a first-

135

order perspective on coastal wetland loss. Subsequently, the data on coastal wetland stocks has

136

improved (e.g. Vafeidis et al., 2008; Spalding et al., 2010; Giri et al., 2011), and the understanding of

137

the main drivers of change, including sea-level rise, has increased (Nicholls, 2004; McFadden et al.,

138

2007;  Nicholls  et  al.,  2007).  Hence,  a  re-evaluation  of  these  earlier  assessments  of  wetland

139

vulnerability is timely. This paper discusses the further development and application of a broad-

140

scale wetland change model: the Dynamic Interactive Vulnerability Assessment Wetland Change

141

Model (DIVA_WCM), originally developed within, and subsequently to, the European Community

142

DINAS-COAST Project. In this paper we show how a newly constituted database of contemporary

143

global coastal wetland extent can be linked to a revised conceptual model of the controls on

144

wetland health and resilience. In comparison to its previous version (McFadden et al., 2007), the

145

revised model has been parameterised with quantitative data where possible, calibrated by SLAMM

146

and other model outputs and validated by expert knowledge, including map-based approaches.

147

Thus, in its current form the model provides better-informed macro-scale projections of likely

148

future wetland extents than have been available previously.

149

# Methods

150

2.1 The DIVA modelling framework

151

DIVA  is  an  integrated,  global  modelling  framework  of  coastal  systems  that  assesses

152

biophysical and socio-economic consequences of sea-level rise and socio-economic development,

153

taking  into  account  coastal  erosion,  coastal  flooding,  wetland  change  and  salinity  intrusion

154

(http://www.diva-model.net; Hinkel, 2005; Hinkel and Klein, 2009; Hinkel et al., 2010, 2013, 2014).

155

The DIVA data modelling framework divides the world’s coast (excluding Antarctica) into 12,148

156

variable length coastal segments (mean length: 85 km; range: 0.009 km to 5,213 km) and associates

157

up  to  100  data  values with  each  segment  (Vafeidis  et  al.,  2008).  Each  segment  represents  a

158

relatively  homogenous  unit  based  on  geomorphology,  population  density  and  administrative

159

boundaries; there are a greater number of segments in the more populated areas. Only the DIVA

160

data associated with wetland change are considered in this paper.

161

DIVA  is  driven  by  climate  and  socio-economic  scenarios.  Using  the  HadGEM2-ES  Earth

162

System model from Phase 5 of the Coupled Model Intercomparison Project (CMIP5), three sea-level

163

rise scenarios have been investigated in this paper, representing a subset of the scenarios described

164

by Hinkel et al. (2014). These scenarios consider three Representative Concentration Pathways (RCP)

165

-  RCP2.6,  RCP4.5,  and  RCP8.5.  The  RCPs  correspond  to  different  levels  of  greenhouse  gas

166

concentration trajectories, ranging from a world of strong climate mitigation to one of increasing

167

emissions. A major uncertainty in projecting future sea-level rise is the contribution of land-based

168

ice. In Hinkel et al. (2014), each RCP scenario is associated with three levels of ice melt (low, median

169

and high) to create a ‘very likely’ range. The scenarios represent patterns of change (representing

170

thermal expansion and changes in ocean circulation, plus gravitational changes from ice sheets (the

171

contribution from ice caps is assumed to be uniform)) where some parts of the world have higher

172

or lower sea-level rise compared with the global mean. Projected global mean sea-level rise to the

173

year 2100 with respect to 1995 (mean sea level during 1985-2005 baseline period) for each of the

174

scenarios is given in Table 1 (median values, with 5% and 95% quantiles in parentheses; after Hinkel

175

et al., 2014). In this demonstration paper, we cover the widest range of sea-level rise scenarios,

176

from the lowest (5%) quantile of RCP2.6 (29 cm by 2100), through the median rate of sea-level rise

177

for RCP4.5 (50 cm), to the 95% quantile of RCP8.5 (110 cm). Finally, the sea-level rise scenarios are

178

downscaled to each DIVA database segment, including local land level change, following Peltier

179

(2000a, 2000b), and a 2 mm a-1 subsidence in deltas, reflecting natural sediment compaction.

180

Hence relative sea-level change varies from DIVA segment to segment.

181

For socio-economic scenarios, the Shared Socioeconomic Pathways (SSPs) are used. DIVA

182

considers population and gross domestic product (GDP) growth from the SSP2 scenario which sees

183

the trends typical of recent decades continuing, with moderate global population growth, some

184

progress toward achieving development goals and a slow decrease in the world’s dependency on

185

fossil fuels (IIASA, 2012). The socio-economic scenarios can influence the construction of dikes, and

186

187

hence the availability of accommodation space for wetlands, as explained below.

188

189

Table 1 near here

190

2.2. The Wetland Change Model (DIVA_WCM)

191

The   DIVA_WCM   is   one   module   in   the   DIVA   modelling   framework   (Version   5.1

192

(http://www.diva-model.net/)). DIVA_WCM comprises i) a newly constituted global database of

193

coastal wetlands built on the basis of the original DIVA database (Vafeidis et al., 2008) and ii) an

194

impacts algorithm for coastal wetlands. It improves upon the existing DIVA-WCM (McFadden et al.

195

2007) by extensive new parameterisation and calibration.

196

The coastal wetland database employed in this paper was derived, under licence, from

197

datasets held by the United Nations Environment Programme’s World Conservation Monitoring

198

Centre  (UNEP-WCMC),  the  specialist  biodiversity  assessment  arm  of  UNEP.  A  global  layer  of

199

mangrove forest data, revised after Giri et al. (2011), was augmented with a recently improved

200

saltmarsh data layer; both  layers were imported  into the DIVA  database and  assigned to the

201

appropriate coastal segment. Sub-sets of the data were checked to ensure that there was no

202

corruption of data in the transfer from the original files to the database. In the database, six

203

wetland types associated with coastline segments are considered: 1) coastal forest; 2) mangrove

204

forest; 3) freshwater marsh (of limited extent); 4) saltmarsh; and unvegetated sediments, which are

205

divided into 5) sabka and saline tidal flats (of very limited extent); and 6) mudflat/sand flat. The

206

distinction between these latter two types is based on climatic setting and elevation (derived from

207

the ETOPO2 (NGDC, 2001) dataset): type 5 is above (‘unvegetated high’) and type 6 is below

208

(‘unvegetated  low’)  Mean  High  Water  Springs  (MHWS),  respectively.  Data  on  unvegetated

209

sediments, freshwater marshes and coastal forested originate from the above mentioned GVA

210

(Hoozeman et al., 1993).

211

French (2006, 120) states that ‘… the existence and ecological function of tidally-dominated

212

saltmarshes  are  ultimately  contingent  upon  the  operation  of  hydrodynamic  and  sedimentary

213

processes within constraints imposed by the intertidal accommodation space and the sediment

214

supply’. The DIVA_WCM algorithm considers three key drivers that control wetland response to

215

sea-level rise: 1) local sea-level rise relative to tidal range; 2) lateral accommodation space; and 3)

216

sediment supply, following McFadden et al. (2007). A score of one to five is adopted to represent

217

present and future forcing levels for each of these drivers: one corresponds to the lowest forcing

218

and vice versa.

219

Wetlands respond to sea-level rise over different time horizons. Mangroves and coastal

220

forests respond more robustly to environmental change than the other wetland types because

221

slower growth rates across a wide range of environmental tolerance allows for survival under

222

moderate levels of stress (Ball, 1988). This is represented in the model by different response times

223

for these wetland types. Macro-scale landscape models, and specifically output derived from the

224

WARMER and SLAMM model applications, were used to characterise wetland resilience to sea-level

225

rise  and  other  stresses  where  possible.  Expert  judgment,  from  peer-reviewed  literature  and

226

research project reports, was applied where necessary.

227

Figure  1  outlines  the  primary  components  of  the  DIVA_WCM,  comprising  four  sets  of

228

calculations:

229

- Assessment of the vulnerability of all wetlands to sea-level rise. This vulnerability score (VS)

230

depends upon  sea-level  rise,  lateral accommodation  space  and  sediment  supply and  is

231

universal. As the score is calculated segment-by-segment, it is described as the Coastal

232

Segment Vulnerability Score (CSVS);

233

- Conversion of the Coastal Segment Vulnerability Score into a wetland-specific Ecological

234

Sensitivity Score (ESS);

235

- Calculation of the proportion of wetland loss/change that is expected for each wetland type

236

based on the ESS score; and

237

- Calculation  of  wetland  habitat  successional  changes  and  wetland  loss  to  open  water,

238

239

generating new wetland areas, through a habitat translation model.

240

Figure 1 near here

241

It  is  important  to  note  that  the  DIVA-WCM  is  a  wetland  loss  model  for  sea-level  rise;  we

242

acknowledge that near-future environmental change may also produce new areas of wetland in

243

particular landscape settings, but the generation of new wetland is not considered further in this

244

paper. The following sections detail each of these methodological steps. Further information is

245

246

given in the Supplementary Material.

247

    - Relative sea-level rise / tidal range

248

When relative sea-level rise is sudden and of high magnitude, as might result from tectonic

249

activity, the wetland surface may be abruptly submerged (e.g. Atwater, 1987). More frequently,

250

and the subject of concern here, coastal wetlands are subjected to slow, progressive relative sea-

251

level rise caused by the combination of eustatic factors and regional to local subsidence. This

252

process is reflected in the changing pattern of tidal submergence, or hydroperiod (Reed, 1995). If

253

sediment supply is sufficient, marsh surfaces will accrete vertically, rapidly at first but then slowing

254

over time as fewer tides inundate the progressively higher surface (Allen, 1990).   Conversely, in

255

sediment-poor wetlands, subject to a rise in relative sea level without equal increases in wetland

256

surface elevation from sediment accretion, the duration and depth of tidal flooding will increase

257

over time. In this situation, wetland vegetation may revert to a community composition more

258

typical of lower elevations in the tidal frame (Huiskes, 1990; Mendelssohn and Morris, 2000).

259

No clear relationships have been found at the large-scale between accretion rates and tidal

260

range (Allen, 1990; French and Reed, 2001; Cahoon et al., 2006; Mossman et al., 2012). However,

261

Stevenson et al. (1986) showed accretion deficits (rate of sea-level rise minus rate of near-surface

262

accretion) to be greater in low tidal range saltmarshes than in higher tidal range marshes along the

263

eastern seaboard of the USA. This has been attributed to the expanded intertidal range that can be

264

occupied  by  vegetation  (e.g.  Day  et  al.,  1995)  and  the  increased  flood-dominance,  and  thus

265

enhanced sediment supply (e.g. Friedrichs and Perry, 2001), of macrotidal (> 4 m spring tidal range)

266

marsh  systems compared to  meso-tidal (2-4 m  tidal range) or  micro-tidal (< 2  m tidal range)

267

systems (Kirwan et al., 2010; Fagherazzi et al., 2012). Furthermore, in micro-tidal settings the

268

expansion of the tidal prism on sea-level rise is disproportionately large, with increases in tidal

269

channel geometries leading to loss of wetland area (Kirwan and Guntenspergen, 2010).

270

The first environmental forcing factor captures this process through the dimensionless relative

271

sea-level rise term rslr_d, where the annual relative rise in sea level (RSLR) is scaled by the segment-

272

specific tidal range (Equation 1 in Supplementary Material; and see Table A1 for details of the tidal

273

range  parameter).  Unlike  earlier  applications  of  this  approach  (Nicholls  et  al.,  1999),  the

274

dimensionless  RSLR  is  described  as  a  power  function  with  an  exponent  of  1.4,  based  on  the

275

literature review described above and expert judgement. The scoring of rslr_d is based on fixed

276

class boundaries that are initialized before simulation. Assuming a current 3 mm a-1  global mean

277

sea-level rise rate (after Church et al., 2013), we subtract the segment-specific uplift (obtained from

278

http://www.atmosp.physics.utoronto.ca/~peltier/data.php), and calculate the 95th, 84th, 50th, and

279

16th percentiles of the cumulative distribution of the resulting rslr_d parameter to derive the class

280

boundaries of rslr_tidal_score, while only considering segments where wetlands are present (for

281

exact values see Supplementary Material, Table A2). During simulation, the rslr_d forcing factor is

282

updated  and  scored  at  every time  step  and, hence, is driven  by the associated  sea-level rise

283

284

scenario (see section 2.1 above and Table 1).

285

    - Lateral accommodation space

286

The notion of ‘accommodation space’ comprises two components defined by sea-level rise,

287

namely  vertical  wetland  surface  adjustment  upwards  and  lateral  habitat  migration  landwards

288

(Phillips,  1986;  Allen,  1990).  The  characterisation  of  lateral  accommodation  space  within  the

289

DIVA_WCM is built on an assessment of the impact of two controlling factors: i) coastal slope

290

(Brinson et al., 1995); and ii) the presence or absence of dikes, which limits lateral accommodation

291

space (Feagin et al., 2010). Lateral accommodation space, aspace, is calculated recursively in the

292

model.  The  aspace  value  is  initialised  for  each  segment  using the  average  topographic  slope,

293

derived  from  the  ETOPO2  (NGDC,  2001)  dataset.  Model  categorisation  of  coastal  slope,  and

294

associated forcing scores, are given in the Supplementary Material Table A3. This initialized aspace

295

score is then updated based on the estimated sea-dike height at each time-step. If appropriate,

296

aspace is increased by 0.25 at each time step until the highest forcing score is obtained (Equation 2,

297

Supplementary Material). Thus, the loss of lateral accommodation space is a progressive process in

298

terms of stressing wetlands, being maintained at the highest vulnerability for the remainder of the

299

model run once aspace reaches the maximum score of five. Importantly, the model does not

300

simulate the impact of creating new lateral accommodation space.

301

The DIVA model considers dike construction, and dike upgrading, as an adaptation response

302

to coastal flooding (Hinkel et al., 2014). Three scenarios for dike construction are evaluated in this

303

paper.  Two  bounding  cases  of  ‘no  dikes’  (in  which  no  coastal  floodplains  are  protected)  and

304

‘maximum dikes’ (in which all coastal floodplains are protected) are considered. In addition, dikes

305

built  according  to  a  ‘demand-for-safety’  function,  assuming  the  SSP2  socio-economic  scenario

306

(Hinkel et al., 2014), are also evaluated and termed ‘widespread dikes’. The driver aspace is not

307

influenced by dikes under ‘no dikes’; is most affected under ‘maximum dikes’; and is subject to an

308

309

intermediate effect under ‘widespread dikes’.

310

    - Sediment supply

311

It has been strongly argued that sediment starvation at the coast, associated with the human

312

management of river courses, deltas and erodible coastal cliffs, has had profound consequences for

313

the maintenance of coastal sediment systems (e.g. Syvitski et al., 2005, 2009; Stralberg et al., 2011).

314

The DIVA_WCM also characterises the ability of a wetland to keep pace with relative sea-level rise

315

through a third parameter, sediment supply sedsup. Following Stevenson et al. (1986), a widely

316

adopted methodological approach has been to compare the rate of vertical accretion to relative

317

sea-level rise and thus to calculate a wetland accretionary surplus or deficit. Such an approach

318

assumes that accretion is equal to wetland surface elevation change. This is now known to be a

319

simplification,  as  the  relative  balance  between  the  in  situ  accumulation  of  organic  sediments

320

(Cahoon and Reed, 1995; Middleton and McKee, 2001; Rooth et al., 2003) or external, inorganic

321

inputs (French and Spencer, 1993; Christiansen et al., 2000), or a combination of the two (Saintilan

322

et al., 2013), can affect this balance, as can subsurface processes occurring within the soil column,

323

including compaction, plant growth-decomposition and shrink-swell behaviour related to varying

324

water  storage  (Cahoon  et  al.,  2011;  Krauss  et  al.,  2014).  However,  given  the  scale  of  the

325

DIVA_WCM, these relationships must be simplified for the purposes of modelling, with a distinction

326

between those settings and environmental histories that promote high sediment supply and those

327

that favour low sediment supply.  The model therefore considers a combination of six contextual

328

physical and anthropogenic controlling factors – (1) tectonic context; (2) fluvial sediment inputs to

329

the  coastal  zone;  (3)  sediment  availability  from  Quaternary  glacial  sediments;  (4)  coastal

330

geomorphic setting; (5) degree of coastal protection structures; and (6) timing of sediment supply

331

from historical land use practices – in assessing the impact of varying sediment supply on wetland

332

vulnerability. Each of these factors exhibits a range of values identified by a range in ‘forcing score’

333

across different categories (see Supplementary Material, Table A4). Furthermore, each factor is

334

multiplied by an internal weighting to reflect their relative significance within the sedsup parameter

335

(Equation 3, Supplementary Material). The respective weights are based on expert judgement,

336

derived from field experience and the published literature. It is recognised that sediment supply is

337

the most difficult forcing factor to understand and parameterise in the model, due both to its

338

localized and highly variable nature and to the lack of wetland datasets that specifically estimate

339

340

this parameter.

341

  - Coastal Segment Vulnerability Score (CSVS)

342

The  Coastal  Segment  Vulnerability  Score  (CSVS)  reflects  the  integrated  response  of  a

343

wetland to relative sea level rise / tidal range, lateral accommodation space and sediment supply.

344

The influence of each of these parameters is reflected through the weighted sum of the forcing

345

factors, with the following weights: 0.5 for rslr d;   0.2 for aspace;   and 0.3 for sedsup   (and see

346

Equation 4, Supplementary Material). These relative weightings indicate the importance of each

347

parameter at the macro-scale, the values being derived from expert judgement, in turn based on

348

field experience and published references. For gaining a better understanding of how these weights

349

influence the model results, we performed a sensitivity analysis, comparing the model output of a

350

351

series of different weight combinations.

352

  - Ecological Sensitivity Score (ESS)

353

Ecological   systems   are   characterised   by   varying   reaction   and   relaxation   times   to

354

environmental  perturbation.  Many  saltmarsh  herbs,  shrubs  and  grasses  are  very  sensitive  to

355

landform  change.  Thus,  for  example,  manipulative  experiments  in  freshwater  marsh  systems,

356

where inundation frequencies have been changed by transplanting marsh communities to lower (-

357

10 cm) levels have shown responses in plant stem density and biomass over periods as short as a

358

single growing season (McKee and Mendelssohn, 1989). By comparison, coastal forest trees show

359

slower  responses  to  changing  environmental  conditions.  Thus,  cypress  forests  on  the  Gulf  of

360

Mexico, USA have recorded 50% survival rates after 4 years of +120 cm and 18 years of + 60 – 300

361

cm increases in water levels respectively. Modelling of bottomland forest succession is typically

362

undertaken over 50 year timescales (summarised in Conner and Brody, 1989). However,  resilience

363

characteristics   are   quite   different   from   herbs   and   shrubs,   showing   permanence   (if   not

364

regeneration) until a threshold point is reached when the system collapses catastrophically, as in

365

the case of hurricane-impacted mangrove forest (e.g. Cahoon et al., 2003). Once such a threshold

366

has  been  crossed,  system  re-establishment  may  be  difficult  and  long-delayed  (for  review  see

367

Spencer and Möller, 2013). Lag weights for current and previous 5-year time steps of a model run

368

were applied to parameterize these habitat-specific response lags to changes in the environmental

369

forcing   factors   (Table   A5,   Supplementary   Material).   For   ‘freshwater   marsh’,   ‘salt   marsh’,

370

‘unvegetated low’, and ‘unvegetated high’, a response time of 5 years was assumed and for ‘coastal

371

forest’ and ‘mangrove forest’, a response time of 10 years. The resulting modification of the Coastal

372

373

Segment Vulnerability Score (CSVS) is termed the Ecological Sensitivity Score (ESS) (Fig. 1).

374

  - Habitat successional changes and wetland loss to open water

375

Most existing large-scale models of wetland response to accelerated sea-level rise (e.g. the

376

GVA  and  its  subsequent  revisions  (Nicholls et al.,  1999))  assume  the conversion  of  vegetated

377

surfaces to open water and thus simply generate statistics on total loss of wetland area. Such

378

models  are  most  appropriate  where  local  rates  of  relative  sea-level  rise  are  high,  such  as  in

379

subsiding, sediment-starved deltaic environments. However, under more moderate rates of sea-

380

level  rise,  and  with  an  adequate  sediment  supply,  ecosystem  change  may  be  i)  slower  than

381

predicted;  and  ii)  involve  change  stepped  across  wetland  types  rather  than  outright  loss,  as

382

ecological tolerances of particular plant communities are exceeded in turn. DIVA_WCM assesses

383

both conversion to open water and transitions to other wetland types due to environmental change

384

through i) the construction of a series of wetland response curves, to define the proportion of

385

wetland expected to be lost; and ii) a model of wetland transitions, where losses are distributed

386

between different wetland types and open water.

387

During the first stage in the development of the habitat transition algorithm, a series of

388

habitat-specific response curves were estimated for total wetland loss as a function of Ecological

389

Sensitivity Score (ESS) (Fig. 1). These curves were approximated using the beta distribution (Fig. 2;

390

and see Equation 6, Supplementary Material) as this distribution can describe a wide range of

391

shapes within a constrained distribution (0% to 100% total wetland loss and 0.0 to 5.0 ESS value).

392

This  is  particularly  useful  for  constructing  habitat-specific  response  curves,  reflecting  different

393

resilience characteristics that were then calibrated using WARMER and SLAMM model outputs

394

(described in more detail below).

395

Figure 2 near here

396

Where there is wetland change or loss, Figure 3 outlines the model of transition that was

397

used  within  the  DIVA_WCM.  Except  in  the  case  of  coastal  forest,  with  low  to  moderate

398

environmental forcing (CSVS value < 4) wetland types are transformed not only into open water

399

(50%) but also into other wetland types which are found lower in the tidal frame (50%), created as

400

a result of losses of wetland that occupy higher elevations. Thus, ‘low unvegetated’ wetland (i.e.

401

mudflat-sandflat) can be created where it did not previously exist within a geographical location.

402

Under high levels of environmental forcing (CSVS value >= 4) the model converts all wetland types

403

to open water.

404

405

Figure 3 near here

406

407

  - Model calibration

408

In previous explorations of the DIVA_WCM model structure (McFadden et al., 2007), the

409

model was calibrated qualitatively against model predictions of large-scale wetland type transitions

410

in the Barataria and Terrebonne sub-basins of the Mississippi Delta Plain (Reyes et al., 2000).

411

However, such calibration is problematic because these simulations begin in the period well before

412

the 1985 – 2005 baseline used in this study. More recent scenario modelling in the same region,

413

associated with Louisiana’s 2012 Coastal Master Plan (Couvillion et al., 2013) does provide detailed

414

forecasting of wetland loss on a sub-basin by sub-basin basis, but only until 2060. As an alternative

415

way forward in this study, therefore, DIVA_WCM was calibrated against a set of six recent studies

416

of wetland change undertaken using the WARMER, and particularly, the SLAMM model (Table 2).

417

Table 2 near here

418

A number of difficulties were encountered in the inter-comparison of the DIVA_WCM with

419

WARMER and SLAMM model outputs. Firstly, in many of the SLAMM studies it was difficult to

420

extract the necessary comparative information required on the sea-level rise scenario that had

421

been applied (i.e. starting year, end year, sea-level rise function used (such as SRES, RCP, linear

422

models), total sea-level rise within the simulation period (with reference to starting year)). Where

423

scenarios referred to a SRES-scenario (IPCC SRES, 2000), but with user-defined amplitude, these

424

were calculated, on a globally uniform basis, with the SRES-scenarios supplied with DIVA. Other

425

scenarios  were  identified  as  being  driven  by  a  linear  rate  of  sea-level  rise  and  constructed

426

accordingly. Secondly, the definition of wetland habitat types differed between the six calibration

427

studies and introduced some additional wetland types not present in the DIVA typology. It was thus

428

necessary to re-classify the wetland descriptions into their equivalent DIVA categories. Thirdly, the

429

effect of sea-dikes were included in SLAMM – DIVA_WCM model comparisons where the SLAMM

430

studies made explicit reference that wetland loss had been affected by the presence of a dike.

431

However, it was not always very clear as to whether or not this had actually been the case. For

432

calibration, the DIVA_WCM  model was run against the relevant  WARMER and SLAMM model

433

output for each of the sea-level rise scenarios reported in the respective study, including dikes

434

where  applicable.  The  results  were  aggregated  to  allow  inter-model  comparison,  the  form  of

435

aggregation depending on whether the WARMER / SLAMM study area integrated several DIVA

436

coastal segments or, alternatively, the one DIVA segment integrated several WARMER / SLAMM

437

sites. All loss rates are reported as percent total wetland loss per 5 years. The error measure used

438

to evaluate the model performance with reference to the reported data is the relative mean

439

difference (RMD). The ‘Nelder-Mead simplex direct search’ algorithm (Lagarias et al., 1998) was

440

applied to search for the RMD closest to zero by varying the habitat-specific beta values (Fig. 2,

441

Equation 6 in Supplementary Material). This exercise was conducted without constraints regarding

442

the  relationships  between  the  different  beta  values.  The resilience/sensitivity  estimation  of  a

443

specific habitat (Fig. 2) thus usefully emerged as a result of the calibration exercise. Running the

444

DIVA_WCM with the optimized beta values (Equation 6, Supplementary Material) against the six

445

calibration studies produced a RMD value of 0.000227 with a mean difference of 0.00002% (loss per

446

5 years), indicating a close fit of model values with reported loss rates.

447

Fig.  4  shows  the  DIVA_WCM  model  outputs  against  the  SLAMM  and  WARMER  calibration

448

studies. Interestingly, the WARMER model, which is not a derivative of SLAMM but a numerical

449

saltmarsh model that includes biophysical feedbacks, performed in a very similar manner to the

450

SLAMM models when it came to comparisons with the DIVA_WCM model. Outliers relate to a small

451

number of particular DIVA segment comparisons across two SLAMM studies (Craft et al., 2009;

452

Geselbracht et al., 2011). It is clear that these examples of poor model fit are not related to

453

vegetated wetland habitats but rather to problems with estimating changes in ‘unvegetated low’

454

habitat  (i.e. mudflat/sandflat). This raises the need for an  improvement  in  model formulation

455

regarding the mechanisms in place when vegetated wetlands are drowning. Not considering the

456

site  exposure  to  wave  activity  may  partly  explain  the  poor  model  representation  of  the

457

‘unvegetated  low’  at  these  sites.  While this  comparison  illustrates  the  difficulty  of  model

458

calibration, these results are calibrated to these more detailed simulations, improving on the earlier

459

methods of Hoozemans et al. (1983), Nicholls et al. (1999) and McFadden et al. (2007). Further

460

efforts at improved model calibration should receive high priority in future research efforts.

461

462

463

Figure 4 near here

464

465

466

  - Model validation

In order to undertake independent validation at a scale appropriate to the DIVA scale of

467

analysis, model outputs of the calibrated DIVA_WCM were compared with two broad-scale coastal

468

wetland  vulnerability  studies,  one  concerned  with  the  modelled  vulnerability  of  Indo-Pacific

469

mangrove forests to sea-level rise and one a qualitative assessment of wetland stability along the

470

US mid-Atlantic coast.

471

Lovelock et al. (2015) developed a model (hereafter referred to as the ‘Lovelock model’) to

472

predict the time to submergence of mangrove ecosystems subject to accelerated sea level rise

473

based on the concept of the loss of ‘elevation capital’, the potential of a mangrove ecosystem to

474

remain within a suitable inundation regime (between Highest Astronomical Tide (HAT) and Mean

475

Sea Level (MSL)).  The key controlling parameters are the rate of sea level rise, the tidal range and

476

suspended sediment supply. Sites with a tidal range of 10 m need to lose 5m of elevation capital to

477

bring them to the critical survival threshold of MSL whereas sites with a tidal range of 1 m only have

478

to lose 0.5 m of elevation to bring them to this threshold. Thus the Lovelock model predicts that

479

sites with low tidal range are significantly more vulnerable to loss than those experiencing a high

480

tidal range. Loss of elevation capital can be offset by elevation gains from vertical accretion as sea

481

level rises. Thus mangrove forest sites with high sediment supply are less vulnerable to conversion

482

to open water than sites with low sediment supply. Total suspended matter in coastal waters was

483

acquired  from  remotely  sensed  imagery  and  converted  to  elevation  gain  through  established

484

relationships between sea-level rise, suspended sediment concentrations and measured changes in

485

surface elevation from SET sites. The Lovelock model excludes consideration of an accommodation

486

space term but it does identify the importance of the relations between rates of sea level rise, tidal

487

range and sediment supply in determining mangrove forest vulnerability to sea-level rise in a similar

488

manner to the DIVA_WCM.   The difference in model structure makes Lovelock et al. (2015) an

489

appropriate validation case for the DIVA_WCM. The two models cannot be compared directly

490

because i) the Lovelock model provides a binary survival or loss indicator whereas the DIVA_WCM

491

estimates percentage loss of mangrove forest over time; and ii) the Lovelock outputs are reported

492

on a 4 km resolution grid defined by the remotely sensed TSM data whereas the DIVA_WCM results

493

are mapped onto the variable length DIVA coastal segments. However, a qualitative assessment is

494

possible,  for  comparable  sea  level  rise  scenarios  to  2100  (Fig.  5).  The  areas  of  mangrove

495

submergence predicted by the Lovelock model (Fig. 5b) map well onto the areas of highest coastal

496

wetland loss predicted by the DIVA_WCM (Fig. 5a). Apart from Australia and Brunei, of the top ten

497

areas of expected mangrove loss identified by the DIVA_WCM in the region shown in Figure 5, eight

498

areas are also highlighted by the Lovelock model: Cambodia (55% coastal wetland loss at country

499

level by 2100 in the DIVA_WCM); Philippines (50%); Sri Lanka (48%); Thailand (46%); Indonesia

500

501

(40%); Federated States of Micronesia (40%); Papua New Guinea (39%); and Solomon Islands (39%).

502

503

Figure 5 near here

504

A qualitative  assessment  of  wetland  stability on  the  eastern  seaboard of  the USA  was

505

performed on behalf of the US Environmental Protection Agency (EPA) (Reed et al., 2008). As with

506

the first validation exercise, a direct comparison between the EPA assessment and the DIVA_WCM

507

output is not possible.  This is partly because the DIVA_WCM segments along the eastern seaboard

508

do not allow the level of disaggregation seen in the EPA assessment and partly because Reed et al.

509

(2008) rely on a more qualitative, expert judgement approach. Nevertheless, validation at the level

510

of aggregation associated with the DIVA_WCM model was possible. Reed et al. (2008) assume a

511

current rate of sea-level rise of 3 mm a-1 and provide estimates for future wetland development for

512

three linear SLR scenarios: a continuation of the current rate of sea-level rise; the current rate plus

513

2 mm a-1 (i.e. 5 mm a-1); and the current rate plus 7 mm a-1 (i.e. 10 mm a-1) (Fig. 6a). The calibrated

514

DIVA_WCM was run for each of these three scenarios for each DIVA_WCM segment that falls

515

within this study area; it assumed that no dikes are present, since the primary driver analysed by

516

Reed et al. (2008) was wetland drowning due to insufficient vertical wetland growth. Percentage

517

wetland losses as predicted by the DIVA_WCM, and mapped into groups suggested by the Reed et

518

al. (2008) categories, are shown in Fig. 6b-6d for each of the three sea-level rise scenarios. The

519

categorical comparison shows that the DIVA_WCM reproduces the general patterns of increasing

520

wetland vulnerability with increasing rates of SLR. While Reed et al. (2008) conclude that most parts

521

along the US mid-Atlantic marshes are unlikely to be converted to open water under current rates

522

of SLR, modelled loss rates with DIVA_WCM for the linear 3 mm a-1  SLR are <50% in most coastal

523

segments of the study area. Following Reed et al. (2008), with 5 and 10 mm a-1 SLR rates wetlands

524

are expected to survive to a marginal extent only or completely disappear respectively. Equivalent

525

conclusions can be drawn from the model validation runs, indicating wetland loss rates between 50

526

and 75% and >90%, respectively.

527

While  the  general  trend  of  modelled  wetland  loss  rates  compares  well  with  the  EPA

528

assessment,  the  spatial  patterns  in  the  area,  as  suggested  by  Reed  et  al.  (2008),  are  poorly

529

represented in the model results. The most important reason for this is the relatively large length of

530

coastal segments in estuarine environments, smoothing estuarine gradient and neglecting local

531

variations in tidal range and sediment supply. This in turn highlights the spatial scale at which the

532

DIVA_WCM results have to be interpreted and the conclusions that can be drawn from them. The

533

model is suitable for identifying hotspot regions (~200 km coastline length) of coastal wetland loss

534

535

but is not applicable for sub-regional (< 100 km) scale analysis.

536

537

Figure 6 near here

538

539

# Results

540

The DIVA database indicates a mapped total global coastal wetland stock (in 2011) of 756

541

x103 km2. This figure compares to the 302 x103 km2 reported by Hoozemans et al. (1993), for data

542

collected in the 1980s. Absolute and relative rates of global wetland loss between 1995 and 2100

543

are shown in Figure 6 for the high, medium and low scenarios of global sea-level rise and the three

544

dike  scenarios,  as  outlined  earlier  (sections  2.1,  2.2.2.  respectively).  These  combinations  give

545

wetland loss by 2100 in the range from 281 to 592 x103 km2, or between 37 and 78 % of the total

546

stock of global coastal wetlands (Table 3). Total wetland loss from 1995 (mean sea level during

547

1985-2005 baseline period) to 2100 strongly varies with sea-level rise, with wetland losses being 27

548

- 31% lower for the lowest SLR scenario in comparison to the highest SLR scenario, independent of

549

550

the dike scenario (‘no dikes’, ‘widespread dikes’, ‘maximum dikes’) applied (Fig. 7, Table 3).

551

552

Figure 7 near here

553

554

Table 3 near here

555

In  order  to  obtain  a  better  understanding  of  which  of  the  applied  forcing  factors  are

556

primarily responsible for these wetland losses, we report the results of a sensitivity analysis for the

557

558

weights of the different forcing factors (see Supplementary Material Equation 4).

559

  - Global coastal wetland loss rates by weighting of environmental forcing factors under high sea-

560

561

level rise

562

Under the high sea-level rise scenario of 110 cm by 2100 (95% quantile, RCP8.5; Table 1)

563

and with no dike building, the DIVA_WCM model predicts a loss of between 392 and 578 x103 km2

564

of coastal wetlands worldwide by 2100 (Fig. 8a), or 52 - 76% of the total global stock, depending

565

upon the comparative weighting of the three environmental forcing factors (Table 4). The loss of

566

total global stock is 11 – 18% by the 2020s, 27 - 43% by the 2050s and 42 - 65% by the 2080s (Fig.

567

568

8a).

569

570

Figure 8 near here

571

A sensitivity analysis shows that, in the absence of dikes, the variation in loss rate is strongly

572

controlled by the influence of the accommodation space term. Where accommodation space has a

573

relatively high weighting (and wetlands can migrate inland over low coastal slopes), loss rates are at

574

the lower bound (Fig. 8a, Table 4(a)); where the influence of accommodation space is neglected by

575

the model, all combinatorial weightings of sea-level rise and sediment supply give rise to high rates

576

577

of total wetland loss by 2100 (Table 4(b)).

578

Table 4 near here

579

580

The  importance  of  accommodation  space  points  to  the  critical  importance  of  dike

581

construction under the high sea-level  rise scenario. Under the  most extreme scenario of dike

582

building  tested,  ‘maximum  dikes’,  the  DIVA_WCM  is  largely  insensitive  to  any  of  the  model

583

parameters (Fig. 8b, Table 5), as the benefit of accommodation space is lost (cf. Table 4(a)). Global

584

585

wetland loss rates are very high, at 574 – 619 x103 km2 of the total stock of 760 x103 km2 (Fig. 8b).

586

587

Table 5 near here

588

  - Global coastal wetland loss rates by weighting of environmental forcing factors under low sea-

589

590

level rise

591

Wetland loss rates are significantly less (Table 3) under the low sea-level rise scenario (5%

592

quantile, RCP2.6, Table 1) and there is less acceleration in the wetland loss rate towards 2100 (Fig.

593

7). With no dike building, the DIVA_WCM model predicts a loss of between 222 – 356 x103 km2 of

594

coastal wetlands worldwide by 2100, or 29 – 47% of the total global stock, depending upon the

595

comparative weighting of the three environmental forcing factors (Fig. 8c). The accommodation

596

space term remains an important discriminator within this range but the overall range in loss rate is

597

a third less than under the high sea level scenario (Fig. 8c, Table 6(a)). When the accommodation

598

space term is removed, it is clear that the main control on wetland loss is the sea-level rise / tidal

599

range term, rslr_d, (Table 6(b)) with lower loss rates where this term is high. Under the highest level

600

of  dike construction  (‘maximum  dikes’), the  envelope of loss rates narrows and  rises but not

601

greatly, to between 312 – 418 x103 km2 of coastal wetlands worldwide by 2100, or 41 – 55% of the

602

total global stock (Fig. 8d). Similar to the results when accommodation space is neglected, the loss

603

rates are controlled by the slr / tidal weight, with lower loss rates when the slr / tidal weight is high

604

605

(Table 7).

606

607

Table 6 near here

608

609

Table 7 near here

610

611

  - Global patterns in predicted wetland loss rates

612

As well as the global estimates of wetland loss, the results can be disaggregated down to

613

individual segment level where wetlands have been recorded. It is thus possible to view both the

614

global pattern of potential absolute wetland loss (Fig. 9a) and relative wetland loss (Fig.  9b). These

615

plots assume ‘widespread dikes’ and the medium sea-level rise scenario (median, RCP4.5; Table 1).

616

In these contexts, the wetlands that appear most at risk are those characterised by micro-tidal

617

618

settings. Regional hotspots include the Mediterranean Sea, the Caribbean Sea and the Baltic Sea.

619

Figure 9a near here

620

621

Figure 9b near here

622

623

# Discussion

624

These results show that  coastal  wetlands are sensitive to sea  level  rise  and,  based on

625

credible scenarios for the 21st  century, there is a potential for considerable wetland loss at the

626

global scale. This will be exacerbated by coastal squeeze caused by the construction, and upgrading,

627

of dikes which, whilst providing flood defence to coastal populations and infrastructure, prevent

628

the onshore and upslope migration of wetlands.   This model-based conclusion is consistent with

629

other assessments in the scientific literature (e.g. Nicholls et al., 2007; Wong et al., 2014). It is also

630

consistent  with  earlier  global  assessments.  While  considering  a  smaller  global  wetland  stock,

631

Hoozemans et al. (1993) concluded that a 1 m sea-level rise might cause coastal wetland loss of 154

632

– 180 x103 km2, or 51 – 60% of total global stock, depending upon assumptions about development

633

and dike construction. For a similar sea-level rise scenario, Nicholls et al. (1999) estimated wetland

634

losses of up to 46% of global coverage. The losses associated with a 38 cm rise in sea level by the

635

2080s were estimated at 0 – 2% by the 2020s, 2 – 11% by the 2050s and 6 – 22% by the 2080s. In

636

this analysis, evaluating the contribution of lateral accommodation space and sediment supply

637

controls as well as sea-level rise, the most comparable sea-level rise scenario, the 5% quantile of

638

RCP2.6 (29 cm by 2100), gives loss rates of 10-11% by the 2020s, 23-28% by the 2050s and 32-40%

639

by the 2080s. However, under the 95% quantile of the RCP8.5 sea-level rise scenario (110 cm by

640

2100), the wetland loss rates rise to 14-15% in 2020s, 33-40% in the 2050s and 53-66% in the

641

2080s. Hence, in its current form the DIVA_WCM model shows higher sensitivity to sea-level rise

642

than these earlier analyses, and losses two or more times higher than these earlier estimates

643

appear possible.

644

However, this sensitivity may be reduced in future iterations if appropriate feedbacks from

645

changing plant physiology and tidal hydrodynamics can be included in the model structure. Thus,

646

for example, increased atmospheric CO2 and warmer temperatures, allied to mid-range rates of sea

647

level rise, may lead to increases in the rates of plant productivity and wetland accretion (Langley et

648

al., 2009; Cherry et al., 2009; Kirwan and Gutenspergen, 2012; Kirwan and Mudd, 2012), These

649

dynamics  might  be  further  reinforced  by  increased  sediment  supply to  wetland  surfaces  with

650

greater tidal energetics under higher sea levels, albeit with limits to ‘ecogeomorphic’ adaptability at

651

higher rates of sea level rise (Kirwan et al., 2010).

652

These  rates  also  need  to  be  seen  in  the  context  of  wetland  losses  resulting  from

653

anthropogenic impacts. Thus, for example, Dodd and Ong (2008) have estimated that the coastal

654

populations of nation states with mangroves will rise by 50%, from 1.8 billion to 2.7 billion, in the

655

period between 2000 and 2025. Human pressures on mangroves include direct conversion to urban

656

use  for  industry,  port  development,  and  housing;  conversion  for  aquaculture  and  agriculture;

657

timber extraction; and modification of hydrology and pollution, particularly oil pollution, nutrients

658

associated with agricultural intensification, and heavy metals contamination. These pressures will

659

be imposed upon mangrove systems (plus other wetlands) already suffering significant long-term

660

declines in extent (Spencer and Möller, 2013). Whilst the exact figure for loss may be debateable,

661

the general sentiment of Nicholls et al.’s (1999, S82) statement that ‘when combined with the

662

direct loss scenarios due to direct human destruction, in the worst case 36 % to 70 % of the world’s

663

wetlands (up to 210,000 km2) could be lost by the 2080s’ surely remains true (and with the total

664

global wetland area reported here the 70% would equate to 529 x103 km2).

665

This paper emphasises the importance of lateral accommodation space in mitigating high

666

rates of wetland loss under high rates of sea-level rise. Such a finding gives support to those

667

management strategies that aim to create or re-create space into which coastal wetlands can

668

retreat landwards under sea level forcing (e.g. UK: Rupp, 2010; Dawson et al., 2011; Canada: Djeza

669

et al., 2011; Australia: Abel et al., 2011). However, it is also clear that in many localities such set-

670

back is not possible, either because of existing human occupation and development (e.g. McLeod et

671

al., 2011) or because natural topographic settings are often not conducive to such migration. Thus,

672

for example, the rapid onshore steepening of coastal profiles inland from wetland fringes along

673

most of the Californian coast severely limits migration sites along this coast (Committee on Sea

674

Level Rise in California, Oregon and Washington, 2012).

675

The broad-scale nature of the model presents a major challenge to model calibration and

676

validation and this in turn depends on the development of more systematic national to regional

677

scale assessments of wetland behaviour. A key model output has been the derivation of a series of

678

habitat-specific wetland response curves describing the transition between different wetland types.

679

Based on recently published estimates for habitat-specific regional wetland change, the calibration

680

of these curves provides an important focus for the linking of regional scale assessments to global

681

scale wetland modelling. Similarly, such regional scale assessments, either empirical or modelled,

682

are necessary for model validation and, within this study, have been shown to give important

683

information on the temporal and spatial accuracy of the DIVA_WCM. An appropriate choice of

684

calibration  and  validation  data  smooths over  the  fine  scale  variability  in  wetland  response to

685

environmental forcing which characterises the vegetation ‘mosaic’ of many wetlands and of which

686

there are many studies. Model validation should be taken over long timescales so as not to give

687

undue weight to the impacts of individual high-magnitude events or even long-term cycles in tidal

688

flooding regimes. However, this remains a considerable challenge because of the lack of suitable

689

large-scale data that explicitly address this question in a truly quantitative manner. Progress in

690

better understanding wetland response to sea-level rise requires continued improvement of the

691

underlying  datasets,  and  studies  across  scales  from  local  to  global,  with  bridging  regional

692

693

694

695

696

assessments as utilised in this study.

# Conclusions

697

The DIVA_WCM has been developed to better identify the vulnerability of coastal wetlands

698

at the macro-scale, over a timescale of up to 100 years and at global, continental and national

699

scales spatial scales. The utility of the model is therefore directed towards decision-makers and

700

analysts interpreting and evaluating wetland vulnerability to climate change on these scales. It gives

701

a new and important perspective on coastal wetland behaviour at a spatial scale where existing

702

models are limited and data is surprisingly poor. Here we focus on the global results as a diagnostic

703

output.

704

The  modelling  approach  described  in  this  paper,  which  considers  three  environmental

705

factors, suggests that the potential rates of global coastal wetland loss over the coming decades to

706

2100 are substantial. Countering these potential losses will require both climate mitigation (a global

707

response) to minimise sea-level rise, and promotion of accommodation space and sediment supply

708

(a  regional  response)  to  promote  wetland  survival.  Collectively,  these  measures  could  greatly

709

reduce losses if applied at a sufficient scale but some net loss appears inevitable given current

710

trends and lock-in to some sea-level rise. Given the now clear ecosystem service value of coastal

711

wetlands, and the magnitude of these long-term predicted losses, wetland management should

712

become an environmental policy priority, even in areas where the existing threat from sea-level rise

713

appears   currently   minimal.   Results   from   DIVA_WCM   suggest   that   developing   a   greater

714

understanding of the specific geomorphic natural slope settings which result in greatest levels of

715

forcing on wetland loss would be useful in developing coastal wetland protection policy.

716

Further   development   of   the   model   is   now   needed   to   better   assess   the   role   of

717

ecogeomorphic feedbacks to see if the incorporation of these terms fundamentally affects model

718

outcomes (e.g. Kirwan et al., 2010; Shile et al., 2014).	In addition, independent validation of the

719

results  predicted  by  the  DIVA_WCM,  particularly  across  different  geographical  regions  and

720

timeframes, remains an important but difficult task. It is hoped that this broad-scale modelling of

721

coastal wetlands will stimulate both the quantity and approach of field measurements, such that

722

the data required to validate this type of model become more widely available. Changes to coastal

723

724

725

wetlands need to be evaluated at multiple scales, including the macro-scale considered here.

726

# Acknowledgements

727

The  authors  gratefully  acknowledge  funding  from  the  European  Union  under  contract

728

number EVK2-2000-22024. They thank all their partners in the DINAS-COAST project Dynamic and

729

Interactive Assessment of National, Regional and Global Vulnerability of Coastal Zones to Climate

730

Change and Sea-level rise. We are grateful to staff at UNEP-WCMC for generous access to evolving

731

databases on global coastal wetland extent: Jon Hutton, Hannah Thomas, Jan-Willem van Bochove,

732

Simon  Blyth  and  Chris  McOwen.  Current  wetland  databases  held  at  WCMC  build  upon  the

733

734

735

736

737

738

739

740

741

742

743

744

745

746

747

748

749

pioneering efforts of Mark Spalding and Carmen Lacambra.

750

751

752

# References

Abel, N., Gorddard, R., Harman, B., Leitch, A., Langridge, J., Ryan, A., Heyenga, S., 2011. Sea level

753

rise, coastal development and planned retreat: Analytical framework, governance principles and an

754

Australian case study. Environmental Science and Policy 14, 279-288.

755

756

doi: 10.1016/j.envsci.2010.12.002.

757

Adam, P., 2002. Saltmarshes in a time of change. Environmental Conservation 29, 39–61.

758

759

doi : 10.1017/S0376892902000048.

760

Akumu, C.E., Pathirana, S., Baban, S., Bucher, D., 2011. Examining the potential impacts of sea level

761

rise on coastal wetlands in north-eastern NSW, Australia. Journal of Coastal Conservation 15, 15-22.

762

763

doi: 10.1007/s11852-010-0114-3.

764

Alongi, D.M., 2008. Mangrove forests: Resilience, protection from tsunamis, and responses to

765

global	climate	change.	Environmental	Conservation	29,	331-349.	doi:

766

767

10.1017/S0376892902000231.

768

Allen, J.R.L., 1990. Salt-marsh growth and stratification: a numerical model with special reference to

769

the   Severn   Estuary,   southwest   Britain.	Marine   Geology   95,   77-96.   doi:   10.1016/0025-

770

771

3227(90)90042-I.

772

Atwater, B.F., 1987. Evidence for great Holocene earthquakes along the outer coast of Washington

773

State. Science 236, 942-944. doi: 10.1126/ science.236.4804.942.

774

775

776

Ball, M.C., 1988. Ecophysiology of mangroves. Trees 2, 129–142. doi: 10.1007/BF00196018.

777

Barbier, E.B., Koch, E.W., Silliman, B.R., Hacker, S.D., Wolanski, E., Primavera, J., Granek, E.F.,

778

Polasky, S., Aswani, S., Cramer, L.A., Stoms, D.M., Kennedy, C.J., Bael, D., Kappel, C.V., Perillo,

779

G.M.E., Reed, D.J., 2008. Coastal ecosystem-based management with nonlinear ecological functions

780

781

and values. Science 321, 319–323. doi: 10.1126/science.1150349.

782

Barbier, E.B., Hacke,r S.D., Kennedy, C., Koch, E.W., Stier, A.C., Silliman, B.R., 2011. The value of

783

estuarine and coastal ecosystem services. Ecological Monographs 81, 169–193. doi: 10.1890/10-

784

785

1510.1.

786

Barbier, E.B., Georgiou, I.Y., Enchelmeyer, B., Reed, D.J., 2013. The value of wetlands in protecting

787

Southeast	Louisiana	from	hurricane	storm	surges.	PLoS	ONE	8(3),	e58715.	doi:

788

789

10.1371/journal.pone.0058715.

790

Brinson, M.M., Christian, R.R., Blum, L.K.,  1995. Multiple states in the sea-level induced transition

791

792

from terrestrial forest to estuary. Estuaries 18, 648-659. doi: 10.2307/1352383.

793

Buchanan, P.A., Morgan, T.L., 2014. Summary of suspended-sediment concentration data, San

794

Francisco Bay, California, water year 2010: U.S. Geological Survey Data Series 808, 30 pp.

795

796

Available at: http://dx.doi.org/10.3133/ds808.

797

Cahoon, D.R., Reed, D.J., 1995. Relationships among marsh surface topography, hydroperiod and

798

soil accretion in a deteriorating Louisiana salt marsh. Journal of Coastal Research 11, 357-369. doi:

799

800

10.2112/JCOASTRES-D-11-00141.1.

801

Cahoon, D.R., Lynch, J., Hensel, P., Boumans, R., Perez, B., Segura, B., Day, J. Jr., 2002. High-

802

precision   measurements   of   wetland   sediment   elevation:   I   recent   improvements   to   the

803

sedimentation-erosion	table.	Journal	of	Sedimentary	Research	72,	730–733.	doi:

804

805

10.1306/020702720730.

806

Cahoon,  D.R.,  Hensel,  P.,  Rybczyk,  J.,  McKee,  K.L.,  Proffitt,  C.E.,  Perez,  B.A.,  2003.  Mass  tree

807

mortality leads to mangrove peat collapse at Bay Islands, Honduras after Hurricane Mitch. Journal

808

809

of Ecology 91, 1093-1105. doi: 10.1046/j.1365-2745.2003.00841.x.

810

Cahoon, D.R., Hensel, P.F., Spencer, T., Reed, D.J., McKee, K.L., Saintilan, N., 2006. Coastal Wetland

811

Vulnerability  to  Relative  Sea-Level  Rise:  Wetland  Elevation  Trends  and  Process  Controls.  In:

812

Verhoeven, J.T.A., Beltman, B., Bobbink, R., Whigham, D.F., (Eds.), Wetlands and Natural Resource

813

814

Management. Springer-Verlag, Berlin Heidelberg, pp. 271-292.

815

Cahoon, D.R., Perez, B.C., Segura, B.D., Lynch J.C.,  2011. Elevation trends and shrink-swell response

816

of wetland soils to flooding and drying. Estuarine, Coastal and Shelf Science 91, 463-474. doi:

817

818

10.1016/j.ecss.2010.03.022.

819

Cherry, J. A., McKee, K. L., Grace, J. B., 2009. Elevated CO2  enhances biological contributions to

820

elevation change in coastal wetlands by offsetting stressors associated with sea-level rise. Journal

821

822

of Ecology 97, 67–77. doi: 10.1111/j.1365-2745.2008.01449.x.

823

Chmura, G.L., 2011. What do we need to assess the sustainability of the tidal salt marsh carbon

824

825

sink? Ocean and Coastal Management. doi: 10.1016/j.ocecoaman.2011.09.006

826

Christiansen T., Wiberg P.L., Milligan T.G., 2000. Flow and sediment transport on a tidal salt marsh

827

828

829

830

surface. Estuarine, Coastal and Shelf Science 50, 315-331. doi: 10.1006/ecss.2000.0548.

Church, J.A, Clark, P.U., Cazenave, A., Gregory, J.M., Jevrejeva, S., Levermann, A., Merrifield, M.A.,

831

Milne, G.A., Nerem, S., Nunn, P.D., Payne, A.J., Pfeffer, W.T., Stammer, D., Unnikrishnan, A.S., 2013.

832

Sea Level Change. In: Stocker, T.F., Qin, D., Plattner, G.K., Tignor, M., Allen, S.K., Boschung, J.,

833

Nauels, A., Xia, Y., Bex, V., Midgley, P.M., (Eds.), Climate Change 2013: The Physical Science Basis.

834

Contribution of Working Group I to the Fifth Assessment Report of the Intergovernmental Panel on

835

836

Climate Change. Cambridge University Press, Cambridge, pp. 1137-1216.

837

Clough, J.S., Park, R.A., Fuller, R., 2010. SLAMM 6 beta Technical Documentation, Release 6.0 beta.

838

839

Warren Pinnacle Consulting, Ine, USA.

840

Committee on Sea Level Rise in California, Oregon and Washington, 2012. Sea-level rise for the

841

coasts of California, Oregon, and Washington: Past, present, and future. The National Academies

842

Press, Washington, D.C.

843

844

Conner, W.H., Brody, M., 1989. Rising water levels and the future of Southeastern Louisiana swamp

845

846

forests. Estuaries 12, 318-323. doi: 10.2307/1351909.

847

Couvillion, B.R., Beck, H., 2013. Marsh collapse thresholds for coastal Louisiana estimated using

848

elevation and vegetation index data. Journal of Coastal Research, Special Issue 63, 58 – 67. doi:

849

850

10.2112/SI63-006.1.

851

Couvillion, B.R., Steyer, G.D., Wang, H., Beck, H.J., Rybczyk, J.M., 2013. Forecasting the effects of

852

coastal protection and restoration projects on wetland morphology in  coastal Louisiana under

853

multiple environmental uncertainty scenarios. Journal of Coastal Research, Special Issue 67, 29 –

854

855

50. doi: 10.2112/SI_67_3.

856

Craft, C.B., Clough, J., Ehman, J., Joye, S.B., Park, R., Pennings, S.C., Guo, H., Machmuller, M.,  2009.

857

Forecasting the effects of accelerated sea level rise on tidal marsh ecosystem services. Frontiers in

858

859

Ecology and the Environment 7, 73-78. doi: 10.1890/070219.

860

Davidson, N. C., 2014. How much wetland has the world lost? Long-term and recent trends in global

861

862

wetland area. Marine and Freshwater Research 65, 934-941. doi: 10.1071/MF14173

863

Dawson,  R.J.,  Ball,  T.,  Werritty,  J.,  Werritty,  A.,  Hall  J.W.,  Roche,  N.,  2011.  Assessing  the

864

effectiveness  of  non-structural  flood  management  measures  in  the  Thames  Estuary  under

865

conditions of socio-economic and environmental change. Global Environmental Change 21, 628-

866

646. doi: 10.1016/j.gloenvcha.2011.01.013

867

868

Day, J.W., Pont, D., Hensel, P.F., Ibanez, C., 1995. Impacts of sea-level rise on deltas in the Gulf of

869

Mexico and Mediterranean: the importance of pulsing events to sustainability. Estuaries 18, 636-

870

871

647. doi: 10.2307/1352382.

872

Drejza, S., Bernatchez, P., Dugas, C., 2011. Effectiveness of land management measures to reduce

873

coastal  georisks,  eastern  Québec,  Canada.  Ocean  &  Coastal  Management  54,  290–301.  doi:

874

875

10.1016/j.ocecoaman.2011.01.001.

876

Dodd, R.S., Ong, J.E., 2008. Future of mangrove ecosystems to 2025. In: Polunin, N.V.C., (Ed.),

877

Aquatic Ecosystems: Trends and Global prospects. Cambridge University Press, Cambridge, pp. 172-

878

879

187.

880

Fagherazzi, S., Kirwan, M.L., Mudd, S.M., Guntenspergen, G.R., Temmerman, S., D’Alpaos, A., van de

881

Koppel, J., Rybczyk, J.M., Reyes, E., Craft, C., Clough, J., 2012. Numerical models of salt marsh

882

evolution: Ecological, geomorphic, and climatic factors. Review of Geophysics 50, RG1002. doi:

883

884

10.1029/2011RG000359.

885

Feagin, R.A., Martinez, M.l., Mendoza-Gonzalez, G., Costanza, R.,  2010. Salt marsh zonal migration

886

and ecosystem service change in response to global sea level rise: A case study from an urban

887

region. Ecology and Society 15, 14.

888

889

Available at: http://www.ecologyandscoiety.org/vol15/iss4/art14/

894

Friedrichs, C.T., Perry, J.E.,   2001. Tidal salt marsh morphodynamics: A synthesis. Journal of Coastal

895

896

Research, Special Issue 27, 7-37.

897

Friess, D.A., Krauss, K.W., Horstman, E.M., Balke, T., Bouma, T.J., Galli, D., Webb, E.L., 2012. Are all

898

intertidal  wetlands  naturally  created  equal?  Bottlenecks,  thresholds  and  knowledge  gaps  to

899

mangrove  and  saltmarsh  ecosystems.  Biological  Reviews  87,  346-366.  doi:  10.1111/j.1469-

900

901

185X.2011.00198.x.

902

Friess, D.A., Webb, E.L., 2014. Variability in mangrove change estimates and implications for the

903

assessment of ecosystem service provision. Global Ecology and Biogeography 23, 715-725. doi:

904

905

10.1111/geb.12140.

906

French,  J.R.,  2006.  Tidal  marsh  sediment  trapping  and  resilience  to  environmental  change:

907

exploratory   modelling   of   tidal,   sea-level   and   sediment   supply   forcing   in   predominantly

908

909

allochthonous systems. Marine Geology 235, 119-136. doi: 10.1016/j.margeo.2006.10.009.

910

French, J.R., Reed, D.J., 2001. Physical contexts for saltmarsh conservation. In: Warren, A., French,

911

J.R., (Eds.), Habitat conservation: Managing the physical environment. Chichester, John Wiley, pp.

912

913

179-228.

914

915

916

917

French, J.R., Spencer, T., 1993. Dynamics of sedimentation in a tide-dominated backbarrier salt marsh, Norfolk, UK. Marine Geology 110, 315-331. doi: 10.1016/0025-3227(93)90091-9 .

918

Gedan,  K.B.,  Silliman,  B.R.,  Bertness,  M.D.,  2009.  Centuries  of  human  change  in  salt  marsh

919

ecosystems.	Annual	Review	of	Marine	Science	1,	117-141.	doi:

920

921

10.1146/annurev.marine.010908.163930.

922

Gedan, K.B., Kirwan, M.L., Wolanski, E., Barbier, E.B., Silliman, B.R., 2011. The present and future

923

role of coastal vegetation in protecting shorelines: answering recent challenges to the paradigm.

924

925

Climatic Change 106, 7–29. doi: 10.1007/s10584-010-0003-7.

926

Geselbracht, L., Freeman, K., Kelly, E., Gordon, D.R., Putz, F.E., 2011. Retrospective and prospective

927

model simulations of sea level rise impacts on Gulf of Mexico coastal marshes and forests in

928

929

Waccassa Bay, Florida. Climatic Change 107, 35–57. doi: 10.1007/s10584-011-0084-y.

930

Giri, C., Ochieng, E., Tieszen, L., Zhu, Z., Singh, A., Loveland, T., Masek, J., Duke, N., 2011. Status and

931

distribution of mangrove forests of the world using earth observation satellite data. Global Ecology

932

933

and Biogeography 20, 154-159. doi: 10.1111/J.1466-8283.2010.00584.

934

Glick, P., Clough, J., Polaczyk, A., Couvillion, B., Nunley, B., 2013. Potential effects of sea-level rise

935

on coastal wetlands in Southeastern Louisiana. Journal of Coastal Research, Special Issue 63, 211 –

936

937

233. doi: 10.2112/SI63-0017.1.

938

Heimann,  D.C.,  Sprague,  L.A.,  Blevins,  D.W.,  2011.  Trends  in  suspended-sediment  loads  and

939

concentrations  in  the  Mississippi  River  Basin,  1950–2009.  U.S.  Geological  Survey  Scientific

940

941

Investigations Report 2011–5200, 33 pp. Available at: http://pubs.usgs.gov/sir/2011/5200/

942

Hinkel, J. (2005). DIVA: an iterative method for building modular integrated models. In: Advances in

943

944

Geosciences, 4, 45–50.

945

Hinkel, J., Klein, R.J.T., 2009. The DINAS-COAST project: developing a tool for the dynamic and

946

interactive assessment of coastal vulnerability. Integrating knowledge to assess coastal vulnerability

947

to sea-level rise: The development of the DIVA tool. Global Environmental Change 19, 384-395. doi:

948

949

10.1016/j.gloenvcha.2009.03.002.

950

Hinkel,  J.,  Nicholls,  R.  J.,  Vafeidis,  A.T.,  Tol,  R.S.J.,  Avagianou,  T.,  2010.  Assessing  risk  of  and

951

adaptation  to  sea-level  rise  in  the  European  Union:  an  application  of  DIVA.  Mitigation  and

952

953

Adaptation Strategies for Global Change 15, 703-719. doi:10.1007/s11027-010-9237-y.

954

Hinkel,  J.,  van  Vuuren,  D.P.,  Nicholls,  R.J.,  Klein,  R.J.T.,	2013.  The  effects  of  mitigation  and

955

adaptation   on   coastal   impacts   in   the   21st   century.   Climatic   Change   117,   783–794.   doi:

956

957

10.1007/s10584-012-0564-8.

958

Hinkel, J., Lincke, D., Vafeidis, A.T., Perrette, M., Nicholls, R.J., Tol, R.S.J., Marzeion, B., Fettweis, X.,

959

Ionescu, C., Levermann, A., 2014. Coastal flood damage and adaptation cost under 21st century

960

sea-level	rise.	Proceedings	of	the	National	Academy	of	Sciences,	USA.	doi:

961

10.1073/pnas.1222469111.

962

963

Hoozemans,  F.M.J.,  Marchand,  M.,  Pennekamp,  H.A.,  1993.  A  Global  Vulnerability  Analysis:

964

Vulnerability Assessment for Population, Coastal Wetlands and Rice Production on a Global Scale,

965

966

2nd Edition. Delft Hydraulics, Delft and The Hague.

967

Howard, J.D., Frey, R.W., 1985. Physical and biogenic aspects of backbarrier sedimentary

968

969

sequences, Georgia coast, U.S.A. Marine Geology, 63, 77--127. doi: 10.1016/0025-3227(85)90080-5.

970

Huiskes, A.H.L., 1990. Possible effects of sea level changes on saltmarsh vegetation. In: Beukema,

971

J.J., Wolff, W.J., Brouns, J.J.W.M. (Eds.), Expected effects of climatic change on marine coastal

972

973

ecosystems. Kluwer Academic Publishers, Dordrecht. pp. 167–172.

974

IIASA (International Institute for Applied Systems Analysis) 2012 Shared Socioeconomic Pathways

975

976

Database. Available at: https://secure.iiasa.ac.at/web-apps/ene/SspDb.

977

IPCC SRES, 2000. Special Report on Emissions Scenarios: A special report of Working Group III of the

978

Intergovernmental  Panel  on  Climate  Change  (Nakićenović,  N.,  Swart,  R.,  (eds.))  Cambridge

979

980

University Press, Cambridge.

981

Kirwan, M.L., Guntenspergen, G.R., 2009. Accelerated sea-level rise – a response to Craft et al..

982

983

Frontiers in Ecology and the Environment 7, 126–127. doi: 10.1890/09.WB.005.

984

Kirwan,  M.L.,  Guntenspergen,  G.R.,  2010.  Influence  of  tidal  range  on  the  stability  of  coastal

985

marshland. Journal of Geophysical Research 115, F02009. doi: 10.1029/2009JF001400.

986

987

Kirwan, M. L., Guntenspergen, G. R., 2012. Feedbacks between inundation, root

988

production, and shoot growth in a rapidly submerging brackish marsh. Journal of Ecology

989

990

100, 764–770. doi: 10.1111/j.1365-2745.2012.01957.x.

991

Kirwan, M.L., Guntenspergen, G.R., D’Alpaos, A., Morris J.T., Mudd, S.M., Temmerman, S., 2010.

992

Limits on the adaptability of coastal marshes to rising sea level, Geophysical Research Letters 37,

993

994

L23401. doi: 10.1029/2010GL045489.

995

Kirwan, M.L., Mudd, S.M., 2012. Response of salt-marsh carbon accumulation to climate change.

996

997

Nature 489, 550–553. doi: 10.1038/nature11440.

998

Koch,  E.W.,  Barbier,  E.B.,  Silliman,  B.R,  Reed,  D.J.,  Perillo,  G.M.E.,  Hacker,  S.D.,  Granek,  E.F.,

999

Primavera, J.H., Muthiga, N.,  Polasky, S., Halpern, B.S.,  Kennedy, C.J., Kappel, C.V., Wolanski, E.,

1000

2009. Non-linearity in ecosystem services: temporal and spatial variability in coastal protection.

1001

1002

Frontiers in Ecology and the Environment 7, 29–37. doi: 10.1890/080126.

1003

Krauss, K.W., McKee, K.L., Lovelock, C.E. , Cahoon, D.R., Saintilan, N., Reef, R., Chen, L., 2014. How

1004

1005

mangrove forests adjust to rising sea level. New Phytologist 202, 19–34. doi: 10.1111/nph.12605.

1006

Lagarias, J.C., Reeds, J.A., Wright, M.H., Wright, P.E., 1998. Convergence properties of the Nelder-

1007

Mead  Simplex  Method  in  low  dimensions.  SIAM  Journal  of  Optimization  9,  112-147.  doi:  10

1008

1009

.1137/S1052623496303470.

1010

Langley, J. A., McKee, K.L., Cahoon, D.R., Cherry, J.A., Megonigal, J.P., 2009. Elevated CO2 stimulates

1011

marsh elevation gain, counterbalancing sea‐level rise. Proceedings of the National Academy of

1012

1013

Sciences, USA 106, 6182–6186. doi: 10.1073/pnas.0807695106.

1014

Leadley, P. W., Krug, C. B., Alkemade, R., Pereira, H. M., Sumaila U. R., Walpole, M., Marques, A.,

1015

Newbold, T., Teh, L. S. L., van Kolck, J., Bellard, C., Januchowski-Hartley, S. R., Mumby, P. J., 2014.

1016

Progress  towards  the  Aichi  Biodiversity  Targets:  An  assessment  of  biodiversity  trends,  policy

1017

scenarios and key actions (CBD Technical Series No. 78). Secretariat of the Convention on Biological

1018

Diversity, Montreal, Canada. Available at: http://www.cbd.int/doc/publications/

1019

1020

cbd-ts-78-en.pdf

1021

Loder,  N.M.,  Irish,  J.L,  Cialone,  M.A.,  Wamsley,  T.V.,  2009.  Sensitivity  of  hurricane  surge  to

1022

morphological parameters of coastal wetlands. Estuarine Coastal and Shelf Science 84, 625–636.

1023

1024

doi: 10.1016/j.ecss.2009.07.036.

1025

Lovelock, C.E., Cahoon, D.R., Friess, D.A., Guntenspergen, G.R., Krauss, K.W., Reef, R., Rogers, K.,

1026

Saunders, M.L., Sidik, F., Swales. A., Saintilan. N., Thuyen, L.X., Triet, T., 2015. The vulnerability of

1027

Indo-Pacific mangrove forests to sea-level rise. Nature 526, 559-563

1028

1029

doi:10.1038/nature15538.

1030

Mariotti, G., Carr, J., 2014. Dual role of salt marsh retreat: Long-term loss and short-term resilience,

1031

1032

Water Resources Research 50, 2963–2974, doi: 10.1002/2013WR014676.

1033

Martin, J. F., Reyes, E., Kemp, G.P., Mashriqui, H., Day, J.W.Jr., 2002. Landscape modeling of the

1034

Mississippi Delta. Bioscience 52, 357-365.

1035

1036

doi:     10.1641/0006-3568(2002)052[0357:LMOTMD]2.0.CO;2

1037

McFadden, L., Spencer T., Nicholls, R.J., 2007. Broad-scale modelling of coastal wetlands: what is

1038

1039

required? Hydrobiologia 577, 5-15. doi: 10.1007/s10750-006-0413-8.

1040

McIvor, A.L., Spencer, T., Möller, I. and Spalding. M., 2012. Storm surge reduction by mangroves.

1041

Natural Coastal Protection Series: Report 2. Cambridge Coastal Research Unit Working Paper 41.

1042

Published by  The Nature  Conservancy  and  Wetlands  International.  35 pages. ISSN  2050-7941.

1043

Available	at:	http://www.naturalcoastalprotection.org/documents/storm-surge-reduction-by-

1044

1045

mangroves.

1046

McKee,  K.L.,  Mendelssohn,  I.  A.,  1989.  Response  of  a  freshwater  marsh  plant  community  to

1047

increased  salinity  and  increased  water  level.  Aquatic  Botany  34,  301-316.  doi:  10.1016/0304-

1048

1049

3770(89)90074-0.

1050

McLeod, E., Chmura, G.L., Bouillon, S., Salm, R., Björk, M., Duarte, C.M., Lovelock, C. E., Schlesinger,

1051

W.H., Silliman, B.R., 2011. A blueprint for blue carbon: toward an improved understanding of the

1052

role of vegetated coastal habitats in sequestering CO2. Frontiers in Ecology and the Environment 9,

1053

1054

552–560. doi: 10.1890/110004.

1055

Mendelssohn I., Morris J.T., 2000. Ecophysiological controls on the growth of Spartina alterniflora.

1056

In: Weinstein, M.P., Kreeger, D.A., (Eds.), Concepts and controversies in tidal marsh ecology. Kluwer

1057

1058

Academic Publishers, Dordrecht, pp. 59-80.

1059

Middleton, B.A., McKee, K.L., 2001. Degradation of mangrove tissues and implications for peat

1060

formation  in  Belizean  island  forests.  Journal  of  Ecology  89,  818-828.  doi:  10.1046/j.0022-

1061

1062

0477.2001.00602.x.

1063

Millennium Ecosystem Assessment, 2005. Ecosystems and human well-being: wetlands and water

1064

1065

synthesis. World Resources Institute, Washington, DC.

1066

Morris, J.T., Sundareshwar, P.V., Nietch, C.T., Kjerfve, B., Cahoon, D.R., 2002.  Responses of coastal

1067

1068

wetlands to rising sea-level. Ecology 83, 2869-2877. doi: 10.1890/0012-9658.

1069

Mossman, H.L., Davy, A.J., Grant, A., 2012. Quantifying local variation in tidal regime using depth-

1070

logging	fish	tags.	Estuarine,	Coastal	and	Shelf	Science	96,	122-128.	doi:	10.

1071

1072

.1016/j.ecss.2011.10.019.

1073

Mudd, S.M., Howell, S.M., Morris, J.T., 2009. Impact of dynamic feedbacks between sedimentation,

1074

sea-level   rise,   and   biomass   production   on   near-surface   marsh   stratigraphy   and   carbon

1075

1076

accumulation. Estuarine, Coastal and Shelf Science 82, 377-389. doi: 10.1016/j.ecss.2009.01.028.

1077

Murdiyarso,  D.,  Purbopuspito,.  J.,  Kauffman,  J.B.,  Warren,  M.W.,  Sasmito,  S.D.,  Donato,  D.C.,

1078

Manuri, S., Krishnawati, H., Taberima, S., Kurnjanto, S., 2015. The potential of Indonesian mangrove

1079

forest	for	global	climate	change	mitigation.	Nature	Climate	Change	5,1089-1092	doi:

1080

1081

10.1038/nclimate2734.

1082

NGDC (National Geophysical Data Centre), 2001.   NGDC CD-ROM, ETOPO2 Global 2’ Elevations.

1083

1084

Available at: http://www.ngdc.noaa.gov/mgg/fliers/01mgg04.html

1085

Nicholls, R.J., 2004. Coastal flooding and wetland loss in the 21st century: changes under the SRES

1086

climate	and	socio-economic	scenarios.	Global	Environmental	Change	14,	69–86.	doi:

1087

1088

10.1016/j.gloenvcha.2003.10.007.

1089

Nicholls, R.J., Hoozemans F.M.J., Marchand M., 1999. Increasing flood risk and wetland losses due

1090

to global sea-level rise: regional and global analyses. Global Environmental Change 9, S69-S87. doi:

1091

1092

10.1016/S0959-3780(99)00019-9.

1093

Nicholls, R.J., Wong, P.P., Burkett, V.R., Codignotto, J.O., Hay, J.E., McLean, R.F., Ragoonaden, S.,

1094

Woodroffe,  C.D.,  2007.  Coastal  systems  and  low-lying  areas.	In:  Parry,  M.L.,  Canziani,  O.F.,

1095

Palutikof, J.P., van der Linden, P.J., Hanson, C.E., (Eds.),  Climate Change 2007: Impacts, Adaptation

1096

and  Vulnerability.  Contribution  of  Working  Group  II  to  the  Fourth  Assessment  Report  of  the

1097

1098

Intergovernmental Panel on Climate Change. Cambridge University Press, Cambridge, pp. 315-356.

1099

Nicholls, R.J., Woodroffe, C.D., Burkett, V., Hay, Wong, P., Nurse, L., 2011. Scenarios for coastal

1100

vulnerability assessment. In: van den Belt, M., Costanza, R., (Eds.), Treatise on Estuarine and Coastal

1101

Science, Volume 12: Ecological Economics of Estuaries and Coasts. Elsevier, Amsterdam, pp. 289-

1102

303.

1103

1104

Peltier,  W.R.,  2000a.  Global  glacial  isostatic  adjustment  and  modern  instrumental  records  of

1105

relative sea level history. In: Douglas B.C., Kearney MS, Leatherman SP (Eds.) Sea level rise; history

1106

1107

and consequences. Academic Press, San Diego, pp. 65-95.

1108

Peltier,  W.R.,  2000b.  ICE4G  (VM2)  glacial  isostatic  adjustment  corrections.  In:  Douglas,  B.C.,

1109

Kearney, M.S., Leatherman, S.P., (Eds.), Sea level rise; history and consequences. Academic Press,

1110

1111

San Diego, on CD.

1112

Phillips, J.D., 1986. Coastal Submergence and marsh fringe erosion.  Journal of Coastal Research 2,

1113

1114

427-436.

1115

Rebelo, L.M., Finlayson, C.M., Nagabhatla, N., 2009. Remote sensing and GIS for wetland inventory,

1116

mapping  and  change  analysis.  Journal  of  Environmental  Management  90,  2144–2153.  doi:

1117

1118

10.1016/j.jenvman.2007.06.027.

1119

Reed, D.J., 1995. The response of coastal marshes to sea-level rise: survival or submergence? Earth

1120

1121

Surface Processes and Landforms 20, 39-45.

1122

Reed, D.J., Bishara, D.A., Cahoon, D.R., Donnelly, J., Kearney, M., Kolker, A.S., Leonard, L.L., Orson,

1123

R.A., Stevenson, J.C., 2008. Site-specific scenarios for wetlands accretion as sea-level rises in the

1124

Mid-Atlantic  Region.  Section  2.1.  In:  Titus,  J.G.,  Strange,  E.M.,  (Eds.),  Background  documents

1125

supporting climate change science program synthesis and assessment product 4.1 (EPA 430R07004).

1126

1127

United States Environmental Protection Agency (EPA), Washington, D.C.

1128

Reyes, E.,  2009. Wetland landscape spatial models.  In: Perillo, G.M.E., Wolanski, E., Cahoon, D.R.,

1129

Brinson,  M.M.,  (Eds),  Coastal  Wetlands:  An  Integrated  Ecosystem  Approach.  Elsevier  Science,

1130

1131

Amsterdam, pp. 885-908.

1132

Reyes,  E.,  White,  J.L.,  Martin,  J.F.,  Kemp,  G.P.,  Day,  J.W.,  Aravamuthan,  W.,  2000.  Landscape

1133

modeling  of  coastal  habit  change  in  the  Mississippi  delta.	Ecology  818,  2331-2349.  doi:

1134

1135

10.2307/177118.

1136

Rooth,  J.E.,  Stevenson, J.C.,  Cornwell,  J.C.,  2003.  Increased  sediment  accretion  rates  following

1137

1138

invasion by Phragmites australis: The role of litter. Estuaries 26, 475-483. doi: 10.1007/BF02823724.

1139

Ross, M.S., O’Brien, J.J., Ford, R.G., Zhang, K., Morkill, A., 2009. Disturbance and the rising tide: the

1140

challenge  of  biodiversity  management  for  low  island  ecosystems.  Frontiers  in  Ecology  and

1141

1142

Environment 7, 471-478. doi: 10.1890/070221.

1143

Rupp, S. G., 2010. The status and future of managed realignment of coastal flood plains in western

1144

1145

Europe: A comparative study. Unpublished PhD thesis, University of Southampton.

1146

Saintilan, N., Rogers, K., Mazumder, D., Woodroffe. C., 2013. Allochthonous and autochthonous

1147

contributions  to  carbon  accumulation  and  carbon  store  in  southeastern  Australian  wetlands.

1148

1149

Estuarine, Coastal and Shelf Science 128, 84–92. doi: 10.1016/j.ecss.2013.05.010.

1150

Saintilan, N., Rogers, K., McKee, K., 2009. Salt marsh-mangrove interactions in Australasia and the

1151

Americas. In: Perillo, G.M.E., Wolanski, E., Cahoon, D.R., Brinson, M. (Eds.), Coastal Wetlands: an

1152

1153

Integrated Approach. Elsevier Science, Amsterdam, pp. 855–884.

1154

Shepard, C.C., Crain, C.M., Beck, M.W., 2012. The protective role of coastal marshes: a systematic

1155

1156

review and meta-analysis. PLoS ONE 6, e27374. doi: 10.1371/journal.pone.0027374.

1157

Shile, L.M., Callaway, J.C., Morris, J.T., Stralberg, D., Parker, V.T., Kelly, M., 2014. Modelling tidal

1158

marsh  distribution  with sea-level rise: Evaluating the role  of  vegetation, sediment  and upland

1159

1160

habitat in marsh resiliency. PLOS ONE, e88760. doi: 10.1371/journal.pone.0088760.

1161

Sklar, F.H.,  Costanza, R.,  Day, J.W. Jr.,	1985. Dynamic spatial  simulation  modelling of  coastal

1162

1163

wetland habitat succession. Ecological Modelling 29, 261-281. doi: 10.1016/0304-3800(85)90056-0.

1164

1165

Spalding, M., Mami K., Collins L., 2010. World Atlas of Mangroves. Earthscan, London.

1166

Spencer, T., Möller, I., 2013. Mangrove Systems. In: Shroder, J.F., (Ed.), Treatise on Geomorphology,

1167

1168

Volume 10, Academic Press, San Diego, pp. 360-391.

1169

Stevenson, J.C., Ward, L.G., Kearney, M.S.,  1986. Vertical accretion in marshes with varying rates of

1170

1171

sea-level rise. In: Wolfe D.A., (Ed.), Estuarine variability. Academic Press, Orlando. pp.241-259.

1172

Stralberg, D., Brennan, M., Callaway, J.C., Wood, J.K., Schile, L.M., Jongsomjit, D., Kelly, M., Parker,

1173

V.T., Crooks, S., 2011. Evaluating tidal marsh sustainability in the face of sea-level rise: a hybrid

1174

modeling	approach	applied	to	San	Francisco	Bay.	PloS	ONE	6,	e27388.	doi:

1175

1176

10.1371/journal.pone.0027388.

1177

Syvitski, J.P.M., Vörösmarty, C.J., Kettner, A.J., Green, P., 2005. Impact of humans on the flux of

1178

terrestrial	sediment	to	the	global	coastal	ocean.	Science	308,	376–380.	Doi:

1179

1180

10.1126/science.1109454.

1181

Syvitski, J.P.M., Kettner, A.J., Overeem, I., Hutton,  E.W.H., Hannon, M.T., Brakenridge, G.R., Day, J.,

1182

Vörösmarty, C., Saito, Y., Giosan, L., Nicholls, R.J., 2009. Sinking deltas due to human activities.

1183

1184

Nature Geoscience 2, 681 – 686. doi: 10.1038/ngeo629.

1185

Takekawa,  J.Y.,  Thorne,  K.M.,  Buffington,  K.J.,  Spragens,  K.A.,  Swanson,  K.M.,  Drexler  J.Z.,

1186

Schoellhamer, D.H.,  Overton,  C.T.,  Casazza  M.L.,  2013.  Final  report  for  sea-level  rise response

1187

modeling for San Francisco Bay estuary tidal marshes. U.S. Geological Survey Open File Report

1188

1189

1190

2012- 1081. U.S. Geological Survey, Reston, Virginia, U.S.A.

1191

Traill, L.W., Perhans, K., Lovelock, C.E., Prohaska, A., McFallan, S., Rhodes, J.R., Wilson, K.E., 2011.

1192

Managing  for  change:  wetland  transitions  under  sea-level  rise  and  outcomes  for  threatened

1193

1194

species. Diversity and Distributions 17, 1225-1233. doi: 10.1111/j.1472-4642.2011.00807.

1195

Vafeidis, A.T., Nicholls, R.J., McFadden, L., Tol, R.S.J., Hinkel, J., Spencer, T., Grashoff, P.S., Boot, G.,

1196

Klein, R.J.T.,  2008. A new global coastal database for impact and vulnerability analysis to sea-level

1197

1198

rise. Journal of Coastal Research 24, 917-924. doi: 10.2112/06-0725.1.

1199

Webb, E.L., Friess, D.A., Krauss, K.W., Cahoon, D.R., Guntenspergen, G.R., Phelps, J., 2013. A global

1200

standard for monitoring coastal wetland vulnerability to accelerated sea-level rise. Nature Climate

1201

1202

Change 3, 458-465. doi: 10.1038/nclimate1756.

1203

Wong, P.P., Losada, I.J., Gattuso, J.-P., Hinkel, H., Khattabi, A., McInnes, K.L., Saito, Y., Sallenger, A.,

1204

2014. Coastal systems and low-lying areas. In: Climate Change 2014: Impacts, Adaptation, and

1205

Vulnerability. Part A: Global and Sectoral Aspects. Contribution of Working Group II to the Fifth

1206

Assessment Report of the Intergovernmental Panel on Climate Change [Field, C.B. et al. (Eds)].

1207

1208

Cambridge University Press, Cambridge, pp. 361-409.

1209

You,  L-Z.,  2005.  Fine  sediment  resuspension  dynamics  in  a  large  semi-enclosed  bay.  Ocean

1210

1211

1212

1213

1214

Engineering 32, 1982-1993. doi: 10.1016/j.oceaneng.2005.01.006.

1215

1216

# List of Figures

1217

1218

Fig. 1. Model structure in the revised DIVA_WCM wetland loss and transition algorithm.

1219

Fig.  2.  Proportion  of  wetland  loss  versus  wetland  sensitivity  as  measured  by  the  Ecological

1220

Sensitivity Score (ESS; see text for definition). Values of beta: Unvegetated sediments (Sabka and

1221

Mudflat/Sandflat) (0.093); Freshwater Marsh (0.188); Saltmarsh (0.137); and Coastal Forest and

1222

1223

Mangrove Forest (0.074).

1224

Fig. 3. Model of wetland loss and transition between wetland types and open water for low to

1225

moderate  environmental  forcing  (Vulnerability  Score  <  4).  For  high  environmental  forcing

1226

1227

(Vulnerability Score >= 4) all wetland areas are lost to open water.

1228

Fig. 4. DIVA_WCM model outputs vs. wetland loss rates derived from the one WARMER and five

1229

SLAMM studies. In (a) all available data points are displayed (CSLID = DIVA segment number). In (b)

1230

only the central data points are shown. The blue line denotes the 1:1 line (perfect fit). Negative

1231

wetland losses (wetland gains), reported by the SLAMM studies were put to zero for all wetland

1232

1233

types, where no gain is possible (all habitat types except unvegetated low).

1234

Fig.5. Coastal wetland loss in the Indo-Pacific region. A: Predicted relative coastal wetland loss by

1235

DIVA segment. DIVA_WCM simulation with medium sea-level rise scenario (RCP4.5 median, 50 cm

1236

by year 2100) and SSP2 scenario with ‘widespread dikes’; B: Predicted loss of mangrove habitat

1237

under an RCP6 sea level rise scenario of 0.48 m by year 2100, as modelled by Lovelock et al. (2015).

Green indicates mangrove extent in 2011 (after Giri et al., (2011)) and red identifies areas of

1239

predicted mangrove habitat loss to sea level rise.

1240

Fig. 6. Comparison of the qualitative assessment by Reed et al. (2008) (a) with the outputs of the

1242

calibrated DIVA WCM for constant sea-level rise of 3 (b), 5 (c), and 10 (d) mm a-1. (a): Reed et al.

1243

(2008) assumes a current SLR rate of sea-level rise of 3 mm a-1. Future scenarios are a continuation

1244

of the current rate of sea-level rise; the current rate plus 2 mm a-1 (i.e. 5 mm a-1); and the current

1245

rate plus 7 mm a-1. Figures 6(b-d) can be compared to 6(a) as follows:

1246

%Total Loss < 50%: Wetlands will not be converted to open water (“No”)

1247

50%<%Total Loss < 75%: Wetlands will be marginal (“?”)

1248

75%<%Total Loss < 90%: Wetland will be marginal or lost (“Yes?”)

1249

1250

%Total Loss > 90%: Wetlands will be converted to open water (“Yes”)

1251

Fig. 7. Absolute and relative cumulative global coastal wetland loss from 1995 to 2100. Three

1252

scenarios describing the construction of sea dikes are considered: (i) ‘no dikes’; (ii) ‘widespread

1253

dikes’ built according to a ‘demand-for-safety function’, assuming an SSP2 scenario; (iii) ‘maximum

1254

dikes’. For each dike-scenario low (lower dashed), medium (full line) and high (upper dashed) sea-

1255

level rise scenarios are run: Low = RCP2.6 (5% quantile; 29 cm by 2100); medium = RCP4.5 (median;

1256

1257

50 cm); high = RCP8.5 (95% quantile; 110 cm).

1258

Fig. 8. Absolute cumulative global wetland loss from 1995 to 2100 for different combinations of

1259

weights for the three environmental forcing factors (sea-level rise / tidal range (rslr_tidal_score);

1260

sediment supply (sedsup); accommodation space (aspace). See Tables 4 – 7 for more details. Dark

1261

line  colours represent  low weights for accommodation  space, whereas  bright colours indicate

1262

higher weights for accommodation space. Sensitivity runs were conducted assuming A) ‘no dikes’

1263

and high sea level rise (HIG = 110 cm by year 2100); B: ‘maximum dikes’ and high sea level rise; C:

1264

‘no dikes’ and low sea level rise (LOW = 29 cm by year 2100); and D: ‘maximum dikes’ and low sea

1265

1266

level rise.

1267

Fig. 9. Global wetland loss by DIVA segment between 1995 and 2100. A: absolute loss; B: relative

1268

loss. DIVA_WCM simulation with medium sea-level rise scenario (RCP4.5 median, 50 cm by year

1269

1270

1271

1272

2100) and SSP2 scenario with ‘widespread dikes’.

1273

1274

# List of Tables

1275

Table 1. Global mean sea-level rise in 2100 with respect to 1985-2005. Median values, with 5% and

1276

95% quantiles in parentheses. After Hinkel et al. (2014). The DIVA_WCM uses low sea-level rise =

1277

RCP2.6 (5% quantile; 29 cm by 2100); medium sea-level rise = RCP4.5 (median; 50 cm); high sea-

1278

1279

level rise = RCP8.5 (95% quantile; 110 cm).

| Scenario | Model | Steric (cm) | Land-ice (cm) | Total (cm) | Acronym |
| --- | --- | --- | --- | --- | --- |
| RCP2.6 | HadGEM2-ES | 14 | 21 (16, 39) | 35 (29,52) | LOW |
| RCP4.5 | HadGEM2-ES | 18 | 32 (23, 56) | 50 (41,75) | MED |
| RCP8.5 | HadGEM2-ES | 29 | 44 (31, 81) | 72 (60,110) | HIG |

1280

Table 2. Location, habitat, sea-level rise scenario and model characteristics for the six calibration

1281

studies used to calibrate the behavioural  curves used  in the DIVA_WCM. Tidal range data: in

1282

reference	or	for	USA	sites	from	NOAA	Tides	and	Currents

1283

(https://tidesandcurrents.noaa.gov/stations.html?type=Datums).

1284

Suspended sediment concentrations: (1) 2011 annual mean of MERIS geophysical product Total

1285

Suspended	Matter	(TSM)	in	0.017	degree	resolution,	northern	NSW,	Australia

1286

(http://hermes.acri.fr/); (2)   Howard and Frey (1985); (3) Heimann et al., (2011); (4) Buchanan and

1287

1288

Morgan (2014); (5) You (2005).

1289Table 3. Absolute global wetland loss (x103 km2) and relative loss of total global wetland stock (%)

1290

by 2100 under the diking and sea-level rise scenarios (see Table 1 and text for details on the

1291

1292

scenarios employed).

1293

Table 4. Percentage of total wetland area loss at 2100 under different weighting combinations of

1294

(a) three environmental forcing factors and (b) sea-level rise / tidal range and sediment supply only,

1295

given high sea-level rise scenario of 110 cm by 2100 (95% quantile, RCP8.5; Table 1) and ‘no dikes’.

1296

Shaded area = ‘standard’ DIVA_WCM output (see Equation 4, Supplementary Material for details).

1297

1298

Table 5. Percentage of total wetland area loss at 2100 under different weighting combinations of

1299

the three environmental forcing factors given high sea-level rise scenario of 110 cm by 2100 (95%

1300

quantile, RCP8.5; Table 1) and ‘maximum dikes’. Shaded area = ‘standard’ DIVA_WCM output (see

1301

1302

1303

Equation 4, Supplementary Material for details).

Table 6. Percentage of total wetland area loss at 2100 under different weighting combinations of

1304

(a) three environmental forcing factors and (b) sea-level rise / tidal range and sediment supply only,

1305

given low sea-level rise scenario of 29 cm by 2100 (5% quantile, RCP2.6; Table 1) and ‘no dikes’.

1306

1307

Shaded area = ‘standard’ DIVA_WCM output (see Equation 4, Supplementary Material for details).

1308

Table 7. Percentage of total wetland area loss at 2100 under different weighting combinations of

1309

the three environmental forcing factors given low sea-level rise scenario of 29 cm by 2100 (5%

1310

1311

1312

1313

1314

1315

1316

1317

1318

1319

1320

1321

quantile, RCP2.6; Table 1) and ‘maximum dikes’.

1324

1325

1326

1327

1328

1329

1330

1331

1332

1333

1334

1335

1336

1337

# Supplementary Material

Background tables and equations used in the DIVA_WCM algorithm

- Relative sea-level rise and tidal range forcing

We first compute rslr_tidal as:

rslr_d = rslr_annual^1.4 / htidal	Eq.1

where rslr_annual is the annual rise in relative sea level in metres, htidal is the tidal range in metres derived from the LOICZ typology (Maxwell and Buddemeier, 2002) as shown in Table A1.

| Tidal Range classes from LOICZ typology | Tidal Range (metres), LOICZ typology | htidal (Tidal forcing score within the DIVA Wetland Change Model) |
| --- | --- | --- |
| <2 | 0-2.5 | 0.25 |
| 2 | 2.5-3.5 | 1.25 |
| 3 | 3.5-5.0 | 3 |
| 4 | 5.0-6.5 | 6 |
| 5 | >6.5 | 9 |

1338

1339

1340

1341

1342

1343

1344

Table A1 Derivation of tidal forcing scores (representing tidal range) based on the tidal range classes from the LOICZ typology.

Then we convert rsrl_d into an rslr_tidal_score between 1 and 5 based on the 95, 84, 50, 16 percentiles of all rslr_d values where wetlands are reported (assuming a current global SLR of 3 mm a-1). Resulting class values are reported in the following table:

| rslr_d | rslr_tidal_score |
| --- | --- |
| >=0.001121 | 5 |
| >=0.000402 | 4 |
| >=0.000178 | 3 |
| >=0.000044 | 2 |
| >0 | 1 |

1345

1346

1347

1348

1349

1350

1351

1352

1353

1354

Table A2: Assigning a forcing value to the impact of relative sea-level rise and tidal range on wetland vulnerability.

# Lateral accommodation space

We initialize the forcing score for lateral accommodation space based on the coastal slope (degrees) using Table A3.

| Average slope (slopecst, degrees) | Forcing score for lateral accommodation space (aspace) |
| --- | --- |
| >4.5 | 5 |
| >1.5 < 4.5 | 4 |
| > 0.5 < 1.5 | 3 |
| > 0.25 < 0.5 | 2 |
| <0.25 | 1 |

1359

1360

1361

1362

1363

1364

1365

1366

1367

1368

1369

1370

1371

1372

1373

1374

1375

1376

1377

1378

1379

1380

1381

1382

1383

1384

1385

1386

1387

1388

1389

1390

1391

1392

1393

1394

1395

1396

1397

Table A3: Forcing score used to represent the impact of coastal slope on wetland vulnerability. slopecst is the average topographic slope (in degrees) along the segments.

aspace value is initialised using the average topographic slope, derived from the ETOPO2 (NGDC, 2001) dataset. This initialized aspace score is then updated based on the actual computed dike height within each time-step using Equation 2. DIVA_WCM builds sea dikes along the entire coastal segment given sea level and socio-economic forcing following the demand function of safety given in Hinkel et al. (2014).

if (sdikehght > htidal/2 and aspace < 5): aspace = aspace + 0.25	Eq. 2

Where htidal/2 is a critical value of sdikehght defining its functioning as barrier to landward movement of wetland and to flooding. This threshold value is based on expert judgement.

# Sediment supply

External of DIVA we calculate a constant sediment supply factor sedsup based on a variety of biophysical coastal properties.

sedsup = (t * tw) + ((dis + d_dis)/2) * fw) + (gl * glw) + (geo * gew) + (man * mw) + (his *hw)

# Eq. 3

where:

t = Tectonic control parameter tw = Tectonic control weighting

dis = Annual river discharge parameter

d_dis = Distance from point of discharge parameter fw = Fluvial weighting

gl = Glacial limit parameter glw = Glacial limit weighting

geo = Geomorphic setting parameter gew = Geomorphic setting weighting

man = Management parameter (presence or absence of sea dikes) mw = Management weighting

his = History of resource exploitation parameter hw = History of resource exploitation weighting

In the DIVA database we have values of sedsup between 1.7 and 4.9.

1401

1402

| Sediment Supply Factor | Description | Key data reference/source | Category | Forcing Score | Weighting Factor |
| --- | --- | --- | --- | --- | --- |
| Tectonic Context | Global tectonic setting | Inman and Nordstrom (1971) | Passive Margins | 1 | 0.07 |
|  |  |  | Marginal Seas | 3 |  |
|  |  |  | Active Margins | 5 |  |
| Fluvial Context | Annual river discharge | Ludwig and Probst (1998) | >500 FTSS (1012 g/yr) | 1 | 0.2 |
|  |  |  | 100-500 FTSS (1012 g/yr) | 2 |  |
|  |  |  | 50-100 FTSS (1012 g/yr) | 3 |  |
|  |  |  | 5-10 FTSS (1012 g/yr) | 4 |  |
|  |  |  | <5 FTSS (1012 g/yr) | 5 |  |
|  | Distance to the point of fluvial discharge | Calculated by GIS | 0-30km | 1 |  |
|  |  |  | 30-70km | 2 |  |
|  |  |  | 70-120km | 3 |  |
|  |  |  | 120-180km | 4 |  |
|  |  |  | >180km | 5 |  |
| Glacial Context | Location relevant to maximum extent of last glaciation | Williams et al. (1991) | 100km-300km | 1 | 0.1 |
|  |  |  | >300km | 3 |  |
|  |  |  | <100km | 5 |  |
| Geomorphic Context | Coastal geomorphic setting | McGill (1958) | Sheltered coast (Inlet/delta/estuary) | 1 | 0.03 |
|  |  |  | Open coast | 5 |  |
| Management Context | Degree of coastal protection | DIVA adaptation algorithm or user inputs (Tol et al., 2005) | Sea dike absent (< 0.5 m high) | 1 | 0.3 |
|  |  |  | Sea dike present (> 0.5 m high) | 5 |  |
| Historical Context | Timing of peak resource exploitation | Expert judgment | Classical | 1 | 0.3 |
|  |  |  | Medieval | 2 |  |
|  |  |  | Colonial | 3 |  |
|  |  |  | 20th Century | 5 |  |

1403

1404

1405

1406

1407

1408

1409

1410

1411

1412

1413

1414

1415

1416

Table A4 Factors influencing sediment supply and their incorporation into the DIVA_WCM via forcing scores and weighting factors.

# Coastal segment vulnerability score (csvs)

The above calculated three forcing scores are then combined into the coastal segment vulnerability score (csvs) following Equation (4)

csvs = rslr_tidal_score * 0.5 + aspace * 0.2 + sedsup * 0.3	Eq. 4

where rslr_tidal score is the relative sea-level rise and tidal range forcing (Equation 1), aspace the lateral accommodation space forcing score (Equation 2) and sedsup the sediment supply forcing score (Equation 3).

1421

1422

1423

1424

1425

1426

1427

1428

1429

1430

1431

1432

1433

1434

1435

1436

# Ecological Sensitivity Score (ESS) by wetland type

Finally, we compute the ecological sensitivity score (ess) by combining the csvs values of the current and last time step.

# Eq. 5

where ESStype is the ecological sensitivity score for the given wetland type, csvscurrent is the coastal segment vulnerability score and csvslast is the value of this variable calculated within the previous time step, weight_currenttype is the lag weight associated to the current time step and weight_lasttype is the lag weight associated to the csvs value from the previous time step. The weights used are given in Table 4.

| Wetland Type (type) | Previous 5 year lag weight (weight_lasttype) | Current 5 year lag weight (weight_currenttype) | Response time (yrs) |
| --- | --- | --- | --- |
| Coastal forest | 1 | 0 | 10 |
| Freshwater marsh | 0 | 1 | <5 |
| Saltmarsh | 0 | 1 | <5 |
| Mangrove forest | 1 | 0 | 10 |
| Unvegetated wetland | 0 | 1 | <5 |
| Mudflat and sand flat | 0 | 1 | <5 |

1437

1438

1439

1440

1441

1442

1443

1444

1445

1446

1447

1448

Table A5: Response to environmental change by wetland type as modelled by relative importance of previous and current ecological state. Response time = 5 / Current 5 year lag weight.

# Wetland response (Annual wetland loss rate)

The ess values are then translated into relative 5-years wetland loss rates (RLR5), which is the proportion of wetlands lost for a specific wetland type during a 5-year time step.

RLR5 = 1- (β +1) * (1 – ESStype/5) ^ β + β * (1 – ESStype/5) ^ (β + 1)	Eq. 6

Values of beta:

| 1449 | 1. | Unvegetated high and low: 0.093 |
| --- | --- | --- |
| 1450 | 2. | Freshwater Marsh: 0.188 |
| 1451 | 3. | Saltmarsh: 0.137 |
| 1452 | 4. | Coastal Forest and Mangrove Forest: 0.074. |

1456

1457

1458

1459

1460

1461

1462

1463

1464

1465

1466

1467

1468

1469

# Supplementary material references

Inman, D.L., Nordstrom, C.E., 1971. On the tectonic and morphologic classification of coasts. Journal of Geology 79, 1-21.

Ludwig, W., Probst, J.L., 1998. River sediment discharge to the oceans: present-day controls and global budgets. American Journal of Science 298, 265-295.

Maxwell, B.A., Buddemeier, R.W., 2002. Coastal typology development with heterogeneous data sets. Regional Environmental Change 3, 77-87.

McGill, J.T., 1958. Map of coastal landforms of the world. Geographical Review 48, 402- 405. NGDC  (National  Geophysical  Data  Centre),  2001.	NGDC  CD-ROM,  ETOPO2  Global  2’

1470

Elevations. Available at: http://www.ngdc.noaa.gov/mgg/fliers/01mgg04.html

1471

1472

1473

1474

1475

1476

1477

1478

Nicholls, R.J., Tol, R.S.J., 2006. Impacts and responses to sea-level rise: A global analysis of

the SRES scenarios over the twenty-first century. Philosophical Transactions of the Royal Society of London 364A, 1073–1095.

Williams, M.A.J., Dunkerley, D.L., De Deckker, P., Kershaw, A.P., Stokes, T., 1991. Quaternary Environments. Edward Arnold, London.