Cavity-induced phase noise suppression in Fabry-Perot modulator-based optical frequency comb

Joonyoung Kim,* David J. Richardson, and Radan Slavík

Optoelectronics Research Centre, University of Southampton, Southampton, U.K.

*Corresponding author: j.kim@soton.ac.uk

Received XX Month XXXX; revised XX Month, XXXX; accepted XX Month XXXX; posted XX Month XXXX (Doc. ID XXXXX); published XX Month XXXX

We investigate for the first time, both theoretically and experimentally, how the phase noise of the RF drive signal affects the phase noise of the individual tones of a Fabry-Perot (F-P) modulator-based optical frequency comb. We observe that the expected deleterious effect of the RF drive signal phase noise on the comb output is partially suppressed due to the filtering characteristics of the F-P cavity. We found that the cavity-induced phase noise suppression is strongest for high-order comb tones, e.g., reaching up to 40 dB for the 100th comb tone at high offset frequencies. The phase noise suppression becomes even stronger for low RF-drive powers, or when the seed laser does not resonate in the F-P cavity. For both cases we observe up to 10dB increase in phase noise suppression. We also evaluate the timing jitter improvement obtained thanks to the cavity-induced phase noise reduction. The timing jitter (integrated from 2.5 MHz to 2.5 GHz) decreased by a factor of 7 for the beat signal obtained between two comb tones that are 100 tones apart (in comparison with the timing jitter obtained in a cavity-less comb generator). © 2017 Optical Society of America

OCIS codes: (060.2310) Fiber optics; (070.2615) Frequency filtering; (230.5750) Resonators; (120.2230) Fabry-Perot.

http://dx.doi.org/10.1364/OL.99.099999

Optical frequency combs generated by optically-seeded electro-optic modulators have several unique properties such as a wide comb-tone spacing (10-50 GHz), simplicity, and robustness [1], which are all relevant for applications such as the generation of microwave to THz signals [2-5], line-by-line optical pulse shaping and arbitrary waveform generation [6-8], high-precision phase-coherent wavelength conversion [9], coherent wireless and wavelength division-multiplexed (WDM) communications [10,11]. The key parameters of these optical frequency combs are the number of comb tones within a particular power range (e.g. <10 dB) and the phase/frequency stability [1].

Two groups of schemes have been mainly used: (i) based on a cascade of amplitude/phase modulators [12] (Fig. 1a) and (ii)

Fig. 1.  Optical frequency comb generation with: (a) cascaded modulators, (b) the phase modulator enclosed by Fabry-Perot (F-P) cavity, where fm is modulation frequency. (c) shows the generated optical frequency comb where the noise of RF signal, δf, is multiplied as the comb tone number increases.

based on phase modulator placed inside a resonator (e.g., Fabry-Perot) [13,14]. The key advantage of the first approach is the comb spacing tunability that is achieved by controlling the RF drive signal frequency fm [12] and the ability to generate flat-top comb spectra. The number of comb tones, however, is rather limited using this approach and larger comb bandwidths require the cascading of more modulators. The various implementations of this scheme include the use of a dual-drive Mach-Zehnder modulator, for which the desired flat-top comb response can be obtained [15,16]. To further decrease the required RF power, size, and cost, alternative materials such as InP have been researched [16].

The Fabry-Perot cavity-enclosed phase modulator (i.e. F-P modulator), see Fig. 1(b) [13,14], has limited tunability (given by multiples of its free-spectral range (FSR)), but large bandwidths can be covered using a single device. To be specific, when the F-P modulator is driven at a frequency fm, which is an integer times the F-P cavity’s FSR, the seed continuous wave (CW) light is converted into an optical comb spanning over several THz bandwidth and consisting of more than a hundred comb tones within  the 10-dB power bandwidth.

For both classes of scheme mentioned previously (using a cascade of modulators or a modulator inside a resonator), the phase noise characteristics of the individual comb tones are determined by (i) the phase noise of the seed laser and (ii) the phase noise of the RF drive signal [3,5,13]. As far as the phase noise of the seed light is concerned, it is just “copied” onto each comb tone, thus the use of a frequency/phase-stabilized narrow-linewidth seed laser allows for low-noise optical frequency comb performance [15]. The phase noise of the RF signal obtained from a high-quality RF synthesizer is generally lower than that of the seed laser, but its impact (phase/frequency noise) is ‘multiplied’ by N for the Nth comb-tone, as sketched in Fig. 1 (c). Thus the higher the comb tone number (N), the higher the impact of the RF signal noise on the comb-tone phase noise [5]. Consequently, the phase noise of high order comb tones (i.e., large N) can be significantly higher than the phase noise of the seed laser.

As we show here for the first time (to the best of our knowledge) the high phase noise of high-order comb tones is partly suppressed in comb generators based on modulators embedded inside resonators. We show this for a F-P modulator-based comb generator, Fig. 1 (b). This suppression is more efficient for the noisier high-N comb tones, providing efficient means of noise suppression for high-N comb tones.

This paper follows on from our preliminary work [17], where we reported this phenomenon for the first time and evaluated it via simulations. Here, we carry out experimental verification and discuss its implications with regards to applications.

First, we derive the electric field equation of the F-P modulator-based optical frequency comb and subsequently obtain the phase noise of the optical comb tones. After M-roundtrips, the output electric field of the F-P modulator EoutMτR can be written as [13]:

EoutMτR=EinMτR1-Rη×

exp-jπΔf/FSR+ϕk=M+1 ,           (1)

where R is the reflectivity of the F-P cavity mirrors (in power), η is the single-pass power transmission efficiency in the resonator, τR is the roundtrip time, Δf is the frequency detuning between the seed light frequency and the F-P resonant frequency, and ϕk is the phase modulation after the kth roundtrip.  EinMτR is the electric field inside the cavity at the Mth roundtrip, which can be written as:

EinMτR=Ec1-R+

ηREin(M-1)τRexp-j2πΔf/FSR+ϕk=M,         (2)

where Ec is electric field of the seed laser and Ein(M-1)τR is the electric field inside the cavity at the previous (M-1)th roundtrip. In this way, the amplitude and phase of the electric field inside the cavity is updated every roundtrip. The phase modulation ϕk is expressed as [13]:

ϕk=βsin2πfmt-2M-2k+3τR2+φt-2M-2k+3τR2                        (3)

where β is the modulation depth and φ is the phase noise of the

Fig. 2. Experimental set-up to measure the comb-tone phase noise of  the Fabry-Perot (F-P) modulator-based optical frequency comb.

RF signal at the drive frequency fm, which is white Gaussian noise in the simulation. As mentioned, the modulation frequency fm is an integer multiple of the F-P resonator FSR. The phase modulation is presented only in the forward direction as the phase modulator is a traveling-wave type, where only the forward-propagating light is modulated [13]. In our analysis, we neglect the phase noise of the seed light and which is simply copied on to all comb tones as we described earlier.

We obtain the electric field of the Nth comb tone by applying a rectangular band-pass filter centered at the Nth comb tone frequency (Nfm):

EN=FEoutM→∞×Hrec(f-Nfm)                     (4)

where FEoutM→∞ is the output electric field of the F-P modulator in the frequency domain (F represents Fourier transform) and Hrec(f-Nfm) is a rectangular filter transfer function centered at Nfm. Finally, we calculate the phase noise of EN.

To experimentally confirm the results of our simulations, we used two F-P modulator-based Comb generators (WR-250-03-25, OptoComb Inc.) that have a 2.5 GHz FSR and a Finesse of ~50, as in Fig. 2. The first comb serves as a reference (“reference Comb”) for measuring the phase noise of the second one (“Comb under test”). Both Combs are seeded with the same seed laser (<1 kHz linewidth, SlowLight laser, Orbit Lightwave Inc.). The reference Comb is driven by a 10 GHz RF signal directly-generated by a high-quality RF signal generator. The Comb under test is driven by a 25 GHz signal that has a high level of white noise intentionally added to it via a mixer (Fig. 2). This is important for two reasons. Firstly, it ensures that the phase noise of the Comb under test is significantly higher than that of the reference Comb, enabling measurement of the phase noise of the Comb under test by comparing it to the reference Comb. Secondly, it ensures that the phase noise of the Comb under test is dominated by the RF drive signal phase noise: the influence of which we aim to study here. Both combs are phase-locked via a 10-MHz reference, Fig. 2. Finally, we combine the outputs of both combs optically and use a narrow-band pass optical filter (bandwidth: ~10 GHz, Alnair Inc.) to filter a single tone of the Comb under test that we want to measure, together with the nearest tone from the reference Comb. In this way, we obtain a 5 GHz beat signal between the reference Comb tone and the odd-number Comb-under-test tones, as sketched in Fig. 2. Although this allows us to measure the noise properties of the odd-tones of the Comb under test only, it is sufficient for our study as we show later. We use two EDFAs to compensate for the loss in the Comb generators (insertion loss of ~30 dB) as well as through the narrow-bandwidth optical filtering, Fig. 2.

In the experiment, we used a variable RF-attenuator on the 25 GHz signal path to adjust the modulation depth, (β in Eq. 3). Also, we set the seed laser frequency to match the F-P resonator frequency (i.e. Δf=0 in Eq. 1-2), generating a wide comb spectrum.

Fig. 3 shows the measured RF spectrum (i.e. the beat signal at 5 GHz) of the 1st and 11th comb tones of the 25 GHz OptoComb when the modulation depth (β) is 0.7π. Comparing the two RF spectra, we see that the noise is only increased at low offset frequencies (by up to 21 dB), while there is no increase at high offset frequencies. It is worth mentioning that for cavity-less comb generators (e.g., as shown in Fig. 1a), the noise would increase over the entire spectral range by 21 dB (at a rate of  20log10N  [3], being  20log1011=21 dB for the 11th tone). Thus, Fig. 3 clearly shows the strong benefit of the F-P cavity filtering effect, which is of our interest here.

Fig. 4 shows the simulated and measured phase noise of various comb tones normalized to the phase noise of the input 25 GHz RF signal when β=0.7π. The x-axis of Fig. 4 is the offset frequency normalized to the FSR of the F-P cavity. The simulation results are obtained based on Eq. (1-4). The phase noise at offset frequencies below the FSR/Finesse (~50 MHz in the experiment), where the OptoComb filtering effect is negligible, increases by 20log10N (N is the comb tone number). The phase noise reduction at high offset frequencies (>FSR/Finesse) is more pronounced for higher order comb tones, e.g., we obtained 30 dB suppression at 0.5×FSR offset frequency (1.25 GHz in our experiment) for the 31st comb tone. In the experiment, the measurement noise floor was limited by the ASE noise generated by the EDFAs, which precluded us from  measuring the phase noise of comb tones with N>31. However, simulations suggest ~40 dB noise suppression for the 100th comb tone.

Subsequently, we investigated the effect of the modulation depth  β on the cavity-induced phase noise suppression, which was controlled via an RF attenuator placed at the Comb under test RF input (Fig. 2). Fig. 5(a) shows simulated and measured optical comb spectra for β=0.7π and 0.35π, respectively. Both experiment and simulations show 1.2 dB/nm slope for β=0.7π  and 2.5 dB/nm slope for  β=0.7π, following well the previously-published sideband power scaling of exp-2NπβF, where N is the comb tone number and F is the Finesse [13]. The relative phase noise (normalized to the phase noise of the 25 GHz drive signal) of the 11th comb tones for two different modulation indices (β), are shown in Fig. 5(b). Both the simulation and the experiment show that the phase noise is more heavily suppressed for the smaller modulation index. To find out why, we analyzed the dynamics of the Comb formation using our numerical model presented earlier: we found out that the required number of cavity roundtrips for the 11th comb tone to reach steady state was 11 and 5 for β=0.35π

Fig. 3. RF beat signal at 5 GHz for the 1st (black dashed) and the 11th (blue solid) comb tones of the 25 GHz F-P comb generator.

Fig. 4. Measured and calculated relative phase noise of various comb tones. Solid lines: simulation; dashed lines: measured.

and =0.7π , respectively. The larger number of roundtrips induces a stronger cavity filtering effect, which is why the stronger phase noise suppression is observed (in our experiment, 10 dB stronger noise suppression at an offset frequency of 0.1×FSR).

Additionally, we investigated the effect of frequency detuning (Δf) between the seed laser (i.e., carrier) frequency and the F-P resonance on the phase noise suppression, Fig. 6. At the detuning Δf that leads to the maximum power-transmission (i.e. Δf=β2π) [16], the phase noise suppression becomes strong as compared to Δf=0 which gives the widest comb spectrum [16]. Thus, the detuning of the F-P cavity and the seed laser carrier frequency has the same effect on phase noise suppression as the low RF-power modulation does. The phase noise suppression of the high-N comb tones shown above is expected to have a significant effect on the timing jitter of any signal derived from the comb. This is because the timing jitter is calculated as the phase noise integral over the bandwidth of interest. As an example, the timing jitter of the sinusoidal signal at the frequency of Nfm generated by beating the carrier (at fC) and the Nth comb tone (at fC+Nfm) can be expressed as:

JitterN=210-3×FSRFSRLNfdf2πNfm                                         (5)

where LNf is the single-sideband (SSB) phase noise of the beat signal (between the carrier and the Nth comb tone) which would be dominated by the multiplied RF drive signal phase noise. As in Eq. (5), we integrate from 10-3×FSR up to FSR which corresponds to 2.5 MHz (limited by the resolution frequency in the numerical simulation) – 2.5 GHz (limit of the experimentally measured data). Fig. 7 shows the measured and simulated timing jitter of the beat signals at various frequencies (i.e. at N×fm) relative to that of the beat signals based on the cavity-less Comb.

Fig. 5. (a) Optical spectrum for β=0.7π (upper blue) and  β=0.35π (lower red). (b) Relative phase noise of the 11th comb tone for β=0.35π (red) and β=0.7π (blue).

Fig. 6. Relative phase noise of the 11th comb tone for two different frequency detunings between the seed laser and the F-P modulator that give the widest comb spectrum (blue) and the narrowest comb spectrum, i.e., the maximum transmission (red).

The experiment and simulation results show good agreement. For the beat signal at 31×fm and β=0.7π, the jitter was reduced by a factor of ~3 due to the cavity-induced phase noise suppression. For the beat signal at 100×fm, the jitter was calculated to be reduced by a factor of 7. This is of importance, e.g., when generating a 1-THz signal from a 10-GHz comb, as using an F-P comb would result in 7 times lower timing jitter as compared to the cascaded-modulator approach. Although Fig. 7 suggests that lower  could further reduce the jitter of each comb tone, this would be at the expense of the comb bandwidth, as illustrated in Fig. 5(a), suggesting the comb bandwidth and noise suppression need to be traded off for a given application.

In conclusion, we have investigated the cavity-induced suppression of comb-tone phase noise in the F-P modulator-based comb generator. The noise suppression was strong at high-order comb tones: e.g., 30 dB at 0.5×FSR offset frequency for the 31st comb-tone and 40 dB for the 100th comb-tone when the modulation index (β) was 0.7π. The noise suppression became stronger for lower RF drive power (e.g. β=0.35 π) and for non-resonant detuning between the seed laser and the F-P cavity as the comb tone needs a larger number of roundtrips inside the F-P cavity to reach steady state, resulting in >10 dB lower phase noise at 0.1×FSR offset frequency. The timing jitter of a beat signal

Fig. 7. Relative timing jitter of beat signals (between the carrier and the N-th comb tone) using the F-P modulator-based comb normalized to that of a cavity-less Comb generator. The solid blue, red, and green lines are the simulated relative jitters for 0.7π, 0.35π, and 0.2π, respectively. Blue rectangles and red circles: measured results for =0.7π and 0.35π, respectively.

generated by two comb tones that are 100 comb tones apart  (integrated phase noise from 2.5 MHz – 2.5 GHz in our experiment) was reduced by a factor up to 7 when compared to that of non-cavity comb generators, providing an additional advantage to relevant applications such as RF photonics. The jitter could be further reduced using an F-P modulator with a higher Finesse and/or narrower FSR, which suppresses the phase noise at lower offset-frequencies.

Funding. EPSRC Fellowship grant agreement (EP/K003038/1).

Acknowledgment. The data for this work is accessible through the University of Southampton Institutional Research Repository (doi: xxxxxxxxxx).

References

V. Torres-Company and A. M. Weiner, Laser Photon. Rev. 8, 368 (2014).

J. Li, X. Yi, H. Lee, S. A. Diddams, and K. J. Vahala, Science 345, 309 (2014).

G. J. Schneider, J. A. Murakowski, C. A. Schuetz, S. Shi and D. W. Prather, Nat. Photon 7, 118 (2013).

D. Sun, Y. Dong, L. Yi, S. Wang, H. Shi, Z. Xia, W. Xie, and W. Hu, Opt. Lett 39, 1493 (2014).

A. Ishizawa, T. Nishikawa, T. Goto, K. Hitachi, T. Sogawa, and H. Gotoh, Sci. Rep. 6, 24621 (2016).

Z. Jiang, D. E. Leaird, C.-B. Huang, H. Miao, M. Kourogi, K. Imai, and A. M. Weiner, IEEE J. Sel. Topics Quantum Electron. 43, 1163 (2007).

V. Torres-Company, A. J. Metcalf, D. E. Leaird, and A. M. Weiner, IEEE Photon. Technol. Lett. 24, 891 (2012).

S. Liao, Y. Ding, J. Dong, S. Yan, X. Wang, and X. Zhang, Opt. Express 24, 24390 (2016).

J. Kim, G. Marra, D. S. Wu, D. J. Richardson, and R. Slavík, Opt. Lett 41, 1716 (2016).

T. Nagatsuma, in Optical Fiber Communication Conference (OFC), OM2B.7 (2012).

S. K. Ibrahim, J. Zhao, and F. C. G. Gunning, IEEE Photon. J. 2, 833 (2010).

R. Wu, V. R. Supradeepa, C. M. Long, D. E. Leaird, and A. M. Weiner, Opt. Lett. 35, 3234 (2010).

T. Saitoh, S. Mattori, S. Kinugawa, K. Miyagi, A. Taniguchi, M. Kourogi, and M. Ohtsu, J. Lightw. Technol. 16, 824 (1998).

S. Xiao, L. Hollberg, N. R. Newbury, and S. A. Diddams, Opt. Express 16, 8498 (2008).

T. Sakamoto, T. Kawanishi, and M. Izutsu, Opt. Lett. 32, 1515 (2007).

R. Slavík, S. G. Farwell, M. J. Wale, and D. J. Richardson, IEEE Photon. Technol. Lett. 27, 217 (2015).

J. Kim, D. J. Richardson, and R. Slavík, in IEEE Photonics Conference, WA2.4 (2016).

Full References (for review)

- Victor Torres-Company and Andrew M. Weiner, “Optical frequency comb technology for ultra-broadband radio-frequency photonics,” Laser & Photonics Review 8, 368-393 (2014).

Jiang Li, Xu Yi, Hansuek Lee, Scott A. Diddams, and Kerry J. Vahala, “Electro-optical frequency division and stable microwave synthesis,” Science 345, 309-313 (2014).

Garrett J. Schneider, Janusz A. Murakowski, Christopher A. Schuetz, Shouyuan Shi and Dennis W. Prather, “Radiofrequency signal-generation system with over seven octaves of continuous tuning,” Nature Photonics 7, 118-122 (2013).

Dongning Sun, Yi Dong, Lilin Yi, Siwei Wang, Hongxiao Shi, Zongyang Xia, Weilin Xie, and Weisheng Hu, “Photonic generation of millimeter and terahertz waves with high phase stability,” Optics Letters 39, 1493-1496 (2014).

A. Ishizawa, T. Nishikawa, T. Goto, K. Hitachi, T. Sogawa, and H. Gotoh, “Ultralow-phase-noise millimetre-wave signal generator assisted with an electro-optics-modulator-based optical frequency comb,” Scientific Reports 6, 24621 (2016).

Zhi Jiang, Daniel E. Leaird, Chen-Bin Huang, Houxun Miao, Motonobu Kourogi, Kazuhiro Imai, and Andrew M. Weiner, “Spectral Line-by-Line Pulse Shaping on an Optical Frequency Comb Generator,” IEEE Journal of Selected Topics in Quantum Electronics 43, 1163-1174 (2007).

V. Torres-Company, A. J. Metcalf, D. E. Leaird, and A. M. Weiner, "Multichannel Radio-Frequency Arbitrary Waveform Generation Based on Multiwavelength Comb Switching and 2-D Line-by-Line Pulse Shaping," IEEE Photonics Technology Letters 24, 891-893 (2012).

Shasha Liao, Yunhong Ding, Jianji Dong, Siqi Yan, Xu Wang, and Xinliang Zhang, “Photonic arbitrary waveform generator based on Taylor synthesis method,” Optics Express 24, 24390-24400 (2016).

Joonyoung Kim, Giuseppe Marra, David S. Wu, David J. Richardson, and Radan Slavík, “Wavelength conversion technique for optical frequency dissemination applications,” Optics Letters 41, 1716-1719 (2016).

T. Nagatsuma, "Photonic generation of millimetre wave signals and its applications," in Optical Fiber Communication Conference (OFC), OM2B.7 (2012).

S. K. Ibrahim, J. Zhao, and F. C. Garcia Gunning, “Towards a Practical Implementation of Coherent WDM: Analytical, Numerical, and Experimental Studies,” IEEE Photonics Journal 2, 833 – 847 (2010).

Rui Wu, V. R. Supradeepa, Christopher M. Long, Daniel E. Leaird, and Andrew M. Weiner, “Generation of very flat optical frequency combs from continuous-wave lasers using cascaded intensity and phase modulators driven by tailored radio frequency waveforms,” Optics Letters 35, 3234-3236 (2010).

Takanori Saitoh, Shigenori Mattori, Shigeru Kinugawa, Koichiro Miyagi, Akira Taniguchi, Motonobu Kourogi, and Motoichi Ohtsu, “Modulation Characteristic of Waveguide-Type Optical Frequency Comb Generator,” Journal of Lightwave Technology 16, 824-832 (1998).

Shijun Xiao, Leo Hollberg, Nathan R. Newbury, and Scott A. Diddams, “Toward a low-jitter 10 GHz pulsed source with an optical frequency comb generator,” Optics Express 16, 8498-8508 (2008).

Takahide Sakamoto, Tetsuya Kawanishi, and Masayuki Izutsu, “Asymptotic formalism for ultraflat optical frequency comb generation using a Mach–Zehnder modulator,” Optics Letters 32, 1515-1517 (2007).

Radan Slavík, Selina G. Farwell, Mike J. Wale, and David J. Richardson, “Compact Optical Comb Generator Using InP Tunable Laser and Push–Pull Modulator,” IEEE Photonics Technology Letters 27, 217-220 (2015).

Joonyoung Kim, David J. Richardson, and Radan Slavík, “Cavity Effect on Phase Noise of Fabry-Perot Modulator-based Optical Frequency Comb,” IEEE Photonics Conference, WA2.4 (2016).