Supplementary material

The supplementary material provided here offers additional details of the derivations on the equivalences of helical axes and SARA/SCoRE. The material is structured into 3 sections, where section S1 demonstrates the derivation of the vector of angular velocity ω as a function of ϑ and k,  S2 shows that the position of an instantaneous helical axis (IHA) is identical to those of the finite helical axis (FHA) or SARA, and S3 verifies that the mean finite helical axis (MFHA) and the SARA axis are mathematically equivalent.

S1

Here we derive for ω = RτRTτ a representation as function of  ϑ and k. Using equation (6) for Rτ and equation (20) for Rτ, i.e.

Rτ=cosτϑ I+sinτϑ k+(1-cosτϑ) kkT R0 ,

Rτ=ϑ -sinτϑ I+cosτϑk+sinτϑkkT R0 ,

one obtains

ωτ=     ϑ –sinτϑcosτϑI+sin2τϑk-sinτϑ (1-cosτϑ) kkT

+ cos2τϑk-cosτϑsinτϑkk+cosτϑ(1-cosτϑ)kkkT

+ sinτϑcosτϑkkT-sin2τϑkkTk+sinτϑ(1-cosτϑ)kkTkkT

and further with the relationships kk=0, kkTkkT=kkT, see equation (3),

ωτ=      ϑ –sinτϑcosτϑI+k –cosτϑsinτϑkk

+ sinτϑcosτϑkkT-sin2τϑkkTk .

Since with kk=0 also kTk=0 holds and together with kk+I=kkT this yields finally

ω= ϑ k.

As expected, ω does not depend on τ. The final result is the well-known relationship between the vector of angular velocity ω, the rotation axis unit vector k, and the angle of rotation ϑ. Here, we have demonstrated that the notation of the unique parameterization of the shortest geodesic path in SO3 is consistent with this definition.

S2

In order to determine the position of the IHA-axis as a function of ϑ and k, we use equation (19)

cG,IHAτ= tτ+ωτ×tτωTτ ωτ

and insert in this equation the terms for tτ, equation (11), ω, equation (23), and tτ, equation (21), into equation (19) and obtain:

cG,IHAτ=I-kkTcG-I-kkTcosτϑ+sinτϑkcG-t0+kkTt0+τt1-t0

+  ϑk×ϑ sinτϑI-kkT-cosτϑkcG-t0+kkTt1-t0ϑ2

which yields

cG,IHAτ=I-kkTcG- I-kkT cosτϑ+sinτϑkcG-t0+kkTt0+τt1-t0

+ sinτϑk-cosτϑkk cG-t0

Thus finally we have, again with kk+I=kkT,

cG,IHAτ=I-kkTcG+kkTt0+τt1-t0 .

For the unique point on the axis with minimal norm, i.e. using kTcG,IHAτ=0 one obtains cG,IHAτ=cG. This is the analytical proof that the instantaneous helical axis (IHA) coincides with the FHA- or the SARA-axis.

S3

In order to show the mathematical equivalence between the MFHA approach and SARA we start from the equation for the MFHA (27):

i,j=1,i<jnwijI-kijkijTcMHFA=i,j=1,i<jnwijcij                                 (S31)

In which we replace now the terms I-kijkijT with the results from (16), i.e.

2I-RiRjT-RjRiT=21-cosϑijI-kijkijT,

and cij according to (17), considering that kijTcij=0 holds:

21-cosϑijcij=ti+tj-RiRjTtj-RjRiTti                                      (S32)

this yields:

i,j=1,i<jnwij2I-RiRjT-RjRiT1-cosϑijcMFHA=  i,j=1,i<jnwijti+tj-RiRjTtj-RjRiTti1-cosϑij

Setting wij=1-cosϑij or, since 1-cosϑij=2 sin2ϑij2 , wij=sin2ϑij2 leads to:

i,j=1,i<jn2I-RiRjT-RjRiT cMFHA=i,j=1,i<jnti+tj-RiRjTtj-RjRiTti

Further, with i,j=1,i<jn1=nn-12 and  i,j=1,i<jn ti+tj=n-1i=1nti , the relationship can be written as:

nn-1I-i,j=1,i≠jnRiRjT cMFHA=n-1i,j=1,i≠jnRiRjTtj               (S33)

This equation (S33) above is identical to equation (8) in (Ehrig et al., 2006) and was presented in there as the closed form solution of the SCoRE approach for the determination of the centre of rotation of a ball joint, demonstrating the mathematical equivalence between the MFHA approach and SARA.

It is important to note that this derivation does not depend on how exactly the axes were determined and which specific method was used (FHA, IHA, etc.); the only requirements are a representation of each axis as the unique norm minimal point on the axis together with the unit axis vector.

In summary, with wij=1-cosϑij the formulae (S31) and (S33) differ only in that (S31) defines the centre in terms of rotations and translations whereas (S33) describes the centre as a function of kij and  cij. It therefore follows that the derivation of equivalence also holds for the mean instantaneous helical axis.

References

R.M. Ehrig, W.R. Taylor, G.N. Duda, and M.O. Heller (2006). A survey of formal methods for determining the centre of rotation of ball joints. J Biomech 39: 2798-809.