Adaptive energy management of a battery-supercapacitor energy storage system for electric vehicles based on flexible perception and neural network fitting

Tao Zhu a, Richard G. A. Wills a, Roberto Lot a,b, Haijun Ruan c, Zhihao Jiang d,*

a Department of Mechanical Engineering, University of Southampton, Southampton, SO17 1BJ, UK.

b Department of Industrial Engineering, University of Padova, Padova, 35131, Italy.

c Dyson School of Design Engineering, Imperial College London, London, SW7 2AZ, UK.

d School of Mechanical, Aerospace and Civil Engineering, University of Manchester, Manchester,

M13 9PL, UK

* Corresponding author: zhihao.jiang@postgrad.manchester.ac.uk (Z. Jiang).

Abstract: The hybrid energy storage system (HESS) composed of batteries and supercapacitors (SCs) is a dual energy storage technology that can compensate for the shortcomings of a single energy storage technology acting alone. The energy management of HESS splits the power and energy demands from the electric vehicle (EV) to the battery and SC and thus is vital to EV propulsion. This paper presents an online energy management strategy (EMS) that optimises the operating costs of battery-SC HESS and can be adaptive to real-time EV driving conditions. We analyse the optimal offline benchmarks to guide online EMS design and propose the adaptive online EMS with variable perception horizon based on both neural network and rule-based techniques. Compared with existing research, the proposed EMS features reduced complexity, flexible perception and intelligent rulemaking. Case study results show that the proposed variable perception horizon and neural network fitting can improve EMS optimality compared with the conventional methods in existing research. The proposed EMS can realise more than 97% cost optimisation efficacy of offline benchmarks. By the proposed EMS, this paper is expected to provide a practical and effective energy management approach for the battery-SC HESS to reduce costs in EV applications.

Keywords: Electric vehicle, Hybrid energy storage system, Energy management, Cost optimisation, Variable perception horizon, Neural network fitting.

1. Introduction

The past decade has witnessed increasing electrification of public and private transportation [1]. Electric vehicles (EVs), as clean transport agents powered by electricity, are attaining tremendous development inputs and booming sales in the market [2]. The onboard energy storage system (ESS) is the heart of an EV since it delivers power and energy for propulsion. The most popular ESS is the battery-only ESS, which comprises hundreds or thousands of cells, typically of Li-ion chemistry, due to their high energy density compared to other cell types [3]. EV propulsion requires the ESS to possess a high power level for rapid accelerations and a sufficient energy capacity for long-range driving. Most ESSs use one single energy storage technology and thus cannot be optimal for both power and energy [4]. The hybrid energy storage system (HESS), which pairs two or more complementary energy storage components, is a solution to compensate for the shortage of single energy storage acting alone. By paring energy-intense batteries with power-intense supercapacitors (SCs), the battery-SC HESS is one widely studied practice of HESS [5]. It has been reported that the HESS surpasses battery-only ESS in terms of energy efficiency and battery degradation rate [4].

The HESS can be regarded as a plant with two controlled objects – battery pack and SC pack, plus an actuator – DC/DC converter [6]. The control strategy of onboard HESS, also named the energy management strategy (EMS), is responsible for splitting the power and energy demands from the EV to battery pack and SC pack [7]. The fundamental requirement for the EMS is to sustain EV propulsion: as the EV proposes a power demand with time, the EMS will instruct the battery pack and SC pack to deliver respective power. In combination, the total power delivery should exactly meet EV power demand [8]. Beyond this point, there are two further requirements for the EMS, optimality and adaptiveness. The requirement of optimality aims to solve optimisation problems such as minimising electricity usage and battery degradation while sustaining EV propulsion [9]. The requirement of adaptiveness considers that real-life driving has a degree of unpredictability, so that the EMS should be adaptive to real-time driving conditions [10]. Existing studies of the EMS of HESS can be classified as either “objective-oriented” or “technique-oriented” ones.

The “objective-oriented” studies target various engineering requirements that are the objectives of the EMS and can be formulated as the cost functions to be optimised [8]. For example, electricity usage of the HESS is the most frequently used cost function in the literature [7] since electricity consumption determines the electricity costs of vehicle operations. Battery degradation is another common cost function since it determines the occurrence of battery replacement [11] and leads to battery degradation costs [12]. Besides, additional objectives can be formulated as penalty functions along with main cost functions. For example, considering the thermal safety of battery pack, the temperature of battery pack can be formulated as a penalty [13]. To prevent the SC pack from being depleted or overcharged, the state-of-charge (SOC) of SC pack can also be used as a penalty [14]. Despite various objectives being adopted, the formulation of cost functions needs to take into account practical engineering requirements.

The “technique-oriented” studies provide specific control schemes or algorithms to solve the formulated cost functions [8]. The current literature in the past few decades shows that studies of EMS evolve with the progress of novel control techniques. For example, the rule-based technique is an early but also the most robust control technique that has long been used [15]. It sets up thresholds for certain control variables and decides control actions based on rules associated with the thresholds [16]. Later, the fuzzy logic control technique appears, updating the rule-based technique. The fuzzy technique also uses prescribed rules, but these rules are no longer related to thresholds but are defined by membership functions [17]. Both rule-based and fuzzy techniques are empirical techniques that need applied experiences to formulate accurate rules [18]. In comparison, control algorithms such as genetic algorithm (GA), dynamic programming (DP) and Pontryagin’s minimum principle (PMP) arise as less judgemental techniques that follow normative optimisation policies [19]. With these algorithms being widely applied, some modified algorithms are proposed to overcome the shortcoming of the original algorithms. For example, the non-dominated sorting GA II has an improved computation efficiency with a simplified iteration process [20]; the stochastic DP and adaptive PMP can be suboptimal online implementable algorithms, while the original DP and PMP can only be used to obtain optimal offline benchmarks [8]. The EMS should perform online to fit real-time driving conditions, but real-life driving always has a degree of “future uncertainty” [21]. With this regard, more prediction-based control schemes are proposed, such as the Markov chain and model predictive control (MPC) techniques [22]. For example, the MPC technique uses an EV model and historical driving states (e.g., vehicle velocity and acceleration) within a specific perception horizon to make predictions about future driving states [23]. Both historical and predicted driving states are imported into an online algorithm, and the algorithm works out the control strategies to be used in the coming control horizon [24]. With the development of artificial intelligence, recent years see the machine learning techniques like the neural network (NN) being increasingly applied to the field of EMS [25]. Machine learning is a broad discipline with many branches such as supervised learning, unsupervised learning, deep learning and reinforcement learning, while the state-of-art turns to deep reinforcement learning [26]. Compared with the MPC technique, the machine learning techniques do not need explicit models or representations to make predictions [27], and the transformation from control inputs to outputs is hidden behind artificial intelligence [28]. It should be noted that the control techniques reviewed above are not mutually exclusive; instead, some practice of EMS, such as the drive pattern recognition (DPR) based EMS, adopts a combination of more than one control techniques [29]. The DPR-based EMS defines a few typical drive patterns and works out the optimal offline control strategies for each drive pattern. The online efforts of DPR-based EMS are to sense the historical and real-time driving states, then in some cases to predict the future driving states.  They recognise similar drive patterns to recall the appropriate control strategies, exploiting the fact that the EMSs are similar for drive cycles with similar statistical properties [29]. Implementing such an EMS needs the cooperation of several control techniques; for example, Ref. [22] uses the combination of DP, MPC and NN, plus a PID controller.

Considering these objectives and techniques, the EMS is expected to be adaptive to real-time driving conditions while optimally solving the cost functions. Existing studies offer various approaches to design the expected EMS successfully; however, there are limitations as summarised here. Firstly, existing studies rely too much on the collection and prediction of velocity-related driving states (e.g., average/standard deviation of positive/negative vehicle velocity [30]) but neglect the power-related ones. Using the collected velocity-related driving states, existing studies tend first to predict future vehicle velocity, based on which future vehicle power is then estimated [22]. Finally, the control strategies are determined by referring to the estimated power other than the predicted velocity, and the control actions are to manage future power other than future velocity. Therefore, the predicted velocity is actually an intermediate variable that has no direct impact on the control strategies, but the velocity-related intermediate transformations may retard the fast response of EMS. In other words, the aim of EMS should come back to power management rather than velocity prediction. Secondly, most studies adopt a constant perception horizon to sense and process driving states [31] regardless of the singularity of micro-trips. The constant perception horizon may subdivide one micro-trip with consistent driving behaviour or group several micro-trips with very different driving behaviour. Consequently, control strategies are determined based on incomplete or mixed driving behaviour and thus have inferior optimality. Thirdly, most studies adopt redundant driving states to make predictions; for example, Ref. [32] uses as many as 11 driving states. However, these driving states are not always the necessary and efficient ones that can contribute much to accurate predictions. The adopted driving states should be proved helpful; otherwise, the unnecessary driving states can increase the computation loads of onboard processors. Lastly, some studies use offline control strategies of known driving conditions to fit for online new driving conditions [29] but fail to generate intelligent control strategies specifically adaptive to the new driving conditions. Targeted the above limitations, this paper proposes an adaptive online EMS that optimises operating costs of battery-SC HESS in EVs, with the following contributions:

(1) This paper (in Section 3.2) performs a deep analysis of the optimal offline benchmark to guide online EMS design. We refine effective control rules from the offline benchmark and exploit the control rules to enable a concise yet efficient online EMS.

(2) This paper (in Section 4.1) proposes the variable perception horizon that can break down real-time driving conditions into successive driving behaviours. The consistent driving behaviours can be clustered as a whole to feed the online prediction, consequently improving the EMS optimality. In other words, this paper achieves better EMS optimality through adapting the perception horizon, something no existing literature has reported.

(3) This paper (in Section 4.2) adopts a NN with significantly lowered complexity to realise the online prediction. The complexity of NN is reduced by only accepting the necessary inputs, which are refined through analytical deductions and regression analysis.

(4) The overall EMS (in Section 4.3) can be easily implemented using simple hardware configurations. A workflow is suggested as one implementable hardware scheme to realise the proposed EMS. Simulation results indicate that the proposed EMS can achieve more than 97% cost optimisation efficacy of the offline benchmark.

The remainder is organised as follows. Section 2 introduces the EV and HESS for which the EMS is designed. Section 3 formulates the cost function of HESS operating costs and solves the optimal offline EMS to minimise the cost function. By analysing the offline benchmarks, effective control rules are extracted to guide the design of online EMS. The adaptive online EMS, which accepts three necessary driving states and returns a predicted power threshold to split the power demands between the battery pack and SC pack, is designed in Section 4. The EMS adopts the NN and rule-based techniques as well as a variable perception horizon. Section 5 demonstrates the control performances of the proposed EMS in comparison with two comparative online EMSs and the offline benchmarks. Finally, Section 6 draws a conclusion.

2. Configuration of EV and HESS

The studied EV is the Tesla Model S P85, a rear-wheel-drive EV with a battery-only ESS. In this paper, the battery-only ESS is replaced with a battery-SC HESS while the other EV configurations keep the same as the original Tesla EV. The converted EV with the HESS is structured as Fig. 1. The HESS adopts the SC/battery semi-active topology, with the battery pack being main energy storage and the SC pack being second energy storage interfaced by a DC/DC converter [33]. With this topology, the power correlation of HESS can be expressed as Eq. (1), where PHESS, PBAT and PSC are the power of HESS, battery pack and SC pack, ηDC is the efficiency of DC/DC converter and is a function of voltage and current [34]. The battery pack and SC pack each is considered to have a depth-of-discharge of 80% (i.e., the minimum SOC is 20%) [35]. The initial SOC of battery pack and SC pack is set at 100% and 70%, respectively. Parameters of the EV come from the literature [36], while configurations of the HESS come from our previous work [37] for optimal sizing and are presented in Table 1. The battery cells and SC cells used in the HESS are Panasonic NCR18650B and Maxwell BCAP 3400 2.7V, respectively, and their specifications can be found in Refs. [38, 39]. The EV and HESS are modelled in MATLAB, and the state representations of the system can be found in [31].

Fig. 1.   Structure of the Tesla Model S P85 with a battery-SC HESS.

(1)

Table 1.   Configurations of the battery-SC HESS [37].

|  | Battery Pack | SC Pack |
| --- | --- | --- |
| Electrical connection * | 96s 74p | 40s 1p |
| Nominal voltage (V) | 346 | 108 |
| Power level (kW) | 166 | 152 |
| Energy capacity (kWh) | 83 | 0.14 |

*: 96s 74p means the pack has 96 cells in series and 74 cells in parallel.

3. Formulation of cost function and analysis of optimal offline benchmark

This section formulates the HESS operating costs as the cost function of EMS and presents the optimal offline EMS to minimise the cost function under different drive cycles, which is the benchmark to guide designing online EMS. By analysing the benchmark, both common and specific control rules are extracted to fit different drive cycles.

3.1   Cost function - HESS operating costs

The operating costs of HESS are the financial costs that accumulate with HESS service [40]. In this regard, electricity consumption and battery degradation with EV driving are considered two causes of HESS operating costs, and the corresponding costs are electricity consumption costs (Jelec) and battery degradation costs (JBAT). As Eq. (2), Jelec is calculated as the product of HESS electricity consumption  (QHESS) and electricity price (Priceelec, considered as 0.1 USD/kWh [41]), where QBAT and QSC are the electricity consumption of battery pack and SC pack, EBAT and ESC are the energy capacity of battery pack and SC pack, ∆SOC and ∆SOE are the delta state-of-charge of battery pack and delta state-of-energy of SC pack.

(2)

As Eq. (3), JBAT is calculated as the product of battery energy capacity loss (EBAT,loss, the lost energy capacity caused by battery degradation) and battery price (PriceBAT, considered as 260 USD/kWh [42]). Besides, a divisor of 20% is used because the battery pack reaches end-of-life and loses all values when EBAT,loss becomes 20% of the original battery energy capacity [43]. EBAT,loss is further calculated as the product of the original battery energy capacity (EBAT) and battery degradation rate (α, defined as EBAT,loss divided by EBAT). α is calculated based on a representation derived from Ref. [44], which considers α as a function of battery current rate (Irate) and battery ampere-hour throughput (Ahput).

(3)

The HESS operating costs (JOP) are the sum of electricity consumption costs and battery degradation costs, as Eq. (4).

(4)

3.2   Analysis of optimal offline EMS and inspiration to online EMS

The online EMS is expected to optimise the cost function while offering battery-SC hybrid powering strategies for unforeseen driving conditions. Before designing the online EMS, this section works out and analyses the optimal offline EMS for known driving conditions. The optimal offline EMS is a significant benchmark for online EMS because the optimal offline EMS may have specific characteristics that can be imitated and implemented into the online EMS.

The DP technique is exploited to obtain the optimal offline EMS, which optimally distributes the power demands from the HESS to the battery pack and SC pack. It thus obtains the optimal battery operating power and SC operating power. It should be noted that DP requires prior knowledge of the whole drive cycle before performing optimisation; hence, the optimal EMS solved by DP can only be used as an offline benchmark but cannot be an online implementable EMS. The detailed implementation of DP technique can be found in our previous work [37], while this section focuses on demonstrating the optimisation results given by DP. We choose nine drive cycles (ARB02, HWFET, IM240, Manhattan, Nuremberg, NYCC, SC03, US06, UDDS) and one amended drive cycle (Amended UDDS) to represent known driving conditions. The criteria to select the drive cycles are: (1) They should be standard drive cycles that are widely used. (2) They should come from real vehicle testing so that reflect real-life driving conditions. (3) They should be typical of three kinds of driving scenarios: urban (Manhattan, Nuremberg, NYCC, UDDS and Amended UDDS), suburban (IM240 and SC03) and highway (ARB02, HWFET and US06) [45]. The vehicle velocity and HESS power demands of these drive cycles are plotted in Fig. 2. As shown in Fig. 2 (i) and (j), the Amended UDDS is transformed from UDDS by removing the “singular micro-trip”. The micro-trip is defined as an excursion between two successive time points at which the vehicle velocity is zero [46], while the singular micro-trip refers to the micro-trip that is significantly different from the others in terms of velocity and power. The singular micro-trip, which is plotted by a dashed red curve in Fig. 2 (i), is considerably different from the other micro-trips in UDDS and breaks the consistency of driving behaviour. Its maximum vehicle velocity and HESS power demand are much higher than those of the other micro-trips and may thus influence the EMS. The Amended UDDS in Fig. 2 (j) is tailored to investigate whether one singular micro-trip can make a big difference to the optimal offline EMS.

Fig. 2.   Vehicle velocity and HESS power demands of ten drive cycles: (a) ARB02, (b) HWFET, (c) IM240, (d) Manhattan, (e) Nuremberg, (f) NYCC, (g) SC03, (h) US06, (i) UDDS, (j) Amended UDDS.

The optimal HESS operating costs (JOP) and the corresponding offline EMS for each drive cycle are then solved by the DP. The optimal JOP is listed in Fig. 3, while the optimal offline EMS is presented in Fig. 4, where the optimal operating power of SC pack (PSC, represented by hollow blue circles) is plotted as a function of the total power demands of HESS (PHESS). For every single PHESS, the DP determines the optimal PSC while the battery pack takes the residual power demand. Fig. 4 also presents the best fit curve of PSC, as represented by the red polyline. The best fit curve is acquired by using the piecewise linear fitting method of the Shape Language Modeling [47] toolbox, and the freedom of curve fitting is constrained by three preconditions: (1) The best fit curve can be divided into three linear regions, as represented by Regions I, II and III in Fig. 4. (2) The boundary between Regions I and II is PHESS =0. (3) The slope of Region II (kII) is zero. These preconditions are determined by a rough observation of the distribution of PSC with PHESS; however, the preconditions can be validated as rational by the coefficient of determination for the best fit curve in Table 2. Table 2 collects the statistics of the best fit curve under each drive cycle. Based on Fig. 4 and Table 2, the optimal offline EMS along with the curve fitting results under the ten drive cycles are discussed in terms of similarities and differences, as follows.

Fig. 3.   Optimal HESS operating costs (JOP) under ten drive cycles.

From the perspective of similarities, firstly, PSC can always be fitted as an “N-shape” function of PHESS, and the coefficient of determination (R2) is always greater than 0.9, which indicates the “N-shape” curve fitting has high precision. Secondly, the slope of Region I (kI) is always very close to 1, which means when PHESS is below zero, PSC is approximately equal to PHESS. Thirdly, the slope of Region II (kII) is zero, which means when PHESS is between zero and the power boundary (PHESS, bound), PSC equals zero. Lastly, the slope of Region III (kIII) is always around 1, which means when PHESS is greater than PHESS, bound, PSC approximately equals (PHESS - PHESS, bound). These similarities imply that under the ideal EMS, the SC pack should absorb all the negative power demands and perform power peaking only if the positive power demands exceed PHESS, bound. The above similarities are summarised from ten standard drive cycles and thus can be general control rules for general driving conditions, which can be imitated by the online EMS and are realised in Section 4.3.

Fig. 4.   Optimal operating power of SC pack as a function of HESS total power demands, plus the best fit polyline under ten drive cycles: (a) ARB02, (b) HWFET (c) IM240, (d) Manhattan, (e) Nuremberg, (f) NYCC, (g) SC03, (h) US06, (i) UDDS, (j) Amended UDDS.

Table 2.   Statistics of the best fit curve under each drive cycle.

|  | Coefficient of determination R2 | Slope of Region I kI | Slope of Region III kIII | Boundary between Regions II and III PHESS, bound |
| --- | --- | --- | --- | --- |
| ARB02 | 0.9007 | 1.1496 | 0.9313 | 30.6 kW |
| HWFET | 0.9304 | 1.0105 | 1.0361 | 24.5 kW |
| IM240 | 0.9811 | 0.9384 | 1.0804 | 22.8 kW |
| Manhattan | 0.9452 | 1.0009 | 1.0704 | 12.4 kW |
| Nuremberg | 0.9452 | 1.0072 | 1.0746 | 13.5 kW |
| NYCC | 0.9698 | 0.9908 | 1.0866 | 14.3 kW |
| SC03 | 0.9620 | 0.9957 | 1.0171 | 21.0 kW |
| US06 | 0.9152 | 1.1243 | 0.9827 | 34.6 kW |
| UDDS | 0.9196 | 0.9586 | 0.8536 | 17.2 kW |
| Amended UDDS | 0.9604 | 0.9745 | 1.0331 | 15.6 kW |

From the perspective of differences, firstly, the ARB02, US06 and UDDS have more divergent PSC distribution and thus are worse fitted, compared with the other drive cycles. This can be reflected by the evident outliers in Fig. 4 or the R2 in Table 2, while can be explained by the comparison between UDDS and Amended UDDS. The UDDS has one singular micro-trip that breaks the consistency of driving behaviour, which obliges the optimal offline EMS to fit mixed driving behaviour. As a result, the UDDS has a relatively divergent PSC distribution and a low R2 at 0.9196. In comparison, the Amended UDDS only removes the singular micro-trip but consequently has less divergent PSC distribution and a significantly improved R2 at 0.9604. Hence, even one singular micro-trip can dramatically impact the optimal offline EMS of the whole drive cycle. Inspired by this, when implementing the online EMS, every micro-trip should be taken into account separately; the perception horizon should not cover more than one micro-trip in case the singular micro-trips are averaged with the other micro-trips. This concern is realised by a variable perception horizon, which is illustrated in Section 4.1. Secondly, the kI and kIII of ARB02, US06 and UDDS are more deviated from 1, compared with the other drive cycles. This can also be attributed to the consistency of driving behaviour. As can be seen from Fig. 2, Fig. 4 and Table 2, the drive cycles with more consistent driving behaviour can have more congregate PSC distribution and more precise fit curve and thus enable the kI and kIII to be closer to 1. Thirdly, the PHESS, bound of each drive cycle varies considerably from 12.4 kW to 30.6 kW. As the boundary between Regions II and III, PHESS, bound is a significant parameter determining whether the SC pack should output power. Inspired by this, when implementing the online EMS, the PHESS, bound needs to be predicted accurately, and this is realised in Section 4.2. The above differences provide guides for tailoring the online EMS adaptive to different driving conditions.

Inspired by the above similarities and differences of the optimal offline EMS, the online EMS can be implemented by imitating the “N-shape” distribution of PSC with PHESS, while adjusting the perception horizon and predicting the PHESS, bound in real-time. In other words, the real-time workloads of the online EMS are only adapting the perception horizon and the PHESS, bound, which enables the online EMS to have low complexity of processing and fast response.

4. Adaptive online EMS with variable perception horizon and NN fitting

Following the inspirations from the optimal offline EMS, this section implements the online EMS to optimise HESS operating costs in real-time. Based on the analysis in Section 3.2, the EMS can leap from optimal offline to adaptive online by realising the following functions: (1) Capture the driving behaviour of every micro-trip; (2) Predict and update the power boundary PHESS,bound in real-time; (3) Imitate the “N-shape” relationship between SC operating power and HESS power demands. These functions are realised by the variable perception horizon, the NN fitting and the rule-based strategy, respectively, as described in the following subsections. The complexity of the proposed online EMS is particularly analysed from a computational perspective.

4.1   Variable perception horizon to capture every micro-trip

This section designs a variable perception horizon, which divides the real-time driving conditions into successive micro-trips based on the consistency of driving behaviours. The consistent driving behaviours can be then captured to feed the prediction of PHESS,bound. In this section, the variable perception horizon is described in terms of working principle, while the detailed implementation will be illustrated with the whole online EMS in Section 4.3.

The working principle of the variable perception horizon can be explained with Fig. 5, where the velocity with time of a random drive cycle is depicted in red while the variable perception horizon is green. In general, each horizon is supposed to exactly contain one micro-trip, as Horizons I and II in Fig. 5. Velocity sensing is needed to determine the length of the horizon since every micro-trip starts when the velocity grows from zero and ends when the velocity drops to zero. One exception is when the micro-trip is longer than 200 seconds, the horizon will end every 200 seconds until the micro-trip ends, and the length of the horizon is from the micro-trip start to the current end, as Horizons III, IV and V. Namely, the minimum update frequency of EMS is limited to once every 200 seconds in case the EV performs long cruises with no stops, in which case the EMS should be updated at set intervals. The minimum update frequency is inspired by the fact that most micro-trips end within 200 seconds except for the ones with long cruises, as shown in Fig. 2. As each horizon ends, the driving behaviour of the current horizon will be extracted to enforce the prediction of PHESS,bound, as Predictions I to V. Each prediction returns a PHESS,bound that will be used immediately until being updated by the next prediction, as PIHESS,bound to PVHESS,bound in Fig. 5. Namely, the PHESS,bound predicted by the current horizon will be used in the next horizon, with the hypothesis that the two adjacent micro-trips is probable to have similar driving behaviour [8] so that the driving behaviour of the current horizon can be used to determine the EMS for the next horizon. P0HESS,bound is the initial PHESS,bound and is set as 20.7 kW (the mean of the data in Table 2) in this paper.

Compared with the conventional constant perception horizon, the proposed variable perception horizon does not necessarily result in more horizons, which depends on the drive cycle used and will be demonstrated in Table 7. In other words, the computational complexity of the variable perception horizon is subject to the specific drive cycle, while in general, the computational complexity of the variable perception horizon is close to that of the constant perception horizon.

Fig. 5.  Explanation of the variable perception horizon with a random drive cycle.

4.2   Prediction of power boundary based on NN fitting

This section implements the NN fitting technique to realise the prediction of PHESS,bound. The implemented NN has a low complexity because it only accepts three necessary driving states refined through analytical deductions and regression analysis, as follows.

The analytical deduction follows on from the “N-shape” relationship discussed in Section 3.2. To make the deduction transparent, Fig. 6 is plotted as a general schematic diagram derived from Fig. 4, showing the optimal distribution of PSC with PHESS under a random drive cycle. Similar to Fig. 4, the optimal PSC in Fig. 6 can be fitted into an “N-shape” polyline with three linear regions, while the slope of Regions I, II and III is 1, 0 and 1, respectively. It is assumed that the global PHESS comprises a number of discrete power demands, and the number of discrete power demands lying in Regions I and III is m and n, respectively, as denoted in Fig. 6. According to Fig. 6, the relationship between PSC and PHESS in Regions I, II and III can be expressed as Eqs. (5), (6) and (7), where PISC, PIISC and PIIISC are the optimal operating power of SC pack in Region I, II and III, respectively, PIHESS, PIIHESS and PIIIHESS are the total power demands of HESS in Region I, II and III, respectively.

Fig. 6.   General schematic diagram showing the optimal distribution of SC pack operating power with HESS total power demands under a random drive cycle.

Region I:

(5)

Region II:

(6)

Region III:

(7)

Further considering the “charge-sustaining principle” [8], the SOC of SC pack should be controlled the same before/after the EV undertakes a drive cycle. This is because the SC pack is charged during driving rather than from an external power supply so that maintaining the SOC can enable the SC pack to better sustain the proceeding drive cycles in the future. Before/after a drive cycle, the net energy consumption of SC pack (QSC) should be zero, while QSC can be further expressed as the sum of the negative (Q-SC) and positive (Q+SC) energy consumption of SC pack, as Eq. (8).

(8)

According to Fig. 6, the negative power of SC pack comes from the PISC in Region I, and the positive power of SC pack comes from the PIIISC in Region III. Also considering Eqs. (5) and (7), Q-SC and Q+SC can be then expressed as Eq. (9), where ∆t is the time step of the drive cycle, PIHESS,mean and PIIIHESS,mean are the mean power demand of HESS in Regions I and III, respectively, P-HESS,mean is the mean negative power demand of HESS and equals PIHESS,mean.

(9)

Combining Eqs. (8) and (9), PHESS,bound can be solved as Eq. (10). It can be seen that PHESS,bound is likely to be influenced by four factors: m, n, P-HESS,mean and PIIIHESS,mean. m and P-HESS,mean are online sensible, but n and PIIIHESS,mean are dependent on PHESS,bound and can only be sensed after PHESS,bound is confirmed. To further relate the four factors with PHESS,bound, Table 3 lists the four factors with PHESS,bound under ten different drive cycles. Besides, the mean positive power demand of HESS (P+HESS,mean) and the standard derivation of positive power demands of HESS (P+HESS,std) are also presented in Table 3 and will be discussed later.

(10)

Table 3.   Possible influence factors of the power boundary (PHESS,bound) under ten drive cycles *

|  | m | n | P-HESS,mean | PIIIHESS,mean | P+HESS,mean | P+HESS,std | PHESS,bound |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARB02 | 366 | 449 | -12.6 kW | 49.5 kW | 27.5 kW | 21.9 | 30.6 kW |
| HWFET | 67 | 84 | -7.3 kW | 30.0 kW | 15.5 kW | 9.4 | 24.5 kW |
| IM240 | 52 | 54 | -9.3 kW | 31.6 kW | 16.1 kW | 10.5 | 22.8 kW |
| Manhattan | 164 | 88 | -3.3 kW | 21.0 kW | 6.5 kW | 7.6 | 12.4 kW |
| Nuremberg | 209 | 104 | -2.6 kW | 21.5 kW | 7.2 kW | 8.0 | 13.5 kW |
| NYCC | 102 | 47 | -3.4 kW | 25.9 kW | 7.1 kW | 11.4 | 14.3 kW |
| SC03 | 115 | 84 | -6.9 kW | 32.9 kW | 13.4 kW | 12.2 | 21.0 kW |
| US06 | 135 | 157 | -14.6 kW | 55.6 kW | 31.4 kW | 20.4 | 34.6 kW |
| UDDS | 235 | 182 | -5.2 kW | 26.1 kW | 10.8 kW | 9.7 | 17.2 kW |
| Amended UDDS | 194 | 153 | -5.1 kW | 22.7 kW | 8.2 kW | 8.4 | 15.6 kW |

* m: Number of HESS power demands in Region I;  n: Number of HESS power demands in Region III; P-HESS,mean: Mean negative power demand of HESS;  PIIIHESS,mean: Mean power demand of HESS in Region III;  P+HESS,mean: Mean positive power demand of HESS;  P+HESS,std: Standard derivation of positive power demands of HESS;  PHESS,bound: Power boundary between Regions II and III.

Using the data in Table 3, regression analysis is performed with the possible influence factors. The ratio between m and n (m/n) is found having a linear regression relationship with P-HESS,mean and PIIIHESS,mean, as Fig. 7 (a). This regression relationship has an R2 at 0.8558 and a root-mean-square error (RMSE) at 0.2206, which means m/n is highly dependent on P-HESS,mean and PIIIHESS,mean. Therefore, m/n can be dismissed from Eq. (10) while P-HESS,mean and PIIIHESS,mean are sufficient factors to predict PHESS,bound. However, as mentioned previously, P-HESS,mean is online sensible but PIIIHESS,mean can only be sensed with a known PHESS,bound. Namely, PIIIHESS,mean is not an online implementable factor to predict PHESS,bound and should be substituted by the online implementable ones. Considering PIIIHESS,mean represents the local positive power demands in Region III and is thus dependent on PHESS,bound, the substitutes of PIIIHESS,mean should represent the global positive power demands in both Regions II and III so that they can be independent on PHESS,bound, such as the mean positive power demand (P+HESS,mean) and standard derivation of positive power demands (P+HESS,std) [48]. According to the regression analysis in Fig. 7 (b), P+HESS,mean and P+HESS,std can significantly determine PIIIHESS,mean with an R2 at 0.9891 and an RMSE at 1.399, which means PIIIHESS,mean can be substituted by P+HESS,mean and P+HESS,std. Based on the above, the online implementable factors to predict PHESS,bound are confirmed as three power-related driving states: P-HESS,mean, P+HESS,mean and P+HESS,std. These driving states are refined using the data under ten different drive cycles; hence, they are the necessary driving states for predicting the PHESS,bound under general drive cycles.

Fig. 7.   Linear regression analysis: (a) m/n with P-HESS,mean and PIIIHESS,mean; (b) PIIIHESS,mean with P+HESS,mean and P+HESS,std.

Combining Eq. (10) and the fitting equations in Fig. 7, PHESS,bound can be numerically fitted as a polynomial function of P-HESS,mean, P+HESS,mean and P+HESS,std, as Eq. (11), which is a quadratic equation combining two optimally tunned linear parts. This function can be a method to predict PHESS,bound, while the more accurate method can be realised by the NN fitting. Compared with NN fitting, the numerical fitting has to customise the type and degree of the fitting equations, which makes the prediction accuracy subject to the custom fitting equations. In contrast, we adopt the NN fitting because it does not need custom equations to predefine the relationship between prediction inputs and outputs, improving the prediction accuracy and control performances. The control performances of numerical and NN fitting will be compared in Section 5.2.

(11)

The NN is created as a feed-forward network with ten sigmoid neurons in the hidden layer and one linear neuron in the output layer, as Fig. 8, where w and b represent the weight vector and bias vector, respectively [22]. The hidden layer translates the nonlinear relationship between the input and output, while the output layer enables a linear target as the predicted outcome [30]. The number of sigmoid neurons is determined as ten by trial and error since more neurons can improve the fitting accuracy, but excessive neurons can lead to overfitting. The input of NN is P-HESS,mean, P+HESS,mean and P+HESS,std, while the output is PHESS,bound. The data of the ten drive cycles in Table 3 is used to train and validate the NN. The ten drive cycles provide ten samples in two groups. One group of eight samples is for training, which uses the Levenberg-Marquardt backpropagation algorithm to adjust the NN according to the R2. The other group of two is for validation, which measures the network generalisation and halts training when an increase occurs in the RMSE. The training group and validation group are randomly divided as each time the NN is trained and validated. The NN is trained until the R2 of both training and validation approaches 1. In our case, the training and validation are halted with the status in Table 4. It can be seen that the NN can achieve very high regression with an R2 at 0.99 and very high accuracy with an RMSE at below 1.

Noted that the training and validation are performed offline, but after trained and validated, the NN can be deployed to make online predictions and generate new PHESS,bound, given the sensed driving states. When deployed online, the NN simply performs like a three-inputs-single-output system, and the system’s correlations are predefined through offline training and validation. In other words, the online NN can have low computational complexity since the primary complexity has been decoupled by offline training and validation.

Fig. 8.   Structure of the two-layer feed-forward neural network for prediction making.

Table 4.   Status of the neural network when training and validation are halted.

|  | Training | Validation |
| --- | --- | --- |
| R2 | 0.99 | 0.99 |
| RMSE | 1.05*10-21 | 0.84 |

4.3   The whole online EMS with rule-based strategy

Based on the aforementioned variable perception horizon and NN, the whole online EMS can be accomplished by further imitating the “N-shape” relationship between SC power and HESS power, and this is realised by the rule-based strategy. Moreover, the rule-based strategy is also responsible for constraining the SOC of SC pack, considering the SC pack has a small energy capacity (as Table 1) and thus can easily get over-charged/-discharged in practical engineering. In this section, the rule-based strategy is described together with the variable perception horizon and NN as parts of the whole online EMS. From an engineering viewpoint, the whole online EMS can be implemented with simple hardware of timer, sensor, memory and microprocessor. The flow chart in Fig. 9 shows one implementable hardware scheme to realise the proposed EMS, and the workflow is explained as follows.

Fig. 9.  Flow chart of the implementation of the whole online EMS.

As the trip starts, the timer will reset and then accumulate every second, offering index (j) to memorise the velocity and power. The velocity sensor detects the current vehicle velocity (vj), which is memorised along with the previous vehicle velocity (vj-1). The vj and vj-1 are then passed to the microprocessor to diagnose whether a new micro-trip just gets started, in which case vj-1 is zero and vj is nonzero. If yes, a new horizon will be considered starting at time tstart=j-1, and the HESS power demand (PHESS,j) will begin being memorised with time until the new micro-trip ends. Meanwhile, the prediction and update of PHESS,bound is skipped, and the previous PHESS,bound is adopted in the following rule-based strategy. If the micro-trip start is not triggered, the microprocessor will further diagnose whether the current micro-trip comes to an end, in which case vj-1 is nonzero and vj is zero. If yes, the memory of PHESS,j with time will stop, and the current horizon will be considered ending at time tend=j so that the current horizon length is from tstart to tend. If the micro-trip end is not triggered, the microprocessor will further diagnose whether the EMS minimum update frequency is reached. The Mod function [49] can realise this diagnosis by identifying whether the duration from tstart to current time can be dividable by 200. If yes, the current horizon will also be considered ending at time tend=j so that the current horizon length is from tstart to tend, but PHESS,j will continue being memorised with time because the current micro-trip has not yet ended. Once the current horizon length is confirmed, the PHESS,j memorised within the current horizon will be extracted to calculate the P-HESS,mean, P+HESS,mean and P+HESS,std, which will be further passed to the NN trained in Section 4.2. Subsequently, a new PHESS,bound is predicted and updated into the rule-based strategy.

As the workflow proceeds to the rule-based strategy, the microprocessor will first diagnose whether the current PHESS,j is positive or negative. If negative, the SC pack is expected to take advantage of the negative PHESS,j to recover its charge. However, the SOC of SC pack (SOCSC,j) should be detected and compared with SOCSC,high (set at 0.99 in this paper), which is the threshold indicating whether the SC pack is nearly full of charge and cannot be charged any more. If yes, the battery pack will accept the negative PHESS,j while the SC pack idle. Otherwise, the SC pack will be charged with its operating power (PSC,j) equal to PHESS,j, and the value of PSC,j is determined by the “N-shape” relationship discussed in Section 3.2. Consequently, the operating power of battery pack (PBAT,j) can be calculated according to Eq. (1). If PHESS,j is positive, it will be compared with the PHESS,bound, which is passed from the NN previously and is the threshold indicating whether PHESS,bound is high and SC power assistance is needed. If no, the battery pack will work alone while the SC pack idle. If yes, the SC pack is expected to perform power peaking and work together with the battery pack. However, the SOCSC,j should be detected and compared with SOCSC,low (set at 0.21 in this paper), which is the threshold indicating whether the SC pack is nearly depleted and cannot provide positive power any more. If yes, the battery pack will have to load the entire PHESS,j while the SC pack idle. If no, the SC pack will offer positive power at (PHESS,j- PHESS,bound), which is determined by imitating the “N-shape” relationship, and PBAT,j can be then confirmed by Eq. (1).

This rule-based strategy only adopts four decision conditions, and its outputs simply reflect the linear relationships between PHESS,j and PSC,j. Therefore, the rule-based strategy can have low complexity. The whole online EMS, as the combination of the variable perception horizon, NN and rule-based strategy, is not computationally complex since each component has low complexity.

5. Results and discussion

This section presents the control performances of the proposed EMS under two extra drive cycles, compared with other EMSs. The follows first introduces the comparative EMSs and the drive cycles used in the comparison, and then discuss the performances of different EMSs.

5.1   Comparative EMSs and drive cycles in the comparison

Three comparative EMSs are selected to be compared with the proposed EMS. The first EMS is the DP-based EMS mentioned in Section 3.2, which provides the optimal offline benchmark. The proposed EMS can validate its optimality by comparing itself with the optimal offline benchmark. The second and third EMSs keep the other characteristics the same as the proposed EMS, while the second EMS substitutes the variable perception horizon with a constant perception horizon of 100 seconds (the value does not directly determine EMS optimality, which will be explained by Table 7), and the third EMS substitutes the NN fitting with the numerical fitting using the fitting equation in Eq. (11). The second and third EMSs are used to validate the control performances with the variable perception horizon and NN fitting, which are the features of the proposed EMS. To distinguish these EMSs in the following presentation, the three comparative and the proposed EMSs are numbered with features listed in Table 5.

Table 5.   Four EMSs implemented in the comparison and their features.

| EMS No. | Online/offline | Features |
| --- | --- | --- |
| EMS 1 | Offline | Globally optimal benchmark |
| EMS 2 | Online | Constant perception + Neural fitting |
| EMS 3 | Online | Variable perception + Numerical fitting |
| EMS 4 (The proposed EMS) | Online | Variable perception +Neural fitting |

The drive cycles used in the comparison are LA92 and REP05, as Fig. 10. The micro-trips of LA92 differ from each other noticeably in terms of velocity and power demands, so that LA92 is selected to test the EMSs in terms of adaptiveness to changing driving conditions. REP05 has continuously high velocity and dramatically fluctuating power demands with the maximum power demand at around 130 kW so that REP05 is selected to challenge the EMSs at aggressive driving conditions. Although the global velocity and power demands are presented, the online EMSs can only read the current and historical velocity and power demands but cannot know the future ones in advance.

Fig. 10.   Vehicle velocity and HESS power demands of the drive cycles used in the comparison: (a) LA92, (b) REP05.

5.2   Control performances of different EMSs

The control performances of the four EMSs are compared from four aspects: (1) Cost optimisation performances, which reflect the four EMSs’ optimality of reducing HESS operating costs. (2) Perception performances, which compare the constant and variable perception horizon in terms of horizon statistics. (3) Prediction performances, which demonstrate the online EMSs’ accuracy of predicting PHESS,bound, compared with the optimal offline EMS. (4) Imitation performances, which focuses on the proposed EMS’s capability of imitating the optimal offline EMS.

(1) Cost optimisation performances.

Table 6 presents the QHESS,  EBAT,loss and JOP, which are formulated in Section 3.1, of four EMSs under two drive cycles. The QHESS and EBAT,loss of three online EMSs are all higher than those of EMS 1, while the difference of QHESS among the three online EMSs is less than 1%, but the difference of EBAT,loss can be more than 6%. The JOP of three online EMSs indicates that EMS 4 is the best online EMS with the smallest JOP, followed by EMS 2 with a slightly higher JOP, while EMS 3 has a noticeable gap with the other EMSs. Considering EMS 3 adopts the numerical fitting while EMSs 2 and 4 adopt the neural fitting, the neural fitting technique can enable the EMS to realise much better optimality. Table 6 also presents the cost optimisation rate (τ), which is defined as Eq. (12), where JOP,EMS1 is the HESS operating costs of EMS 1 (optimal benchmark), JOP,EMSX is the HESS operating costs of EMS 2 or 3 or 4. EMS 2 can realise a high τ at more than 97%, but EMS 4 is even more elevated. Considering the only difference between EMS 2 and EMS 4 is the implementation of either constant or variable perception horizon, the variable perception horizon can enable the EMS to further improve optimality.

(12)

Table 6.   Cost optimisation performances of four EMSs under two drive cycles.

| Drive cycle | EMS No. | HESS electricity consumption (QHESS, Wh) | Battery capacity loss (EBAT,loss, Wh) | HESS operating costs (JOP, USD/100km) | Cost optimisation rate (τ) |
| --- | --- | --- | --- | --- | --- |
| LA92 | EMS 1 | 3671 | 4.207 | 42.26 | 100% |
|  | EMS 2 | 3731 | 4.318 | 43.35 | 97.4% |
|  | EMS 3 | 3702 | 4.881 | 48.68 | 84.8% |
|  | EMS 4 | 3733 | 4.286 | 43.06 | 98.1% |
| REP05 | EMS 1 | 7238 | 8.492 | 41.88 | 100% |
|  | EMS 2 | 7321 | 8.753 | 43.05 | 97.2% |
|  | EMS 3 | 7294 | 9.263 | 45.41 | 91.6% |
|  | EMS 4 | 7324 | 8.734 | 42.96 | 97.4% |

(2) Perception performances.

To excavate the influence of constant/variable perception horizon on EMS optimality, the horizon statistics of EMS 2 and EMS 4 are compared in Table 7. EMS 3 adopts the same variable perception horizon as EMS 4 does and is not presented in Table 7. The mean horizon length reflects the mean timespan of driving behaviour being captured, while the number of horizons reflects how many predictions are made and updated into the EMS.

Table 7.   Perception performances of two online EMSs under two drive cycles.

| Drive cycle | EMS No. | Mean horizon length (seconds) | Number of horizons (i.e., number of predictions) |
| --- | --- | --- | --- |
| LA92 | EMS 2 | 100 | 14 |
|  | EMS 4 | 84.4 | 17 |
| REP05 | EMS 2 | 100 | 14 |
|  | EMS 4 | 251.6 | 11 |

In general, the mean horizon length and number of horizons have a negative correlation, as more horizons can be divided with each horizon being shorter. Existing studies have debated whether a long or short horizon length and more or fewer horizons can improve EMS optimality, as follows. On the one hand, existing studies [30] tend to conclude that either a long or short horizon length can improve EMS optimality. For those in favour of the long horizon length, their argument is that a long horizon can collect more comprehensive driving behaviour to feed the prediction; for the short horizon length, the argument is that a short horizon can better capture the fast-changing driving behaviour. However, Table 7 indicates that the horizon length does not necessarily impact EMS optimality, as explained below. Under LA92 drive cycle, EMS 4 has a longer horizon length than EMS 2; while under REP05 drive cycle, EMS 4 has a shorter horizon length than EMS 2. However, under whichever drive cycle, EMS 4 can always achieve a higher cost optimisation rate than EMS 2 (Table 6). Therefore, the horizon length does not directly determine EMS optimality. On the other hand, there are also debates [50] on whether more or fewer horizons can improve EMS optimality. The former argues that more horizons can enable the predictions to be made and updated more frequently so that the EMS can be adaptive to instantaneous driving behaviour, while the latter considers that excessive horizons can result in the HESS being overcontrolled. However, Table 7 indicates that the number of horizons does not necessarily impact EMS optimality, as explained below. As shown in Table 7, EMS 4 has more horizons than EMS 2 under LA92 but fewer horizons under REP05. However, EMS 4 always has better optimality than EMS 2 under whichever drive cycle (Table 6). Therefore, the number of horizons does not directly determine EMS optimality. Based on the above analysis and that in Section 3.2, this paper holds the view that neither horizon length nor the number of horizons necessarily matters with EMS optimality; what matters is whether the horizon can capture the consistent driving behaviour within every micro-trip. By changing the horizon length adaptively, the proposed variable perception horizon aims to recognise every micro-trip and extract the consistent driving behaviour to feed prediction making, something no constant perception horizon can realise. As a result, EMS 4 with the variable perception horizon realises better optimality than EMS 2 with the constant perception horizon.

(3) Prediction performances.

The predicted PHESS,bound of three online EMSs is compared with the PHESS,bound of EMS 1, as Fig. 11. The PHESS,bound of EMS 1 is solved offline and remains unchanged globally, while the PHESS,bound of each online EMS is predicted online and varies with time. The PHESS,bound of EMS 3 stays far above that of EMS 1 and thus is not well predicted. In contrast, the PHESS,bound of EMS 2 and EMS 4 fluctuates closely around that of EMS 1 and is much better predicted. Considering the only difference between EMS 3 and EMS 4 is either numerical fitting or neural fitting, it means that neural fitting can realise much higher prediction accuracy than numerical fitting. The only difference between EMS 2 and EMS 4 is the implementation of either constant or variable perception horizon, while the predicted PHESS,bound of EMS 2 is quite different from that of EMS 4, which indicates that the constant/variable perception horizon also has an evident impact on the prediction. However, it is difficult to tell from Fig. 11 whether EMS 2 or EMS 4 can better predict PHESS,bound. In this case, Table 8 compares the prediction statistics of EMS 2 and EMS 4. Firstly, the mean PHESS,bound of EMS 4 is closer to the PHESS,bound of EMS 1, which indicates that the PHESS,bound of EMS 4 is more accurate in general. Secondly, the standard deviation of EMS 2 is smaller, which means the PHESS,bound of EMS 2 is more stable with smaller shifts. Lastly, the mean relative error of EMS 4 is smaller than that of EMS 2, which confirms that EMS 4 has higher overall accuracy than EMS 2. Namely, variable perception horizon can better assist the prediction of PHESS,bound than constant perception horizon.

Fig. 11.   Predicted PHESS,bound of four EMSs under two drive cycles: (a) LA92, (b) REP05.

Table 8.   Prediction performances of two online EMSs under two drive cycles.

| Drive cycle | EMS No. | Mean PHESS,bound | Standard deviation of PHESS,bound | Mean relative error with the PHESS,bound of EMS 1* |
| --- | --- | --- | --- | --- |
| LA92 | EMS 2 | 19.80 | 4.50 | 16.38% |
|  | EMS 4 | 20.28 | 4.85 | 16.21% |
| REP05 | EMS 2 | 24.28 | 7.38 | 25.24% |
|  | EMS 4 | 27.7 | 8.01 | 23.2% |

*: The PHESS,bound of EMS 1 under LA92 and REP05 are 20.1kW and 30.8kW, respectively.

(4) Imitation performances.

To validate whether the proposed online EMS can perform like the optimal offline EMS, the power and SOC of SC pack are compared between EMS 4 and EMS 1, as Fig. 12. It should be noted that this is a very rigorous comparison since the online EMS cannot know the global driving conditions in advance, but the offline EMS can, so that the online EMS can mostly be locally optimal and is bound to have a gap with the globally optimal offline EMS.

It can be seen from Fig. 12 (a) that under LA92, the SC power of EMS 4 substantially overlaps with that of EMS 1, except at around 400 and 900 seconds. This is because, as shown in Fig. 12 (b), the SOC of EMS 4 stays low at around 400 and 900 seconds; as a result, the SC pack of EMS 4 cannot continuously provide high power output as the SC pack of EMS 1 can. In contrast, under REP05,  the SC power of EMS 4 does not coincide well with that of EMS 1, as Fig. 12 (c). The best match of SC power occurs from 380 to 650 seconds and from 1100 to 1400 seconds, during which the SC pack of EMS 4 has sufficient SOC to support continuous high power output, as shown in Fig. 12 (d). Fig. 12 (d) also shows that the SOC of EMS 4 keeps low-level operating in most the time of REP05. This is because REP05 is a very aggressive drive cycle with continuously high power demands (Fig. 10 (b)), making the SC pack almost depleted most of the time. At the end of LA92 and REP05, the SOC of EMS 4 rises back to 0.63 and 0.66, respectively, which are close to the initial SOC at 0.7 so that EMS 4 greatly fulfils the “charge-sustaining principle”. Based on the above, EMS 4 accords well with EMS 1 under LA92, but does not imitate EMS 1 very well under REP05 because REP05 is a very aggressive drive cycle that depletes the SC pack. Despite this, the proposed EMS 4 is more capable than the referenced online EMSs and has proved its optimality by the cost optimisation rate in Table 6.

Fig. 12.   Comparison between the optimal offline EMS (EMS 1) and the proposed online EMS (EMS 4): (a) Operating power of SC pack under LA92, (b) SOC of SC pack under LA92, (c) Operating power of SC pack under REP05, (d) SOC of SC pack under REP05.

6. Conclusion

This paper presents an adaptive EMS that optimises the operating costs of  EV-mounted battery-SC HESS and can be implemented with simple hardware. The proposed EMS has three components: the variable perception horizon to capture consistent driving behaviour, the neural network to predict the threshold for battery-SC power splitting, and the rule-based strategy to imitate the optimal offline benchmark. Compared with existing studies, the proposed EMS has the following advantages: (1) Rather than using a complicated framework to integrate the whole EMS, the proposed EMS is integrated with low complexity using the rule-based strategy, which is designed based on careful analysis of optimal offline benchmark. It is found that the optimal operating power of SC is highly piecewise linear with HESS power demand, which enables the whole EMS to be integrated with significantly reduced complexity. (2) Rather than perceiving mixed driving behaviours, the proposed variable perception horizon can accurately cluster consistent driving behaviours by breaking down real-time driving conditions into successive micro-trips. By accepting the consistent driving behaviours, the accuracy of online prediction can be improved, and consequently, the EMS optimality is also improved. (3) Rather than burdening the neural network with many unsubstantiated inputs, the proposed neural network only accepts three necessary inputs that are refined based on analytical deductions and regression analysis. In this way, the complexity of the neural network is significantly lowered. Benefiting from the above advantages, the proposed EMS shows high control effectiveness, with the cost optimisation rate being more than 97% under LA92 and REP05 drive cycles. Apart from demonstrating the optimisation results, statistical analysis is performed to explain why the proposed variable perception horizon and neural network techniques can improve EMS optimality. Future work will adopt more drive cycles to train the neural network and improve the variable perception horizon by optimising the maximum horizon length.

Acknowledgements

The first author gratefully acknowledges the financial supports from China Scholarship Council and University of Southampton.

References

[1] Yang B, Zhu T, Zhang X, Wang J, Shu H, Li S, et al. Design and implementation of Battery/SMES hybrid energy storage systems used in electric vehicles: A nonlinear robust fractional-order control approach. Energy. 2019:116510.

[2] Bonges III HA, Lusk AC. Addressing electric vehicle (EV) sales and range anxiety through parking layout, policy and regulation. Transportation Research Part A: Policy and Practice. 2016;83:63-73.

[3] Berckmans G, Messagie M, Smekens J, Omar N, Vanhaverbeke L, Van Mierlo J. Cost projection of state of the art lithium-ion batteries for electric vehicles up to 2030. Energies. 2017;10:1314.

[4] Ostadi A, Kazerani M. A comparative analysis of optimal sizing of battery-only, ultracapacitor-only, and battery–ultracapacitor hybrid energy storage systems for a city bus. IEEE Transactions on Vehicular Technology. 2015;64:4449-60.

[5] Ju F, Zhang Q, Deng W, Li J. Review of structures and control of battery-supercapacitor hybrid energy storage system for electric vehicles.  2014 IEEE International Conference on Automation Science and Engineering (CASE)2014. p. 143-8.

[6] Ehsani M, Gao Y, Emadi A. Modern Electric, Hybrid Electric, and Fuel Cell Vehicles: Fundamentals, Theory, and Design, Second Edition: Taylor & Francis; 2009.

[7] Wieczorek M, Lewandowski M. A mathematical representation of an energy management strategy for hybrid energy storage system in electric vehicle and real time optimization using a genetic algorithm. Applied Energy. 2017;192:222-33.

[8] Onori S, Serrao L, Rizzoni G. Hybrid electric vehicles: energy management strategies: Springer; 2016.

[9] Romaus C, Gathmann K, Böcker J. Optimal energy management for a hybrid energy storage system for electric vehicles based on Stochastic Dynamic Programming.  2010 IEEE Vehicle Power and Propulsion Conference2010. p. 1-6.

[10] Wang B, Xu J, Cao B, Ning B. Adaptive mode switch strategy based on simulated annealing optimization of a multi-mode hybrid energy storage system for electric vehicles. Applied Energy. 2017;194:596-608.

[11] Zhang L, Hu X, Wang Z, Sun F, Deng J, Dorrell DG. Multiobjective optimal sizing of hybrid energy storage system for electric vehicles. IEEE Transactions on Vehicular Technology. 2017;67:1027-35.

[12] Du J, Zhang X, Wang T, Song Z, Yang X, Wang H, et al. Battery degradation minimization oriented energy management strategy for plug-in hybrid electric bus with multi-energy storage system. Energy. 2018;165:153-63.

[13] Ruan H, Sun B, Zhu T, He X, Su X, Cruden A, et al. Compound self-heating strategies and multi-objective optimization for lithium-ion batteries at low temperature. Applied Thermal Engineering. 2020:116158.

[14] Song Z, Zhang X, Li J, Hofmann H, Ouyang M, Du J. Component sizing optimization of plug-in hybrid electric vehicles with the hybrid energy storage system. Energy. 2018;144:393-403.

[15] Xiong R, Chen H, Wang C, Sun F. Towards a smarter hybrid energy storage system based on battery and ultracapacitor-A critical review on topology and energy management. Journal of cleaner production. 2018;202:1228-40.

[16] Peng J, He H, Xiong R. Rule based energy management strategy for a series–parallel plug-in hybrid electric bus optimized by dynamic programming. Applied Energy. 2017;185:1633-43.

[17] Dawei M, Yu Z, Meilan Z, Risha N. Intelligent fuzzy energy management research for a uniaxial parallel hybrid electric vehicle. Computers & Electrical Engineering. 2017;58:447-64.

[18] Zhang L, Hu X, Wang Z, Ruan J, Ma C, Song Z, et al. Hybrid electrochemical energy storage systems: An overview for smart grid and electrified vehicle applications. Renewable and Sustainable Energy Reviews. 2020:110581.

[19] Liu C, Wang Y, Wang L, Chen Z. Load-adaptive real-time energy management strategy for battery/ultracapacitor hybrid energy storage system using dynamic programming optimization. Journal of Power Sources. 2019;438:227024.

[20] Song Z, Hofmann H, Li J, Hou J, Han X, Ouyang M. Energy management strategies comparison for electric vehicles with hybrid energy storage system. Applied Energy. 2014;134:321-31.

[21] Chen Z, Xiong R, Wang C, Cao J. An on-line predictive energy management strategy for plug-in hybrid electric vehicles to counter the uncertain prediction of the driving cycle. Applied Energy. 2017;185:1663-72.

[22] Xiang C, Ding F, Wang W, He W. Energy management of a dual-mode power-split hybrid electric vehicle based on velocity prediction and nonlinear model predictive control. Applied Energy. 2017;189:640-53.

[23] Wang L, Wang Y, Liu C, Yang D, Chen Z. A power distribution strategy for hybrid energy storage system using adaptive model predictive control. IEEE Transactions on Power Electronics. 2019;35:5897-906.

[24] Zhang S, Xiong R, Sun F. Model predictive control for power management in a plug-in hybrid electric vehicle with a hybrid energy storage system. Applied Energy. 2017;185:1654-62.

[25] Du G, Zou Y, Zhang X, Liu T, Wu J, He D. Deep reinforcement learning based energy management for a hybrid electric vehicle. Energy. 2020;201:117591.

[26] Xu B, Rathod D, Zhang D, Yebi A, Zhang X, Li X, et al. Parametric study on reinforcement learning optimized energy management strategy for a hybrid electric vehicle. Applied Energy. 2020;259:114200.

[27] Wang Y, Wang L, Li M, Chen Z. A review of key issues for control and management in battery and ultra-capacitor hybrid energy storage systems. eTransportation. 2020:100064.

[28] Xiong R, Cao J, Yu Q. Reinforcement learning-based real-time power management for hybrid energy storage system in the plug-in hybrid electric vehicle. Applied Energy. 2018;211:538-48.

[29] Zhang S, Xiong R. Adaptive energy management of a plug-in hybrid electric vehicle based on driving pattern recognition and dynamic programming. Applied Energy. 2015;155:68-78.

[30] Zhang Q, Deng W, Li G. Stochastic Control of Predictive Power Management for Battery/Supercapacitor Hybrid Energy Storage Systems of Electric Vehicles. IEEE Transactions on Industrial Informatics. 2018;14:3023-30.

[31] Li X, Han L, Liu H, Wang W, Xiang C. Real-time optimal energy management strategy for a dual-mode power-split hybrid electric vehicle based on an explicit model predictive control algorithm. Energy. 2019;172:1161-78.

[32] Huang X, Tan Y, He X. An intelligent multifeature statistical approach for the discrimination of driving conditions of a hybrid electric vehicle. IEEE Transactions on Intelligent Transportation Systems. 2010;12:453-65.

[33] Cao J, Emadi A. A New Battery/UltraCapacitor Hybrid Energy Storage System for Electric, Hybrid, and Plug-In Hybrid Electric Vehicles. IEEE Transactions on Power Electronics. 2012;27:122-32.

[34] Zhao W, Wu G, Wang C, Yu L, Li Y. Energy transfer and utilization efficiency of regenerative braking with hybrid energy storage system. Journal of Power Sources. 2019;427:174-83.

[35] Song Z, Hofmann H, Li J, Han X, Ouyang M. Optimization for a hybrid energy storage system in electric vehicles using dynamic programing approach. Applied Energy. 2015;139:151-62.

[36] Vynakov O, Savolova E, Skrynnyk A. Modern electric cars of Tesla Motors company. Automation of technological and business-processes. 2016:9-18.

[37] Zhu T, Wills RGA, Lot R, Kong X, Yan X. Optimal sizing and sensitivity analysis of a battery-supercapacitor energy storage system for electric vehicles. Energy. 2021;221:119851.

[38] Kamalisiahroudi S, Huang J, Li Z, Zhang J. Study of temperature difference and current distribution in parallel-connected cells at low temperature. Int J Electr Comput Energ Electron Commun Eng. 2014;8:1471-4.

[39] Datasheet of 2.7V 3400F Ultracapacitor Cell. Maxwell Technologies Inc.; 2018.

[40] Song Z, Hofmann H, Li J, Han X, Zhang X, Ouyang M. A comparison study of different semi-active hybrid energy storage system topologies for electric vehicles. Journal of Power Sources. 2015;274:400-11.

[41] Song Z, Hofmann H, Li J, Hou J, Zhang X, Ouyang M. The optimization of a hybrid energy storage system at subzero temperatures: Energy management strategy design and battery heating requirement analysis. Applied Energy. 2015;159:576-88.

[42] Ruan J, Song Q, Yang W. The application of hybrid energy storage system with electrified continuously variable transmission in battery electric vehicle. Energy. 2019;183:315-30.

[43] Zhu T, Lot R, Wills RGA, Yan X. Sizing a battery-supercapacitor energy storage system with battery degradation consideration for high-performance electric vehicles. Energy. 2020;208:118336.

[44] Wang J, Purewal J, Liu P, Hicks-Garner J, Soukazian S, Sherman E, et al. Degradation of lithium ion batteries employing graphite negatives and nickel–cobalt–manganese oxide + spinel manganese oxide positives: Part 1, aging mechanisms and life estimation. Journal of Power Sources. 2014;269:937-48.

[45] Wang R, Lukic SM. Review of driving conditions prediction and driving style recognition based control algorithms for hybrid electric vehicles.  Vehicle Power and Propulsion Conference (VPPC), 2011 IEEE: IEEE; 2011. p. 1-7.

[46] Montazeri-Gh M, Ahmadi A, Asadi M. Driving condition recognition for genetic-fuzzy HEV control.  Genetic and Evolving Systems, 2008 GEFS 2008 3rd International Workshop on: IEEE; 2008. p. 65-70.

[47] D’Errico J. SLM—Shape Language Modeling, MATLAB central file exchange. 2014.

[48] Lin C-C, Jeon S, Peng H, Moo Lee J. Driving pattern recognition for control of hybrid electric trucks. Vehicle System Dynamics. 2004;42:41-58.

[49] Attaway S. Matlab: a practical introduction to programming and problem solving: Butterworth-Heinemann; 2013.

[50] Golchoubian P, Azad NL. Real-Time Nonlinear Model Predictive Control of a Battery/Supercapacitor Hybrid Energy Storage System in Electric Vehicles. IEEE Transactions on Vehicular Technology. 2017;66:9678-88.