This is a preliminary PDF of the author-produced manuscript that has been peer-reviewed and accepted for publication in Acta Mechanica Sinica (AMS) , since it is being posted soon after acceptance, it has not yet been formatted, or processed by AMS Publications. This preliminary version of the manuscript may be downloaded, distributed, and cited, but please be aware that there will be visual differences and possibly some content differences between this version and the final published version.

The DOI for this manuscript is doi:.

Please use the following full citation:

# Xing, Jing Tang: Theoretical investigations on lattice Boltzmann method: an amended MBD and improved LBM. Acta Mechanica Sinica., doi:, in press.

# Theoretical investigations on lattice Boltzmann method: an amended MBD and improved LBM

Jing Tang Xing

Maritime Engineering, School of Engineering, FEPS, University of Southampton, Southampton, UK

Abstract

This paper presents theoretical investigations of lattice Boltzmann method (LBM) to develop a completed LBM theory. Based on H-theorem with Lagrangian multiplier method, an amended theoretical equilibrium distribution function (EDF) is derived, which modifies the current Maxwell-Boltzmann distribution (MBD) to include the total internal energy as its parameter. This modification allows the three conservation laws derived directly from lattice Boltzmann equation (LBE) without additional small-parameter expansions adopted in references.  From this amended theoretical EDF an improved LBM is developed, in which the total internal energy like the mass density and mean velocity is a new macroscopic variable to be updated for different times and cells during simulations. The developed method provides a means to consider external forces and energy generation sources as generalised forces in LBM simulations.  The corresponding model and implementation process of the improved LBM are presented with its performance theoretically investigated. Analytically hand-workable examples are given to illustrate its applications and to confirm its validity. The paper will excite more researchers and scientists of this area to numerically practice the new theory and method dealing with complex physical problems, from which it is expected to further advance LBM benefiting science and engineering.

Key Words: Amended Maxwell-Boltzmann distribution, Improved LBM, Macroscopic internal energy, Conservation laws from LBE, Lagrangian multiplier method.

# 1. Introduction

LBM is based on microscopic molecule dynamics [1-5], concerning molecule distribution in space. Boltzmann transport equation (BTE) [6-9] and H theorem confirms that the EDF is MBD [7], contributed a way to understand macroscopic world and from a mesoscopic method dealing with macroscopic motions [10-12]. The solution of BTE is very difficult, and attempts have been made to simplify the collision term, for which the best-known model [13] has been implemented. LBM was developed from the lattice gas automata (LGA) [14-15]. McNamara & Zanetti [16] contributed a historic contribution to replace the Boolean variable in LGA by a real variable, and initially created LBM. The idea of LBM is imaging gases / fluids as a finite number of particles with random motions, and their exchange of momentum and energy achieved through particle streaming and collisions. Key historic publications [13-63] making LBM into new stages should be highlighted. The important review papers [39, 59-81] and the influence books [82-97] have been published. LBM is considered as an effective method to model some phenomena not easily macroscopically described by other method [98].  It has many computer codes of LBM provided [85-87, 91, 99, 100].

In practical simulations based on the current LBM, it has been realized that it is probably most suitable for isothermal weakly-compressible flows.  For complex flows, especially the ones involving high speed and large compressibility and obvious energy exchanges, it has suffered from numerical problems on accuracy, efficiency, and stability. To solve these problems, when simulating multicomponent flows, Swift et al [101,102] added an additional term of free energy in EDF for the conservation of total energy, including the surface, kinetic and internal energies to be satisfied [103]. Meanwhile, studying thermodynamics in incompressible limit, He et al [104] proposed a thermal model with an internal energy density distribution expressed by a multiplication of MBD and kinetic energy, so that the evolution equation of the introduced energy distribution together with original BTE were solved to simulate temperature fields. Until now, many recent publications, such as [105-112], still tried to add some numerical techniques, such as correctly resolve energy equation, high-order lattices, double distribution functions, hybrid LBM with finite difference or finite volume approaches etc., but nobody has deeply asked if there exist some fundamental theoretical incompleteness of the current LBM model. In the history of science advances, to solve a difficult problem reported by practices, it is not only to seek some technique-modifications on it, but also, more importantly, to check the original theory and physical mechanism of the problem to find if there is its inherent theoretical incompleteness. Based on this consideration, author through carefully reading the publications of LBM concerning its original principles and basic physical mechanism has found its following theoretical problems. We know that the background of LBM is the kinetic theory and BTE developed by studying the distribution of molecules of ideal dilute gases. Theoretical EDF obtained is MBD, in which three macroscopic variables: mass density, velocity and temperature T with Boltzmann constant K are the parameters.  In this EDF, the KT reflecting the bulk energy was derived only by the state equation of gas [6], but not concerning other types of energy, such as the work done by viscous stress and the one caused by the gradient of mass density. Therefore, the momentum and energy conservation equations derived from MBD do not include the contribution from the full stress tensor in continuum mechanics, and there is not a term in the energy conservation equation concerning the contribution of external energy generation sources. Physical flows in continuum mechanics are governed by three conservation laws, of which the three variables, the mass density, velocity, and internal energy are key parameters. When we study macroscopic flows from a viewpoint of statistical mechanics, it should have a corresponding inherent theoretical EDF including these three key parameters to allow the three conservation laws to be derived from it. Therefore, this paper intends to tackle these important theoretical problems to develop an improved theory with amended LBM model, aiming for readers to follow in practical simulations to solve the mentioned difficulties of the current LBM.

The paper is a theoretical document written exactly based on the famous Boltzmann H-theorem [6] of the statistical mechanics, the laws of continuum mechanics [113-115], the energy flow theory [116-120], and the variational method in mathematical analysis.  Section 1 from reading historical publications reveals the existing problems of LBM theory, that is the total internal energy is not involved in current EDF causing some theoretical and numerical problems. Sub-sections 2.1-2.3 derive the fundamental equations from continuum mechanics, which is used in sub-section 2.4 to develop an amended theoretical EDF including internal energy as a parameter, that is new contribution of the paper, and section 2 is a base of demonstrations in section 3 and 4. Section 3 demonstrates three conservation laws directly from the amended EDF with no small parameter expansions adopted in references. Section 4 gives the modified LBM based on the developed EDF with mass density, mean velocity and internal energy as parameters to be updated in numerical process. The formulations on generalised forces contributed by external forces and energy generation sources are provided and the implementation process of the improved method with its performance study are given. Section 5 gives some hand-workable examples by analysis to confirm the valid of theory and to illustrate applications.

# 2. Internal energy and amended theoretical EDF

To develop an amended theoretical EDF based on MBD function by introducing the total internal energy density as a macroscopic parameter, the knowledge in continuum mechanics including the state equation, material derivatives, Cauchy tress tensor, stress power, internal mechanical energy and its time / space derivatives with their statistical averages are required, which are discussed in this section.

## 2.1 Ideal gases, state equation and average internal thermal energy (AITE)

For ideal gases in the volume Ω containing N molecule particles, the state pressure p is defined by the equation of state in association with the Boltzmann constant  k in the form [6]

p=NkTΩ=nkT,          n=NΩ ,                                                     (2-1.1)

from which, we know that if the temperature  T is a constant, the particle density is a function of the volume and affects the state pressure per individual particle by

p=pn=kT.                                                                           (2-1.2)

Physically, the dimension of the state pressure per individual particle is the pressure over the mass density, i.e., Nm-2(kgm-3)=(ms)2 that is a square of the instant velocity, equaling the derivative of the pressure with respect to the mass density.

Huang [6] presented the detailed calculation of AITE based on EDF, and obtained the average thermal energy e of a particle

p=nkT=23ne,      e=3kT2,                                                           (2-1.3)

where the pressure p and the temperature T are the physically measured quantities, from which AITE of the particles per unit volume is

e=ne=3nkT2.                                                                            (2-1.4)

From Eq. 2-1.2 we may understand that the average internal energy, e, as the square of averaged velocity of sound in the fluid.

## 2.2 Cauchy tress tensor, power, and average internal mechanical energy (AIME)

In the continuum mechanics, using Cartesian tensor notations, see for examples, [113-115], the Cauchy stress tensor of fluids is given by

σij=-pδij+σij  ,        σij  =2μVij-Vllδij3,     Vij=vi,j +v j,i2,                            (2-2.1)

where p is the pressure caused by fluid flows, μ is the coefficient of viscosity, Vij is deformation rate tensor, and σij=σji denotes a symmetrical viscous stress tensor with σjj=0 , If the fluid is incompressible, the fluid dilation vanishes,  ∂vl∂xl=Vll=0,  the stress tensor reduces to

σij=-pδij+2μVij  .                                                     (2-2.2)

The power of stress per unit volume is given by σijVij, which is a measured quantity. The internal mechanical energy of the fluid per unit mass can be obtained by integration from its reference state “0” to its current state “t”

ψ=0tσijρVijdt,      DψDt=σijρVij.                                               (2-2.3)

Following the discussion on AITE per unit particle above, we can obtain AIME ψ per unit particle  in a similar form as given in Eq. 2-1.3, i.e.

ψ=ψ=32ψ,         DψDt=DψDt=σijρVij.=σijVijn ,                             (2-2.4)

which  ψ has the dimension of pressure p per unit particle shown in Eq. 2.1.1. As a result of this, AIME of the particles per unit volume and its time derivative can be given by

ψ=nψ=3n2ψ,           DψDt=nDψDt= σijVij,                                            (2-2.5)

from which with Eq. 2-2.3, it follows that the time change rate of AIME of the particles per unit volume is

DψDt=3nDψ2Dt=σijui,j.                                                                (2-2.6)

Here (),j=∂()∂xj, as used in tensor analysis [113-115], the σijui,j is understood as the averaged one, although the same stress notations are adopted.

## 2.3 Material derivatives and gradient of internal mechanical energy per unit volume

A material point marked by its original position coordinate Xj moves in the space, and it arrives the position xi=(Xj,t) at time t, so that its velocity vi=∂xi∂tX and acceleration ai=∂vi∂tX  where the subscript X implies the derivatives with respect to time t are taken for the same material point. This time derivative is called as a material derivative [113-115]. In continuum mechanics, the Eulerian description of motion represents motion quantities in ()(xi,t) as functions of spatial position and time, which may be a scalar, vector, or tensor function. In the Eulerian description of motion, the material derivative is

D()Dt=∂()∂t+(dxidt)X∂()∂xi=∂()∂t+vi∂()∂xi.                            (2-3.1)

For a set of material points in a volume Ω with its surface S in the space at time t, the total quantity () of this set of particles is

I(t)=Ω(t)()dΩ ,                                                               (2-3.2)

where volume  Ω is also a function of time due to the motion. The material derivative of the quantity I(t) is

DI(t)Dt=DDtΩ(t)()dΩ=Ω∂()∂tdΩ+S()viνidS ,                                       (2-3.3)

where, physically, on the right-hand side the first integration denotes the time change rate of the quantity in the fixed volume, while the second one is caused by the boundary motion. Using the Green theorem [113-115], we can re-write above equation as

DI(t)Dt=Ω∂()∂t+∂()vi∂xidΩ=Ω∂()∂t+()∂vi∂xidΩ.                                 (2-3.4)

Introducing the mass density ρ and the mass conservation equation, we obtain

DDtΩρ()dΩ=ΩρD()DtdΩ.                                                               (2-3.5)

2.3.1 Gradient of internal mechanical energy per unit volume in equilibrium state

The gradient of the internal mechanical energy per unit volume in equilibrium state can be derived by using Fig. 1, in which a volume element ∆Ω closed by surface ∆S of unit normal vector νi pointing from inside to outside of the volume is in its equilibrium state, so that it satisfies the equilibrium equation

νi

∆Ω

∆S

Ti

Fig. 1 The gradient of internal mechanical energy.

σij,j+fi=0,                                                                       (2-3.6)

∆Ωσij,jdΩ+∆ΩfidΩ=∆SσijνjdS+∆ΩfidΩ=∆SσijνjdS+∆Ωfi=0,

where fi is the body force per unit volume.  When the volume tends zero, the resultant force at a point vanishes

TR=∆SσijνjdS=0.                                                              (2-3.7)

The change of internal energy per unit volume is calculated by the work done by the stress in the gradient ∆Ui,j of displacement field Uiin the form

∆nψ=σij∆Ui,j,                                                               (2-3.8)

from which, when integrated over the volume ∆Ω, and then using the Green theorem, it follows

∆Ω∆nψdΩ=∆Ωσij∆Ui,jdΩ=∆Sσij∆UiνjdS-∆Ωσij,j∆UidΩ.                     (2-3.9)

Since the volume is very small, we approximate the displacement as the one at the central point, therefore, based on Eq. 2-3.7 in equilibrium state, we obtain the gradient of the internal mechanical energy per unit volume as

∆Ω∆nψdΩ=∆Ui∆SσijνjdS-∆Ωσij,j∆UidΩ=-∆Ωσij,j∆UidΩ,                   (2-3.10)

∆nψ=-σij,j∆Ui,            ∆nψ∆Ui=-σij,j ,              ∂nψ∂xi=-σij,j ,

where we have introduced xi=Xi+Ui, and the particle displacement increment ∆Ui=∆xi.

2.3.2 Time change rate of AIME per unit volume in equilibrium state

The time change rate of AIME per unit volume in the equilibrium state can be derived from the principle of virtual power. Since the system is in equilibrium state, there is no power exchange with outside, so that the summation of the stress power and the time change rate of AIME per unit volume vanishes

-∆SσijuiνjdS=DDt∆ΩψdΩ=∆Ω3nDψ2DtdΩ,                                            (2-3.11)

from which, when the Green theorem used, it follows

-∆Ω(σijui),jdΩ=∆Ω3nDψ2DtdΩ.                                                     (2-3.12)

Since the volume ∆Ω  is arbitrary, we have

3nDψ2Dt=-(σijui),j=qj,j,                                                              (2-3.13)

where qj is defined as the averaged energy flow-density vector [116-117]

2.3.3 Equation satisfied by the internal mechanical energy per unit mass

The equilibrium equation of power denoted by the internal mechanical energy per unit mass in the equilibrium state is obtained by investigating Eq. 2-3.11, which, when the material derivative Eq. 2-3.4 used, becomes

∆SσijviνjdS=DDt∆ΩψdΩ=∆ΩDψDt+ψvi,idΩ=∆Ω(σijVijρ+ψvi,i)dΩ,              (2-3.14)

where “^” is to identify the quantity involving the ones for unit mass only. As a result of this, and when the Green theorem used, it follows

∆Ω(σijvi),jdΩ=∆Ω(σijvi),jdΩρ=∆Ω(σijVijρ+ψvi,i)dΩ,                               (2-3.15)

where dΩ denotes the volume element per unit mass equaling dΩρ. Therefore, from Eq. 2-3.15, we have

σijVijρ+ψvi,i=-qj,jρ,          σijVij+nψvi,i+qj,j=0,                              (2-3.16)

where the mass density is replaced by the particle density n.

## 2.4 Boltzmann H-theorem and amended theoretical EDF

The famous MBD function was derived for ideal gases with its AITE in the form e=3nkT/2 [6], therefore, the momentum and energy conservation equations derived from MBD function does not include the contribution from the stress tensor in continuum mechanics. As mentioned in introduction, for simulating complex thermodynamic and multicomponent flows, etc., several technique modifications [101-112] were proposed to deal with some numerical issues met in their practices, but it has not solved the theoretical incompleteness of the current theory.

As discussed in subsection 2.2, we have introduced an AIME per unit volume and per unit particle based on the theory of continuous mechanics and statistical average, which are summarized in the forms

e=K+ε,      ε=e+ψ      ε=e+ψ= 3T2,

K=nK,        e=ne,         ψ=nψ,                                                   (2-4.1)

K=uiui2 ,    e=3kT2,    ψ=3ψ2,

where e denotes the total internal energy per unit volume, which consists of the kinetic one K and the thermal mechanical one ε  equaling a summation of the thermal one  e  and the mechanical one ψ. The hats “^” denote the corresponding variables per unit particle. These averaged internal energies are macroscopic quantities being the functions of position xi  and time t.

Based on the developed AIME and the Boltzmann’s H theorem [6], we can derive the amended theoretical EDE. We can conclude that EDF f0x,v,t in a volume Ω for a prescribed density n , mean momentum nu, and energy per unit volume ne=0.5nu∙u+ε,   ε=1.5nKT+ψ=1.5nT  minimizes the H functional

Ht=fx,v,tlogfx,v,tdΩdV,                                               (2-4.2)

where dV=dv1dv2dv3 is the differential volume element of the velocity space. We require that the mass, momentum, and energy of this particle system in the volume Ω  are conservative, i.e.

nΩ=fx,v,tdΩdV,     nuΩ=vfx,v,tdΩdV,     neΩ=0.5(v∙v)fx,v,tdΩdV,      (2-4.3)

which are the constrains of the H functional.  Using the Lagrangian multiplier method [99], introducing two scalar multipliers λ1 and λ3 , and a vector one λ2 to release the variational constrains in Eq. 2-4.3 of the functional in Eq.2-4.2, we obtain the new functional

Hf,λ1,λ2,λ3=fx,v,tlogfx,v,tdΩdV-λ1nΩ-fx,v,tdΩdV                      (2-4.4)

-λ2∙nuΩ-vfx,v,tdΩdV-λ3neΩ-0.5v∙vfx,v,tdΩdV.

The variation of the functional H gives

δH=δf1+logf+λ1+λ2∙v+0.5λ3v∙vdΩdV-δλ1nΩ-fx,v,tdΩdV          (2-4.5)

-δλ2∙nuΩ-vfx,v,tdΩdV-δλ3neΩ-0.5v∙vfx,v,tdΩdV,

from which, when δH=0,  it yields the constrain conditions in Eq. 2-4.3 and the equation

1+logf+λ1+λ2∙v+0.5λ3v∙v=0.                                           (2-4.6)

The solution of Eq. 2-4.6 is EDF satisfying the conservative constrains in Eq. 2-4.3, which is

f0=exp-1-λ1+λ2∙λ22λ3-λ32λ2λ3+v∙λ2λ3+v,                                         (2-4.7)

where, we have introduced the equality

λ2∙v+0.5λ3v∙v=-λ2∙λ22λ3+λ32λ2λ3+v∙λ2λ3+v                                       (2-4.8)

We consider a new set of Lagrangian multipliers d, β,γ from which the EDF is represented in the standard form

f0=dexp-v-β∙v-βγ,                                                          (2-4.9)

which satisfies the conservative laws

nΩ=f0dΩdV=Ωf0dV,

nuΩ=vf0dΩdV=Ωvf0dV,                                                 (2-4.10)

0.5nu∙u+3kT+ψΩ=0.5v∙vf0dΩdV=Ω0.5v∙vf0dV.

Since the distribution is homogeneous, and it does not involve the spatial variables, the space integration gives the total volume Ω in front of integration on velocity space. Solving Eq. 2-4.10 with Eq. 2-4.9, we can obtain the tree parameters. Here, the main contribution of this paper is the internal mechanical energy is introduced in the energy conservation equation, based which the improved theoretical EDF derived as follows.

Using the Gaussian integrals and doing the mathematical works as deriving MBD function [6], from Eq. 2-4.10, we obtain

n=f0dV=dπγ32,     d=nπγ32,                                             (2-4.11)

nu=(v-β)f0dV+βf0dV=0+nβ,         β=u,

and especially the one for the energy conservation involving ψ

0.5nu∙u+3kT+ψ=0.5v∙vf0dV=0.5v-β∙v-βf0dV+0.5nv∙v,

3nkT+ψ=nπγ32×3γπγ322=3γn2,       γ=2kT+ψ.                     (2-4.12)

A substitution of the parameters obtained in Eqs. 2.4.11~12 into Eq. 2-4.9 gives the new amended EDF as

f0=n2πkT+ψ32exp-v-u∙v-u2kT+ψ.                                               (2-4.13)

Using this amended EDF and the tensor notations [113-115], we have the following integration formulations

f0dV=n,        f0(vi-ui)dV=0,

f0vi-uivj-ujdV=nkT+ψδij,        f0vi-uivj-ujvl-uldV=0,    (2-4.14)

f0vi-uivj-ujv-u∙v-udV=5nkT+ψ2δij.

Using the notation kT+ψ=T for the mechanical energy included, it is not difficult to demonstrate that

f0=n,       f0vi=nui,        f0vivj=nTδij+nuiuj,                       (2-4.15)

f0vivjvl=nT(ulδij+uiδlj+ujδil)+nuiujul,

f0vivjvlvl=5nT2δij+nT7uiuj+ululδij+nuiujulul.

# 3. Conservation laws

Here, we examine if the developed amended EDF satisfying BTE, can be used to obtain the macroscopic conservation laws, which has not been fully addressed by using MBD function. If the answer is positive, this amended EDF will provide the basis to construct an improved LBM scheme to simulate various complex physical problems dominated by the changes of internal energy. This is because the general concepts in BTE are not only applicable for dilute gases, but also for much denser fluids.

## 3.1 Conservation theorem in a differential form

The EDF fx,v,t satisfies a partial differential equation

∂∂t+v∙∂∂x+Fv∙∂∂v+ϑK∂∂0.5v∙vf=∂f∂tC,       Fv=dvdt ,      ϑK=ddtv∙v2=dKdt.              (3-1.1)

Here, a new term on the change of kinetic energy energy v∙v2=K introduced, since the amendes EDF includes the internal energy as marcroscopic parameter, and its change effect needs to be explored that is a new contribution of the paper.

The instant time change rate of the streaming velocity gives the instant acceleration per unit particle, i.e. the instant force Fv per unit partice, while the time change rate of the kinetic energy per unit partice gives the insatnt energy change rate ϑK. Obvously, The instnat values of the force  Fv and the energy change rate ϑKinvolve not only the internal motion variables, such as instant stress and streaming velcocity, but also the external quantities, such as the body force and the external energy source per unit mass. However, in the equilibirum state, the internal contributions to them are canceled each other according to the Newton’s second law, so that in the averaged equilibrium state governed by BTE, the following external force F and external energy gereration rate ϑ are the averaged ones

nϑε=nϑ,            nFv=nF ,                                                (3-1.2)

respectively represnting the external body force and the external energy gneration rate per unit particle, which are assumed being independent of the streaming process, but the macroscopic quantities, possible functions of the time, space position and macroscopic velocity prescribed by the problems.

The right-hand side of Eq. 3-1.1 represents the un-balance part caused by collision in the system. We consider a conserved property ϕ(x,v) with its finite value at any point of space, so that it is independent of time t.  Multiplying Eq. 3-1.1 by this property, and then integrating the resultant equation over the phase space, we obtain

ΩVϕx,v∂∂t+v∙∂∂x+Fv∙∂∂v+ϑK∂∂0.5v∙vfdVdΩ

=ΩVϕ(x,v)∂f∂tCdVdΩ,                                                           (3-1.3)

of which, the total collision term on the right-hand side vanishes, since the total system in the space is momentum conservative, although the collision term  (∂f∂t)C at a local point in the space is not zero. Eq. 3-1.3 is re-written as

ΩV∂fϕ∂t+∂vifϕ∂xi-f∂viϕ∂xi+∂Fivfϕ∂vi-Fivf∂ϕ∂vi-fϕ∂Fiv∂vi

+∂ϑKfϕ∂0.5vivi-ϑKf∂ϕ∂0.5vivi-fϕ∂ϑK∂0.5vividVdΩ=0.                                  (3-1.4)

Using the Green theorem in the velocity space [113-115], we can transform the volume integration in the infinite velocity space V into a surface integration on its boundary S of unit normal vector νi at infinity, i.e.

V∂Fivfϕ∂vidV=SFivfϕνidS=0,                                                    (3-1.5)

since the EDF f→0, when vi→∞ on the infinte boundary S.  Furthermore,  the integration on the infinite volume  V,  VdV∂ϑKfϕ∂0.5vivi=0, can be demonstrated as follows. Referring Fig. 2, on the shpere surface of radius r the kinetic energy is a constant K=0.5vivi, so that along an arbitrary radial direction r, the differential element dK=d(0.5vivi), implying the K is taken as a cordinate to denote the corresponding sphere. As a special case dK=d0.5v12=dK1,  if the direction r is chosen as the direction of velocity cordinate v1 in the vellocity space. Therefore, we can use the Green theorem and the property of EDF vanishing at infinite to  calculate the integration

VdV∂ϑKfϕ∂0.5vivi=V∂ϑKfϕ∂KdV=V∂ϑKfϕ∂K1dV=SϑKfϕν1dS=0.          (3-1.6)

Defining the average value of variable A(x,v,t) as

A=A=fAdVfdV=fAdVn,                                          (3-1.7)

from which, when substituted into Eq. 3-1.4, and considering Eqs. 3-1.5 and 3-1.6, it follows

Ω∂nϕ∂t+∂nviϕ∂xi-nvi∂ϕ∂xi-nFiv∂ϕ∂vi-nϕ∂Fiv∂vi -nϑK∂ϕ∂0.5vivi-nϕ∂ϑK∂0.5vividΩ=0. (3-1.8)

where we have completed the integration in the momentum space and noticed that nA=nA, since n is independent of v.

Equation 3-1.8 is the general conservation theorem in an integration form, in which there is no restrictions on the size Ω. When we choose a small deferential volume Ω, we obtain the conservation theorem in the differential form

∂nϕ∂t+∂nviϕ∂xi-nvi∂ϕ∂xi-nFiv∂ϕ∂vi-nϕ∂Fiv∂vi -nϑK∂ϕ∂0.5vivi-nϕ∂ϑK∂0.5vivi=0,        (3-1.9)

and furthermore

∂nϕ∂t+∂nviϕ∂xi-nvi∂ϕ∂xi-nFiv∂ϕ∂vi-nϑK∂ϕ∂0.5vivi=0,                     (3-1.10)

since in the equilibrium state the external force Fv and the external energy generation rate ϑK are independent of the streaming velocity v.

r

dK

O

v1

dK1

K=0.5vivi

Fig. 2 Sphere surface of radius r on which the kinetic energy is constant.

## 3.2 Conservation laws

Taking a different property ϕ, from Eq. 3-1.10 we can obtain its corresponding conservation law as follows.

### 3.2.1 Mass conservation

Letting ϕ=m, the constant particle mass, we obtain the mass conservation law

∂nm∂t+∂nvim∂xi=0,            ∂nm∂t+∂nuim∂xi=0.                                     (3-2.1)

where we have used the integration formulations of EDF given in Eq. 2-4.14~15. Since m is a constant, we obtain

∂n∂t+∂nui∂xi=0 .                                                            (3-2.2)

### 3.2.2 Momentum conservation

Letting ϕ=mvj, from Eq. 3-1.10, we obtain

∂nvj∂t+∂nvivj∂xi-nvi∂vj∂xi-nFivδij-nϑ∂vj∂0.5vivi=0,                       (3-2.3)

of which,

nvi∂vj∂xi=0,      nϑ∂vj∂0.5vivi=ndKdt∂vj∂K=nvj=nFjK,                         (3-2.4)

due to, as mentioned, the partial derivative of velocity vi with respect to coordinate xi vanishes. Furthermore, from Eqs. 2-4.14~15 it follows

vivj=(vi-ui)(vj-uj)+vjui+viuj-uiuj=pij+uiuj,             (3-2.5)

pij=(vi-ui)(vj-uj)=kT+ψδij,

which with Eqs. 3-2.4 are substituted into Eq. 3-2.3 and using the related integration formulations, we obtain

∂(nuj)∂t+∂(nuiuj)∂xi+∂(nkT)∂xj+∂(nψ)∂xj-nFj=0,                                (3-2.6)

where we have used Eq. 3-1.2 to obtain nFjK+nFivδij=nFjT=nFj.

When the mass conservation Eq. 3-2.1 introduced into Eq. 3-2.6, we obtain its first two terms

∂(nuj)∂t+∂(nuiuj)∂xi=uj∂n∂t+∂nui∂xi+n∂uj∂t+ui∂uj∂xi,                     (3-2.7)

which gives

n∂uj∂t+ui∂uj∂xi=nDujDt=-∂nkT∂xj-∂nψ∂xj+nFj,                                (3-2.8)

that is the momentum conservation law for fluids. In this equation, on the right-hand side, the first term  ∂nkT∂xj denotes the contribution from thermal pressure p=nkT given in Eq. 2-1.1, the second term ∂nψ∂xj is the contribution from the stress tensor as shown in Eq. 2-3.10, therefore we obtain

nDujDt=-∂p∂xj+σji,i+nFj                                                          (3-2.9)

### 3.2.3 Energy conservation

### Mechanical energy conservation

When we do not interest in thermal effects, the term ∂p∂xj=0, so that we have the momentum conservation Eq. 3-2.9 for mechanical systems. Multiplying  uj on both sides of the mechanical momentum equation, we obtain

nD(0.5ujuj)Dt=σji,iuj+nFjuj,                                               (3-2.10)

that is

nD(0.5ujuj)Dt=(ujσji),i-σjiVji+nFjuj=-qi,i-σjiVji+nFjuj,             (3-2.11)

from which, when Eq. 2-2.6 used for AIME of the same mass per unit volume, it yields

nD(0.5ujuj)Dt+3nDψ2Dt=-qi,i+nFjuj.                                 (3-2.12)

Here qi is the energy-flow density vector, of which the positive value implies the energy flowing from the inside to outside of the volume. Physically, Eq. 3-2.12 is the conservation law of mechanical energy, also called as the energy flow equilibrium equation and energy flow density vector included [116-118], which represents that the summation of the time change rate of kinetic and mechanic-internal energy equals a summation of the power of the body force nFj per volume and the power flowing into the body from outside.

### Total energy conservation

To derive the total energy conservation, we take ϕ=mvjvj2  in Eq. 3-1.10 and obtain

∂nvjvj2∂t+∂nvivjvj2∂xi-nvi∂vjvj2∂xi-nFiv∂vjvj2∂vi-nϑK∂vjvj2∂0.5vivi=0,          (3-2.13)

where due to the same reason for Eq.3-2.4, we have

nvi∂vjvj2∂xi=0,  nFiv∂vjvj2∂vi=nvivvjδij=nddt[(vivi)v/2]=nϑv,     (3-2.14)

from which, when substituted into Eq. 3-2.13 and using the result by Eq. 3-1.2

nϑv+nϑK∂vjvj2∂0.5vivi=nϑT=nFjuj+nϑ,                      (3-2.15)

it follows

∂nvjvj2∂t+∂nvivjvj2∂xi-nFjuj-nϑ=0.                             (3-2.16)

Using Gaussian integrations [99], we obtain

vjvj=3kT+ψ+ujuj,               vivjvj=5kT+ψui+uiujuj,                 (3-2.17)

by which, the first two terms on the left-hand side in Eq. 3-2.16 become

nD3kT+ψ2+ujuj2Dt+3kT+ψ2+ujuj2∂n∂t+∂nui∂xi+∂nkT+ψui∂xi =nD3kT+ψ2+ujuj2Dt+∂nkT+ψui∂xi,   (3-2.18)

where the mass conservation Eq. 3-2.1 has been used. Furthermore, defining

∂nkTui∂xi=hi,i,                                                            (3-2.19)

and considering

∂nψui∂xi=∂(nψ)∂xiui+nψ∂ui∂xi=-σij,jui+nψ∂ui∂xi=qj,j+σijui,j+nψ∂ui∂xi=0,      (3-2.20)

obtained from Eq. 2-3.10 and Eq. 2-3.16, we can arrange Eq. 3-2.16 as

nD3kT/2Dt+nD(ujuj/2)Dt+3nDψ2Dt+hi,i-nFjuj-nϑ=0.                            (3-2.21)

When the conservation of mechanical energy in Eq. 3-2.12 is introduced into Eq. 3-2.21, and considering Eq. 2-2.6 and Eq. 2-3.13 averagely giving  qi,i= σijVij, we obtain the thermal energy conservation equation

nD[3kT/2]Dt=-hi,i+qi,i+nϑ=-hi,i+σijVij+nϑ.                          (3-2.22)

Physically, on the right-hand side of this equation, the first term denotes the thermal energy flow, the second term is the tress power, which transforms into thermal energy due to viscosity, and the third one is the energy generation from the external energy source.

It has been noted that the conservation laws derived from BTE by introducing the second order terms of partial derivatives of velocity with respect to xi such as by [99]. Also, as reported in many references, by using the Chapman-Enskog expansions, a multi-scaling expansion [18], to express EDF and its derivatives in the forms of small parameter ϵ, the conservation laws are derived. Here, based on the amended EDF, we can directly derive the conservation laws without additional higher order derivatives of the velocities, or the Chapman-Enskog expansions. It is more important, by adding the terms of the partial derivatives of EDF with respect to the streaming velocity and the instant kinetic energy, the resultant energy conservation equation includes the external energy generation source in it, which has not found in the current publications. The introduced internal mechanical energy in EDF involves the gradient of the velocity, which was not considered in the Boltzmann’s theory for dilute ideal gases. The amended theoretical EDF provides a complete theory on LBM.

# 4. Improved lattice Boltzmann method

The sections 2 and 3 confirm the following key points: 1) the amended EDF satisfies the Boltzmann’s H theorem and the three conservation laws in continuum mechanics, so that it is the theoretical solution of BTE; 2) in EDF, there are three macroscopic parameters: mass density, mean velocity and internal energy, which respectively are functions of time and space point, therefore at different points in the time-space frame, the corresponding EDFs generally are different; 3) if we know the three macroscopic parameters at a point in the time-space frame, we will know the corresponding EDF at the same point, the solution at this point of BTE; 4) in a reverse case, if we obtain the solution of BTE at a point, we will know the corresponding EDF, so that the three macroscopic parameters of continuum mechanics can be derived from the moment equations.

The aim of LBM is to obtain the solution of BTE at every point in the time-space frame, then to obtain the macroscopic parameters which are the solution of the problems in continuum mechanics. To reach this aim, the discrete BTE is necessary.  Here, based on the amended EDF, we develop an improved LBM, of which the main improvement is the total internal energy as a macroscopic parameter required to be updated in each time step.

## 4.1 Discrete lattice Boltzmann equation

It has been shown that discrete LBE can be obtained from the continuous BTE, see for examples, [29, 30-32]. For a single particle I, the BGK form of the continuous BTE [13,34,35] can be modified as

∂∂t+vi∂∂xi+vi∂∂vi+ϑK∂∂0.5v∙vfI=-1τfI-f0I,                                  (4-1.1)

in which, a new term involving energy as shown in Eq. 3-1.1 is added, and the amended EDF f0I for 3-D case is given in Eq. 2-4.13, when using the total internal energy per unit particle ε defined in Eq. 2-4.1, it is

f0Iρ,ε,ui,vi=ρ(4πε3)32exp-3(vi-ui)(vi-ui)4ε,                                         (4-1.2)

which is the function of the particle streaming velocity vi and the three macroscopic variables: mass density ρ, mean velocity ui and internal energy ε. The energy parameter including the thermal and mechanical ones is defined as

ε=3T/2=3kT/2,                 for ideal gases and fluids,3ψ2,                  for non-thermal viscous fluids, 3(kT+ψ)/2,     for thermal viscous fluids,                          (4-1.3)

which is applied to the ideal gases / fluids, the non-thermal viscous fluids, and the thermal viscous fluids, respectively. The dimension of the internal energy per unit particle / mass is Nmkg=(ms-1)2, so that the ratio of the kinetic energy over the internal energy,  0.5u∙uε, is non-dimensional parameter. For example, in the ideal gas case, we have

3u∙u2ε=3ρu∙u3ρkT=pu∙uC2p=M2,          M=uC=ukT,                                   (4-1.4)

where we have used Eq. 2-1.1 for the definition of the gas pressure, and the relationship  p= ρC2  between the dynamic pressure p and the sound speed C for the barotropic fluid [113-115], and M is the Mach number.

Making a time integration of Eq. 4-1.1 from time t to t+δt=t+1, we obtain the change of the EDF from the original equilibrium state (xi,vi,t) to its state (xi+eIi,vi,t+1) due the velocity change eIi, therefore we have

fIxi+eIi,vi,t+1-fIxi,vi,t=-1τffI-f0I-FI-ΘI,                               (4-1.5)

τf=τδt,       vi×δt=1∂fI∂vi=FI=FIi∂fI∂vi,   FIi=vI,      ϑK×δt=1∂fI∂0.5v∙v=ΘI.         (4-1.6)

Here, vI denotes the particle acceleration or the particle force per unit mass, that is assumed as a constant during the small time-period δt. The velocities eIi at the nodes I are referred to as the microscopic velocities. The force  FI consists of the particle internal interaction force and the possible external force acted at the particle. If there is no external forces, the internal interaction forces between the two particles satisfy the third Newton’s law, so that its summation over the total particles vanishes. Also, the term ΘI is the contribution from energy generation rate ϑK, which vanishes if no external energy generation source in the problem.

Using Eqs. 2-4.14~15 by the summations over particle I with velocity vIj replaced by the velocities eIj, and n=ρ, as well as noting ∂fI∂0.5v∙v=-1.5fIε and ∂fI∂vi=-1.5fIvi-uiε=-3fIvi-ui2ε, the following moment equations can be obtained,

IFI=VFi∂f∂vidV=0,                                                                (4-1.7)

IΘI=VϑK∂fI∂0.5v∙vdV=-V3ϑKfI2εdV=-3ρϑ2ε,

IFIeIj=VFi∂f∂vieIjdV=-3ρFj2ε,                                                  (4-1.8)

IΘIeIj=VϑK∂fI∂0.5v∙veIjdV=-V3ϑKfI2εeIjdV=-3ρϑ2εuj ,

IFIeIjeIr=VFi∂f∂vieIjeIrdV=-3ρ2ε(Fjur+Fruj),                                     (4-1.9)

IΘIeIjeIr=VϑK∂fI∂0.5v∙veIjeIrdV=-3ρϑ2ε(2ε3δjr+uruj),

IFIeIjeIreIl=VFi∂f∂vieIjeIreIldV=-3ρ2εFjTδrl+urul+FrTδjl+ujul+FlTδrj+uruj.  (4-1.10)

IΘIeIjeIreIl=VϑK∂fI∂0.5v∙veIjeIreIldV=-ρϑujδrl+urδjl+ulδjr+32εujulur.

It should be noticed that the force Fj and the energy generation rate ϑ respectively are the external force and the external energy generation rate per unit mass of the medium, since the internal interaction ones between the two particles are governed the third Newton’s law, so that the summations cancel them.

The exponential function can be represented in the power series

ex=1+x+x22!+x33!+⋯,                                                            (4-1.11)

of which, the radius of convergence is infinite. Approximating to the second order of  u2ε, we have

e-3(v-u)∙(v-u)4ε=e-3v∙v4εe6v∙u-3u∙u4ε=e-3v∙v4ε(1+3v∙u2ε+9v∙u28ε2-3u∙u4ε),                              (4-1.12)

from which, Eq. 4-1.2 can be approximated as

f0Iρ,ε,ui,vI=f0I2ε3, f0I=ρwI1+3vI∙u+9vI∙u22-3u∙u2,          u=u2ε ,               (4-1.13)

in which, u denotes the non-dimensional macroscopic velocity, and  vI=vI/2ε is a non-dimensional microscopic velocity at node I, and the non-dimensional weight wI absorbs the rest coefficient. The weight can be determined by the set of velocities of the scheme based on the moment equations. It should notice that 2ε-3 of dimension ms-1-3  is used for the volume element dVof the velocity space being non-dimensional one. Therefore, we may write Eqs. 2-4.14~15 in the following forms by means of non-dimensional velocities

zero order :         If0I=ρ,                                                (4-1.14)

1st order :           If0I(vIj-uj)=0,

2nd order :         If0IvIj-ujvIk-uk=ρδjk/3,

3rd order :         If0IvIj-ujvIk-ukvIl-ul=0,

4th order :       If0IvIj-ujvIk-ukvIl-ulvIl-ul=5ρδjk/9.

and

zero order :     If0I=ρ,                                                   (4-1.15)

1st order  :       If0IvIi=ρui,

2nd order :        If0IvIivIj=ρ(δij/3+uiuj),

3rd order :   If0IvIivIjvIl=ρ(ulδij+uiδlj+ujδil)/3+ρuiujul,

4th order :  IvIivIjvIlvIl=5ρδij9+ρ7uiuj+ululδij3+ρuiujulul.

It may be necessary to mention that according to the tensor rule [113-115], we have

δii=1,      1-D case,2,      2-D case3,      3-D case,,                                                          (4-1.16)

so that from Eq. 2-4.15, the energy conservation equations are

If0IvIivIi= ρ(2ε/3+uiui),      1-D case, ρ(4ε/3+uiui),      2-D case ρ(6ε/3+uiui),     3-D case.,     If0IvIivIi= ρ(1/3+uiui),      1-D case, ρ(2/3+uiui),      2-D case ρ(3/3+uiui),     3-D case.,    (4-1.17)

In the history, the approximated form of the original EDF with the DnQb models was developed by Qian and his colleagues [25,26,63].  Here, the number n =1,2,3 denotes the dimension of problem, while b indicates the node number of the scheme. To obtain the weights wI, He & Luo [30] used a third Hermite formula to approximate the integrals of the moment equation, and Abe [29] assumed wIhaving a simple truncated functional form based on eI. More generally, the Gaussian quadrature can be adopted to determine the weights for the exact values of the moment integrals [88], which is supported by the fundamental theorem [89].

It should be noticed that the moment equations in Eqs. 4-1.14~15 do not include the force effect given in Eq. 4-1.5. If there are no external forces, these moment equations are valid, but if there exist some external forces, the force effect in Eq. 4-1.5 must be included.

- b)

Fig. 3 a) D2Q9 scheme for 2-D problems, b) D3Q19 scheme for 3-D problems.[100]

## 4.2 Schemes of LBM

Using the approximate EDF in Eq. 4-1.13, i.e.

f0I=ρwI1+3vI∙u+9vI∙u22-3u∙u2,                                          (4-2.1)

the following schemes of LBM can be established in the DnQb forms.

### 4.2.1 1-D scheme D1Q3

For D1Q3, we choose three nodes (-1,0,1) with the corresponding velocity (-c,0,c), so that from Eq. 4-2.1, it follows

f0-1=ρw-11+-3cuc2+9cu22c4-3u22c2,     c=2ε,

f00=ρw0(1-3u22c2),                                                               (4-2.2)

f01=ρw11+3cuc2+9cu22c4-3u22c2.

Considering the symmetry of node locations, we have no reason more favor one of them, so that to choose the same weights at the two side nodes,  w1=w-1, then from the first two moment equations in Eq. 2-4.15, we obtain the weights w1=w-1=16 and w0=4/6.

### 4.2.2 2-D scheme D2Q9

For 2-D problem, we choose 9 nodes scheme shown by Fig. 3a), with the velocities and the weights at the nodes have been given in many publications, such as: [25,26,29, 30, 63,88,99], which are

v0=0,     w0 =49

vI=cosI-12π ,  sinI-12πc,     wI=19,      I=1~4,                                 (4-2.3)

vI=2cos2I-92π , sin2I-92πc,     wI=136,       I=5~8.

### 4.2.3 3-D schemes D3Q19

For 3-D problems, D3Q19 scheme chooses the 19 nodes as shown by Fig. 3b). The velocities and the weights at the nodes have been presented in the wide publications, which are

v0=0,     w0 =13,

vI=cosI-12π ,  sinI-12π,   0c,     wI=118,      I=1~4,

vI=0,  0,   cos(I-5)π c,     wI=118,      I=5,6,                                (4-2.4)

vI=2cos2I-132π , sin2I-132π, 0c,     wI=136,       I=7~10,

vI=2cos2I-212π , 0, sin2I-212πc,     wI=136,       I=11~14,

vI=20,cos2I-292π , sin2I-292πc,     wI=136,       I=15~18.

## 4.3 Generalised forces

In the publications, some force models have been proposed. For example, reference [48] suggested an acceleration model to denote a fore

FI=-1-12τ(vI-u)∙aCS2f0,                                                     (4-3.1)

where a denotes the macroscopic acceleration of the particle, while reference [49] proposed a physical force model to distribute the physical force on the cell particles using distribution function in the form

FI=wI(vICS2+vI∙uvICS4-uCS2)∙F,                                               (4-3.2)

where F is the physical force vector defined by

F=∇ρCS2-ρ∇μ0-κ∇2ρ.                                                 (4-3.3)

Here μ0 is the chemical potential of the bulk energy of fluid, and κ denotes a gradient parameter relating the interface thickness of two-phase fluids.

In this developed method of the paper, the introduced terms of external force and energy generation resources are in Eqs 4-1.5~6 and the corresponding moment equations by Eqs. 4-1.7~10. Therefore, there is no need to use any above force models proposed in the literatures. Based on the moment Eqs. 4-1.7~10 and using non-dimensional streaming velocities  vI=vI2ε and uI=uI2ε, we define the following three generalised forces contributed by the external force and energy generation source for the three conservation equations used in subsection 4.4. They are

mass:       Fm=IFI+IΘI=-3ρϑ2ε=-3ρϑ ,                                   (4-3.4)

momentum:       Fjv=IFIvIj+IΘIvIj=-3ρFj2ε-3ρϑ2εuj=-3ρ(Fj+ϑuj),        (4-3.5)

energy:       Fjre=IFIvIjvIr+IΘIvIjvIr =-3ρFjur+Fruj-ϑδjr3+uruj,        (4-3.6)

where  Fj=Fj2ε  and   ϑ=ϑ2ε  respectively denote the non-dimensional external force and energy generation source intensity per unit mass of the material. For the force and energy generation source at a time or space point, they can be denoted using Delta function, for example, a force applied at point xa and at time tb can be denoted as

Fj= Fjδ(x-xa,  t-tb)2ε.                                                        (4-3.7)

## 4.4 Implementation

For programing, LBM equation with its characteristic time τ concerning viscosity  of the fluid to be chosen by the problems, can be generally written as

fIxi+vIi,t+1-fIxi,t=-1τfI-f0I-FI-ΘI,                                 (4-4.1)

where FI and ΘI denote the force and energy change rate, including the internal and external ones, per unit mass at point I. For example, the kinematic viscosity has been used in the literatures by

ν=2τ-16(∆x)2∆t=2τ-16C2∆t.                                                      (4-4.2)

In the implementation processes published in most literatures, the collision step is before the streaming one. It has been noticed that in some publications, such as [41], the streaming step is arranged before the collision step, which are: a) streaming move x to x+vI to obtain a distribution function fI*, then update the macroscopic parameters and the equilibrium distribution function using the moment equations; b) replacing fI by fI* in Eq. 4-4.1 to collision to obtain fIx+vI,t+1. Here we describe this process the details as follows.

Streaming step: to obtain the distribution function by using Eq. 4-2.1, i.e.

fI=ρwI1+3vI∙u+9vI∙u22-3u∙u2,  u=u2ε , vI=vI2ε,

fIx+vI,t=fI(*).                                                                      (4-4.3)

Macroscopic variables update calculations: based on fI(*) and the following conservation laws with no external force included,

Mass density                      IfI(*)=ρ(*),

Velocity                            IfI(*)vIi=(ρui)(*),                                                                                   (4-4.4)

Internal energy                 IfI(*)vIivIj=(δij3ρ+ρuiuj)(*),

IfI(*)vIivIi=(d3ρ+ρuiui)(*),   d=1,2,3,  dimension number.

Equilibrium distribution functions based on macroscopic variables (*)

f0Ix+vI,t=   f0I(*) .                                                                     (4-4.5)

Collision step: to obtain the distribution function

fIx+vI,t+1=fI(*)+1τf0I(*)-fI(*)=fI(ϑ+1).                                 (4-4.6)

Final macroscopic variables calculations: based on the distribution function fI(ϑ+1) and the conservation laws including the generalised external force effect given in Eqs 4-3.4~6, i.e.

Mass:                                      IfI(ϑ+1)=ρ(ϑ+1)-Fm(ϑ),

Velocity:                                 IfI(ϑ+1)vIi=(ρui)(ϑ+1)-Fiv(ϑ)                                                        (4-4.7)

Internal energy:

IfI(ϑ+1)vIivIj=(δij3ρ+ρuiuj)ϑ+1-Fije(ϑ),

IfI(ϑ+1)vIivIi=(d3ρ+ρuiui)(ϑ+1)-Fiie(ϑ).

Here the values at the original position x,t marked by (ϑ), since generalised forces in Eq. 4-3.4~5 are the ones at (ϑ), which involves the physical macroscopic quantities only.

## 4.5 Matrix equations for implementation process

### 4.5.1. Implementation steps in matrix form

To investigate the performance of a scheme, it is convenience to adopt the matrix notations to express the implementation processes. For a convenience of notations, we identify the quantities at the position x,t, the middle position after streaming x+vI,t and the last position x+vI,t+1 by the super-indexes ϑ, (*) and ϑ+1, respectively. Therefore, the implementation process in matrix form is as follows.

Streaming: The distribution function vector f(*) at the space-time position x+vI,t is obtained, which is based on the macroscopic variable vector U(ϑ) consisting of the mass density, momentum, and total internal energy ε of the fluid at the position x,t and through the streaming operation by means of the following streaming matrix S,

f(*)=Sε(ϑ)U(ϑ),                                                                       (4-5.1)

S=wV,   V=10011v1iv1iv1j1⋮⋮⋮⋮1vNivNivNj1,   f= f0f1⋮fN,  ε(ϑ)= 100030-3δij209212d0-32δij ,

U(ϑ)=ρρuiρ(δij3+uiuj),  ui=ui2ε   ,   w=w00000w10000⋱0000wN,   d=1,2,3,

where d  denotes the dimension numbers, and the streaming velocity  vI is defined as

v0i=0,     vIi=vIi2ε,       I=1,2,⋯,N,                                                (4-5.2)

from which it follows that

fI(*)=LIε(ϑ)U(ϑ),           LI=wI1vIivIivIj1.                                   (4-5.3)

Updated calculations: The macroscopic displacement vector U(*) and internal energy ε(*) at position x+vI,t is calculated, which is based on the obtained distribution function vector f(*) and through a conservation transformation by means of the following conservation matrix V,

Vf(*)=U(*),                      V=11⋯10v1i⋯vNi0v1iv1j⋯vNivNj.                                         (4-5.4)

The row equations of the matrix Eq. 4-5.4 can be respectively written as

I=0NfI(*)=ρ(*),         I=0NvIi2ε(*)fI(*)=(ρui)(*),      I=0NvIivIj2ε(*)fI(*)=(ρuiuj)(*)+δij3ρ(*),       (4-5.5)

which, when Eqs. 4-5.1~3 substituted, respectively yields the following conservation laws of mass, momentum, and energy,

ρ(*)=Lρε(ϑ)U(ϑ),           Lρ=I=0NLI,                                                      (4-5.6)

(ρui)(*)=12ε(*)Lpε(ϑ)U(ϑ),          Lp=I=0NvIiLI,

12ε*LeεϑUϑ=ρuiui*+d3ρ*,    Le= I=0NvIivIiLI.

From the momentum equation, we obtain

2ε*(ρuiui)(*)=UTεTLpTLpε(ϑ)U(ϑ)ρ(*),                                               (4-5.7)

which, when substituted into the energy equation, gives

23ε(*)d=LeεϑUϑρ(*)-UTεTLpTLpεϑUϑρ*ρ*=UTεTEεϑUϑUTεTHεU,       E=LρTLe-LpTLp,       H=LρTLρ.        (4-5.8)

This equation provides a transformation to obtain the internal energy ε(*) from the original variables before the streaming.

From the macroscopic variable vectors obtained by Eq. 4-5.4, the equilibrium distribution function f0(*) can be derived by using Eq. 4-5.1 as follows

f0(*)=Sε(*)Vf(*) ,               f0T(*)=f00f01⋯f0N.                            (4-5.9)

Collision: From the results obtained by Eq. 4-5.1 and 4-5.9, the distribution function f(ϑ+1) at position x+vI,t+1 can be obtained by the following collision calculation

f(ϑ+1)=τ-1τf(*)+1τf0(*).                                                            (4-5.10)

Further-updated macroscopic variables: Using Eq. 4-5.4 and replacing the distribution function f(*) by the f(ϑ+1)  at position x+vI, t+1, and including the external forces as given in Eqs. 4-3.4~6, we obtain the macroscopic variables at position x+vI, t+1 as

Vfϑ+1=Uϑ+1+Fϑ,        U (ϑ+1)= Vf(ϑ+1)-Fϑ,     Fϑ=Fm(ϑ)Fiv(ϑ)Fije(ϑ).         (4-5.11)

### 4.5.2 Integrated transformation equation

The integrated transformation equation of LBM can be derived by combining the equations of each step given in sub-section 4.5.1. Substituting Eq. 4-5.10 into Eq. 4-5.11, we obtain

U(ϑ+1)=Vτ-1τf(*)+1τf0(*)-Fϑ,                                                     (4-5.12)

from which, when Eq. 4-5.1 and Eq. 4-5.9 substituted, it follows

U ϑ+1=SU(ϑ)-Fϑ,                                                                      (4-5.13)

S=τ-1τV+1τVV,            V=VSε(ϑ),                                                   (4-5.14)

in which the  ε* used in Eq. 4-5.6 marked (*) can be approximated by its original values εϑ, so that it is updated by

23εϑ+1d=UTϑEϑUϑUTϑHϑUϑ,          Eϑ=εTEεϑ,  Hϑ=εTHεϑ.                         (4-5.15)

After the detailed investigations on the matrices Eϑ and Hϑ, we have found that the former has a factor 2εϑ, but the later does not involve εϑ, therefore we can write

Eϑ=2εϑEϑ,      γ2=3dUTϑEϑUϑUTϑHϑUϑ  ,         γ=ε(ϑ+1)ε(ϑ) ,                            (4-5.16)

where γ is called as the ratio of internal energy change from the step ϑ to ϑ+1.

## 4.6 Performance investigations

### 4.6.1 Characteristic matrix

The performance of the scheme is governed by the matrix S, which is essentially determined by the matrix V combining the streaming matrix in Eq. 4-5.1 and the updated matrix V in Eq. 4-5.4 of the scheme. Therefore, this matrix is called as the characteristic matrix of the scheme. Using Eqs. 4-5.1, 4-5.4 and 4-5.14, we can obtain the matrix V in the form,

V=VSεϑ=11⋯10v1r⋯vNr0v1rv1s⋯vNrvNsw10011v1iv1iv1j1⋮⋮⋮⋮1vNivNivNj1100030-3δij209212d0-32δij                          (4-6.1)

=11⋯10v1r⋯vNr0v1rv1s⋯vNrvNsw1+d20-32δij1+d2-32v1iv1jδij3v1i92v1iv1j-32δij⋮⋮⋮1+d2-32vNivNjδij3vNi92vNivNj-32δij

=1+d2-32kijδij3pi92kij-32δijpr(1+d2)-32krijδij3kri92krij-32prδijkrs(1+d2)-32krsijδij3krsi92krsij-32krsδij,

of which, during the matrix multiplications, we have introduced the following definitions

I=0NwI=1,  I=0NwIvIi=pi,  I=0NwIvIivIj=kij,   I=0NwIvIrvIs=krs,

I=0NwIvIrvIivIj=krij,                 I=0NwIvIrvIsvIivIj=krsij.                                   (4-6.2)

As discussed before, geometrically, the weights of the schemes DnQm are center-symmetrical with the center streaming velocities v0i=0, but the streaming velocities at non-central nodes are anti-center-symmetrical. As a result of this, we have  pi=0=krij, so that the characteristic matrix in Eq. 4-6.1 reduces to

V=V10V130V20V310V3,                                                                  (4-6.3)

V1=1+d2-32kijδij,       V2=3kri,       V3=92krsij-32krsδij,    V13=92kij-32δij,    V31=krs(1+d2)-32krsijδij.

For different definitions of streaming velocity, the values of above terms are given as follows.

a) vIi=2εβIi,  βIi denotes the unit directional vector at point I.

kij=I=0NwIβIiβIj,        krs=I=0NwIβIrβIs,      kri=I=0NwIβIrβIi,     krsij=I=0NwIβIrβIsβIiβIj.      (4-6.4)

b) vIi=βIi,

kij=I=0NwIβIiβIj2ε,       krs=I=0NwIβIrβIs2ε,      kri=I=0NwIβIrβIi2ε,      krsij=I=0NwIβIrβIsβIiβIj4ε2.      (4-6.5)

It should be noted that for case a), the change of internal energy 2ε is included in the matrix V. Therefore, for the problems that involves the internal energy changes not neglected, the matrix V should be updated for each cell. In the case b), the internal energy is shown in the equation, and it can be directly updated, if necessary, in simulations.

### 4.6.2 Eigenvalues of characteristic matrix

For 3-D problems, the displacement vector defined is a 10-row vector: 1 mass density, 3 velocity components and 6 components of symmetrical tensor uiuj, so that the characteristic matrix in Eq. 4-6.3 is a 10×10 matrix, and for 1-D and 2-D problems they are respectively a 3×3 and a 6×6 matrix. Generally, the eigenvalue analysis of the matrix with higher order than 3×3 cannot be done by hand, but for the type of matrix in Eq. 4-6.3 can be analyzed as follows.

In 3-D case, the displacement vector and the characteristic matrix in Eq. 4-6.3 can be respectively written as

U=ρρu1ρu2ρu3ρ13+u1u1ρu1u2ρu1u3ρ13+u2u2ρu2u3ρ13+u3u3,      V=V1000V13112V13122V1313V13222V1323V13330V211V212V2130000000V221V222V2230000000V231V232V233000000V3111000V311112V311122V31113V311222V31123V31133V3112000V312112V312122V31213V312222V31223V31233V3113000V313112V313122V31313V313222V31323V31333V3122000V322112V322122V32213V322222V32223V32233V3123000V323112V323122V32313V323222V32323V32333V3133000V333112V333122V33313V333222V33323V33333.

(4-6.6)

We can write the characteristic matrix in a block form

V=A4×4B4×6C6×4D6×6,                                                          (4-6.7)

where each block matrix consists of the elements at the corresponding positions identified by the line and row subscripts in the matrix V in Eq. 4-6.6.  The characteristic equation for the eigenvalues of the matrix V is given by the following equation

V-κI10×10=A4×4-κI4×4B4×6C6×4D6×6-κI6×6=0.                               (4-6.8)

Using the Schur’s determinant identity [121], the determinant in Eq. 4-6.8 is calculated as

A4×4-κI4×4≠0,      V-κI10×10=A4×4-κI4×4D6×6,                   (4-6.9)

D6×6=D6×6-κI6×6-C6×4A4×4-κI4×4-1B4×6,

or

D6×6-κI6×6≠0,      V-κI10×10=D6×6-κI6×6A4×4,                    (4-6.10)

A4×4=A4×4-κI4×4-B4×6D6×6-κI6×6-1C6×4.

From this result, the solutions of Eq. 4-6.8 can be obtained by vanishing one of the following two lower order determinants,

A4×4=0,                     D6×6=0.                                                 (4-6.11)

This analysis provides a means to derive the eigenvalues of the characteristic matrix to analyze the performance of a scheme. However, although the two determinant equations are different orders, the derived algebraic equation with same order of eigenvalue κ,  so that it will not reduce the calculation tasks. It should be noted that eigenvalues of the characteristic matrix are the function of the internal energy of each simulation transformation.

### 4.6.3 Performance analysis based on eigenvalues of characteristic matrix

The matrix V is real matrix, of which the eigenvalues can be real or complex. If there exists a complex eigenvalue, its conjugate complex is also a complex eigenvalue of the matrix.

Eigenvalues and eigenvectors with their orthogonality

The eigenvalues and the corresponding eigenvectors are governed by the following eigenvalue equation

VΨ=κΨ,              V-κI=0.                                               (4-6.12)

Generally, we can represent the eigenvalue equation in the matrix form

VΨ=ΨΛ,    Ψ=ψ1⋯ψ10,   Λ=diagκ1,⋯,κ10,                        (4-6.13)

where  κI and ψI denote the I-th eigenvalues and the corresponding eigenvectors.  If the eigenvalues are different, the eigenvectors are independent, so that the matrix Ψ-1 exists.  As the result of this, we have

Ψ-1VΨ=Λ,        V=ΨΛΨ-1,    VV=ΨΛΨ-1ΨΛΨ-1=ΨΛ2Ψ-1,   V2Ψ=ΨΛ2 ,         (4-6.14)

which implies the eigenvalue of matrix V2 is the square of the eigenvalue of matrix V and the corresponding eigenvector is same as the one of matrix V. Therefore, we have

S=τ-1τΨΛ+1τΨΛ2Ψ-1.                                                         (4-6.15)

Amplifying factor and numerical damping

The independent eigenvectors of the characteristic matrix V span a complete characteristic space in which the displacement vector can be expressed as

U(ϑ)=Ψq(ϑ),        U(ϑ+1)=Ψq(ϑ+1),                                               (4-6.16)

where q denotes a displacement vector in the characteristic space. From this expansion, when substituted into Eq. 4-5.13, it follows

Ψq(ϑ+1)=SΨq(ϑ) ,                                                              (4-6.17)

where the external force term has been excluded due to it is not affected the performance of the characteristic matrix. Substituting Eq. 4-6.15 into Eq. 4-6.17, we obtain

Ψq(ϑ+1)=τ-1τΨΛ+1τΨΛ2q(ϑ) ,                                            (4-6.18)

which, when multiplying both side by Ψ-1, gives

q(ϑ+1)=τ-1τΛ+1τΛ2q(ϑ) .                                                    (4-6.19)

Physically, this result implies that the performance of the scheme is determined by the eigenvalues of the characteristic matrix of the scheme.

Generally, we may assume the eigenvalues of the scheme consist of conjugate complex numbers, since the characteristic matrix is real, therefore we have

Λ=diag(κ1,κ1*,⋯),                                                                 (4-6.20)

where  κJ is complex with its conjugate one marked by *. The two complex eigenvalues can be expressed in the form

κJ=κJe-iϕJ,        κJ*=κJeiϕJ,    J=1,2,3,4,5.                                       (4-6.21)

Here κJ and ϕJ denote the module and phase angle of the complex eigenvalue, respectively. Using these notations, we can re-write Eq. 4-6.19 in the form

q(ϑ+1)=Aq(ϑ) ,     A=τ-1τΛ+1τΛ2.                                        (4-6.22)

Here A is called as a complex amplifying factor matrix, which produces the amplitude amplification and the phase shift. The phase shift implies a damping effect, and the damping factor vanishes if all eigenvalues are all real. For 1-D problems, its characteristic matrix a 3×3 real matrix, so that it must have at least one real eigenvalue.

# 5. Examples

The key contribution of this paper is to include the internal energy parameter in the amended theoretical EDF, which allows the conservation laws to be obtained from the amended BTE. To illustrate the internal energy parameter effect, we consider the following hand-workable examples which can clearly explore the essential physical characteristics and avoid non-essential numerical treatment.

## 5.1. Example 1: the performance of scheme D1Q3

For this scheme, we choose three nodes (0,-1,1) with the corresponding streaming velocity (0,-v,v) and the weights w1=w-1=16 and w0=4/6.

### 5.1.1 Streaming velocity vIi=2εβIi

Characteristic matrix and its eigenvalues

Using the method given in sub-section 4.6.1.1~2 and considering the case of no external forces, we can obtain the following matrices for this D1Q3 scheme. For this 1D problems, we have the values

d=1,  i=j=1=r=s,                                                               (5-1.1)

from which, when substituted into Eq. 4.6.3, it follows the characteristic matrix of the system is

V=V10V130V20V310V3,                                                                  (5-1.2)

V1=32-32k11,     V2=3k11,     V3=92k1111-32k11,      V13=92k11-32,       V31=32k11-32k1111.

Adopting the streaming velocity form shown in Eq. 4-6.4, we obtain

v=2ε,          β0=0,     β-1=-1,    β1=1,       k11=1/3=k1111,     V=I .     (5-1.3)

The eigenvalues and eigenvectors of the matrix V can be obtained as

κ1=1,  ψ1=100;  κ2=1,   ψ2=010;  κ3=1,   ψ3=001;     Λ=I,    Ψ=I= Ψ-1.      (5-1.4)

Amplifying matrix

The amplifying matrix of the transformation of the scheme in Eq. 4-6.22 is

A=I.                                                                                     (5-1.5)

Final transformation

From Eq. 4-6.12 and Eq. 4-6.16, we have the transformation

U(ϑ+1)=U(ϑ),                                                                            (5-1.6)

that is

100012ε00012ε(ϑ+1)ρρuρ(2ε3+u2)(ϑ+1)= 100012ε00012ε(ϑ)ρρuρ(2ε3+u2)(ϑ),               (5-1.7)

from which, when pre-multiplying the inverse matrix of the first matrix on the left-hand side, it follows

ρρuρ(2ε3+u2)(ϑ+1)= 1000γ000γ2(ϑ)ρρuρ(2ε3+u2)(ϑ),            γ=ε(ϑ+1)ε(ϑ),                  (5-1.8)

where the ratio γ of internal energy change is given by Eq. 4-5.16.

### 5.1.2 Streaming velocity vIi=βIi

Characteristic matrix and its eigenvalues

In this case, from Eq.4-6.5, we obtain

k11=16ε,         k1111=112ε2,                                                     (5-1.9)

so that the characteristic matrix of the system is

V=V10V130V20V310V3,                                                                  (5-1.10)

where

V1=32-14ε,     V2=12ε,     V3=38ε2-14ε,      V13=34ε-32,       V31=14ε-18ε2.

The characteristic equation of the system is

V2-κV1-κV3-κ-V13V31=0.                                  (5-1.11)

The eigenvalues of this equation are

κ1=V2,  ψ1;        κ2=V1+V3+∆2,   ψ2;      κ3=V1+V3-∆2,   ψ3;      ∆=V1-V32+4V13V31,    (5-1.12)

of which,  κ1 is real, and the last two ones are real if ∆≥0, otherwise complex. Generally, as the case of complex eigenvalues, we may express these two eigenvalues as

κ2,3=κe±iϕ.                                                  (5-1.13)

Amplifying matrix

The amplifying matrix of the transformation of this scheme in Eq. 4-6.22 is now given by

A=A1000A2000A3,   AI=τ-1τκI+1τκI2,   I=1,2,3.                         (5-1.14)

As mentioned before, in the complex eigenvalue cases, the amplifying matrices produce some damping effects causing the phase shifts in each simulation step.

## 5.2 1-D constant incompressible flow by D1Q3 scheme

We investigate a 1-D constant incompressible flow by the scheme D1Q3 of the weights w1=w-1=16, w0=4/6 with three nodes (0,-1,1), and the corresponding streaming velocity (0,-2ε,2ε), of which the integrated transformation is given in subsection 5.1.1, based on which we have

U ϑ+1=IU(ϑ)=U(ϑ),       γ=ε(ϑ+1)ε(ϑ)=1.                                     (5-2.1)

which is the theoretical solution of the problem.

x

p0,   u0

O

A=1

pL,   uL

L

Fig.4 1-D stable compressed gas flow through a tube of unit cross-section area.

## 5.3 1-D high pressure compressible gas flow in a tube of unit cross-section area

As shown in Fig. 4, a horizontal tube of length L and unit cross-section area A=1 locates along O-x axis. We assume that the left end of the tube connected to a very high pressed gas tank of pressure p0 keeping a prescribed constant, and its right end at x=L connecting to the atmospheric pressure pL, so that the difference of pressures at the two ends produces a flow in the tube. The governing equations and theoretical analysis of the problem are discussed for two cases.

5.3.1 Transient analysis before stable flow

Considering the fluid is compressible and barotropic, we respectively have its conservation equations of mass and momentum, and state equation

∂ρ∂t+∂(ρu)∂x=DρDt+ρ∂u∂x=0,                                                              (5-3.1)

ρ∂ρ∂t+u∂u∂x=-∂p∂x,                                                                    (5-3.2)

dpdρ=c2:    p=c2ρ-ρL+pL,       ρ=ρL-pLc2,                                        (5-3.3)

where c is a speed of sound, and ρ  denotes the mass density of the gas with zero pressure. The initial and boundary conditions are

ux,0=0,   px,0=pL,    ρx,0=ρL,           x∈(0,L),                              (5-3.4)

p0,t=p0,     ρ0,t=ρ0,    pL,t=pL,        ρL,t=ρL.                           (5-3.5)

To find the internal energy per unit mass of the gas at a point, we define the specific volume φ=1ρ,  the volume of unit mass, and use the mass conservation Eq. 5-3.1 to obtain

dρρ=-dφφ=-ρdφ=-∂u∂xdt.                                                  (5-3.6)

Using Eq. 2-2.3, we obtain the physical internal energy per unit mass

ψx,t=-0tpρ∂u∂xdt=ρρpρ2dρ=-φφpdφ,                                    (5-3.7)

ψx,0=ρρLpρ2dρ=-φφLpdφ,                                                 (5-3.8)

and then from Eq. 4-1.3, the statistical internal energy per unit mass is

εx,t=3ψx,t2,        εx,0=3ψx,02 .                                              (5-3.9)

which is a function of x and t.

The prescribed pressures at two ends of tube produce the generalised forces at the nodes of two ends for simulation used in LBM, and from Eqs. 4-3.4~6 they are

F0ϑ=F0m(ϑ)F0v(ϑ)F0e(ϑ)=03ρ0p06ρ0p0u0(ϑ),            FLϑ=FLm(ϑ)FLv(ϑ)FLe(ϑ)=03ρLpL6ρLpLuL(ϑ).                 (5-3.10)

Here the variable () implies its non-dimensional one by dividing with 2εI that involves step ϑ.

5.3.2 Stable flow

When the flow reaches stable, the variables of motion are independent of the time, and Eqs 5-3.1~2 reduces to

∂(ρu)∂x=0,                           ρu∂u∂x=-∂p∂x.                                           (5-3.11)

The integrations of Eq. 5-3.11 along the tube with respect to x yield

ρ(x)u(x)=ρ0u0=ρLuL=constant,                                        (5-3.12)

p(x)=p0-ρ0u0[u(x)-u0]=p0[1-u0c2[u(x)-u0]=p0{1-M0[M(x)-M0]},               (5-3.13)

where the Mach numbers M(x)=u(x)c and M0=u0c. Using Eq. 5-3.3, and the prescribed pressures at the two tube ends, we obtain

p0=c2(ρ0-ρL)+pL,     p(x)u(x)=p0u0=pLuL=constant                    (5-3.14)

p0-pL=ρ0u0[uL-u0]=M0[ML-M0],                                  (5-3.15)

from which, it gives

ux2c2=Mx2=p0+(c2-1)pLp+(c2-1)pL ,        uL2c2=ML2=p0+(c2-1)pLpL+(c2-1)pL=1+p0-pLpLc2.                    (5-3.16)

which shows that the flow is supersonic, since p0>pL. Now we can obtain the speed, pressure, and mass density of the fluid at any point of tube. In this case, the internal energy in Eq. 5-3.9 will be function of x only.

5.3.3 Stable solution by LBM

Now we can use the LBM scheme D1Q3 to deal with the stable problem with the streaming velocity (0,-2ε,2ε) , which is a function of cell position. We have known from the performance study of this scheme that its characteristic matrix Eq. 5-1.3 and amplifying matrix Eq. 5-1.5 are unit matrix, i.e., V=I=A, therefore the transformation U(ϑ+1)=U(ϑ) in Eq. 5-1.6 confirms the initial stable values of variables obtained by Eqs. 5-3.11~16 is stable solution.

The aim of the hand-workable examples given herein is mainly to show that the introduced internal energy variable into LBM equation plays an important role dealing with problems with energy changes in the cell, which caused the difficulty of current LBM theory as mentioned in references cited in introduction of the paper.  Therefore, considering page limits, we do not intend to do some numerical examples but leave it for readers to practice it for further developing the theory and proposed method.

# 6. Conclusion and discussion

To summary the theoretical analysis, we conclude the following contributions of the paper. a) The short introduction on LBM provides the historically important information, which should confirm, based on the author’s check, that there have not been found any references presenting the same idea as given in the paper. b) The amended theoretical EDF derived by the H-theorem with Lagrangian multiplier approach includes the mass density, mean velocity, and total internal energy of fluids as three macroscopic parameters, from which the three conservation laws can be directly derived from the BTE without additional small parameter expansions. c) The improved LBM, requiring the internal energy parameter to be updated in each simulation step for general cases, and the updated non-dimensional streaming velocity provides a means to simulate more complex flow problems concerning obvious energy changes in high-speed, compressible flows. The performance study approach is formulated. d) The modified differential BTE includes a new term concerning energy variation allowing external forces and external energy generation source to be considered in the method. e) The hand-workable examples theoretically illustrate the essential characteristics and confirm the proposed improved LBM with the performance study.

The paper is a theoretical document, which have not given complex practical numerical examples, due to pages limited. Author wishes the interested readers may follow the proposed method numerically to tackle some engineering problems from which to further develop this new improved method benefiting to sciences and engineering advances.

Acknowledgement

Author acknowledges the School of Naval Architecture & Ocean Engineering, HUST, providing the finance support managed by Guoxiang Hou, enabling author to visit HUST to tackle LBM. Thanks also are given to Yuehong Qian of Soochow University for providing some references involved in the paper.

# References

- Landau, L.D., Lifshitz, E.M.: Mechanics. (3rd edn). Butterworth-Heinenann, Oxford (1981)

- Hardy,J., Pomeau, Y., De Pazzis, O.: Time evolution of a two-dimensional system, I. Invariant states and time correlation functions. J. Math. Phys. 14(2), 1746-1759 (1973)

- Hardy,J., Pomeau, Y., De Pazzis, O.: Time evolution of a two-dimensional classical lattice system. Phys. Rev Lett. 31, 276-259 (1973)

- Hardy, J, De Pazzis, O., Pomeau, Y.: Molecular dynamics of a lattice gas: transport properties and time correlation functions. Phys. Rev. A13, 1949-1961 (1976)

- Satoh, A.: Introduction to practice of molecular simulation. Elsevier, London, San Diego (2010)

- Huang, K.: Statistical mechanics. (2nd edn). Jhon Wiley & Sons, Chichester (1987, 1963).

- Lerner, R.G., Trigg, G.L.: Encylopaedia of physics (2nd ed.). VHC publishers, New York (1991)

- Arkeryd, L.: On the Boltzmann equation part I: existence. Arch. Rational Mech. Anal. 45 (1), 1-16 (1972)

- Arkeryd, L.: On the Boltzmann equation part II: the full initial value problem. Arch. Rational Mech. Anal. 45 (1), 17-34 (1972)

- Crank, J., Nicolson, P.: A practical method for numerical evaluation of solution of partial differential equations of the heat conduction type. Proc. Camb. Phil. Soc. 43(1), 50-67 (1947)

- Hirsch, C.: Numerical computation of internal and external flows, volume 1: fundamentals of numerical discretization. John Wiley & Sons, Chichester (1988).

- Inamuro, T., Sturtevant, H.: Numerical study of discrete-velocity gases Phys. Fluids 2, 2196-2203 (1990)

- Bhatnagar, P.L., Gross, E.L., Krook, M.: A model for collision processes in gases. I. Small amplitude processes in charged and neutral one-component systems. Physical Review 94 (3), 511-525 (1954).

- Wilson, G.: The life and times of cellular automata. New Scientist 44-47 (1988)

- Chopard, B., Droz, M.: Cellular automata modelling of physical systems. Cambridge University Press, London (1998)

- McNamara, G., Zanetti, G.: Use of the Boltzmann equation to simulate lattice-gas automata. Physical Review Letters 61, 2332-2335 (1988)

- Broadwell, J.E.: Study of rarefied shear flow by the discrete velocity method. J. Fluid Mech. 19, 401-414 (1964)

- Frisch, U., Hasslacher, B., Pomeau, Y.: Lattice-gas automata for the Navier-Stokes equation. Phys. Rev. Lett. 56 (14), 1505–1508 (1986)

- Frisch, U., D’Humieres, D., Hasslacher, B., Lallemand, P., Pomeau, Y., Rivet, J.P.: Lattice gas hydrodynamics in two and three dimensions. Complex Systems 1, 649-707 (1987)

- Higuera, F.J., Jimenez, J.: Boltzmann approach to lattice gas simulations. Europhys. Lett. 9, 663-668 (1989)

- Higuera, F.J., Succi, S., Benzi, R.: Lattice gas dynamics with enhanced collisions. Europhys. Lett. 9, 345-349 (1989)

- Qian, Y.H.: Lattice gas and lattice kinetic theory applied to the Navier-Stokes equations. PhD thesis, University Pierre et Marie Curie, Paris (1990)

- Chen, S., Chen, H.D., Martinez, D., Matthaeus, W.: Lattice Boltzmann model for simulation on magnetohydrodynamics. Physics Review Letters 67, 3376-3379 (1991)

- Chen, H., Chen, S., Matthaeus, W.H.: Recovery of the Navier-Stokes equation using a lattice-gas Boltzmann method. Phys. Rev. A 45, R5339-5342 (1992)

- Qian,Y.H., D’Humieres, D., Lallemand, P.: Lattice BGK models for Navier-Stokes equation. Europhysics Letters 17, 479 (1992)

- Qian, Y.H.: Simulating thermodynamics with Lattice BGK models. Journal of Scientific Computing 8, 231 (1993)

- Guo, Z., Shi, B., Wang, N.: Lattice BGK Model for Incompressible Navier-Stokes Equation. Journal Computational Physics 165, 288-306 (2000)

- Guo, Z., Shi, B., Zheng, C.: A coupled lattice BGK model for the Boussinesq equations. Int. J. Numer. Meth. Fluids 39, 325-342 (2002)

- Abe, T.: Derivation of the lattice Boltzmann method by means of the discrete ordinate method for the Boltzmann equation. J. Comp. Phys. 131, 241-246 (1997)

- He, X., Luo, L.S.: A priori derivation of the lattice Boltzmann equation. Phys. Rev. E 55, 6333-6336 (1997)

- He, X., Luo, L.S.: Theory of the lattice Boltzmann method: from the Boltzmann equation to the lattice Boltzmann equation. Phys. Rev. E 56(6), 6811-6817 (1997)

- He, X., Luo, L.S.: Lattice Boltzmann for the incompressible Navier-Stokes equation. J. Stat. Phys. 88, 927-944 (1997)

- Shan, X., He, X.: Discretization of the velocity space in the solution of the Boltzmann equation. Phys. Rev. Lett. 80, 65 (1998)

- Chu, C.K.: Kinetic theoretic description of the formation of a shock wave. Phys. Fluids 8, 12-21 (1965)

- Xu, K., Prendergast, K.H.: Numerical Navier-Stokes solutions from gas kinetic theory. Journal of Computational Physics 114, 9-17 (1994)

- Wolfram, S.: Cellular automation fluids, I. Basic theory. J. Stat. Phys. 45, 471-526 (1986)

- Lavallee, P., Boon, J.P., Noullez, A.: Boundaries in lattice gas flows. Physica D47, 233-240 (1991)

- Chen, S., Martinez, D., Mei, R.: On boundary conditions in lattice Boltzmann methods. J. Phys. Fluids 8, 2527-2536 (1996)

- Chen, S., Doolen, G.D.: Lattice Boltzmann method for fluid flows. Annual Rev. Fluid Mech. 30, 329-364 (1998)

- Succi, S.: The Lattice Boltzmann Equation for Fluid Dynamics and Beyond. Oxford University Press, Oxford. (2001)

- Bao, Y. B., Meskas, J.: Lattice Boltzmann Method for Fluid Simulations. appears at webpage https://www.math.nyu.edu (2011)

- Zou, Q., He, X.: Pressure and velocity boundary conditions for the lattice Boltzmann. J. Phys. Fluids 9, 1591-1598 (1997)

- Guo, Z. L., Zheng, C. G., Shi, B.C.: Non-equilibrium extrapolation method for velocity and pressure boundary conditions in the lattice Boltzmann method. Chinese Physics 11(4) 366-374 (2002)

- He, X., Doolen, G.: Lattice Boltzmann method on curvilinear coordinate systems: flow around a circular cylinder. Journal of Computational Physics 134, 306 (1997)

- Filippova, O., Hanel, D.: Grid refinement for lattice-BGK models. Journal of computational Physics 147, 219-228 (1998)

- Mei, R., Shyy, W.: On the finite difference-based Boltzmann method in curvilinear coordinates. Journal of Computational Physics 143, 426 (1998)

- Mei, R., Shyy, W., Yu, D., Luo, L.S.: Lattice Boltzmann method for 3-D flows with curved boundary. Journal of Computational Physics 161, 680-699 (2000)

- He, X., Shan, X., Doolen, G.D.: Discrete Boltzmann equation model for nonideal gases. Physical Review E 57, R13-R16 (1998)

- Lee, T., Lin, C.L.: A stable discretization of the lattice Boltzmann equation for simulation of incompressible two-phase flows at high density ration. Journal of Computational Physics 206 (1), 16-47 (2005)

- Elton, B.H., Levermore, C.D., Rodrigue, H.: Convergence of convective-diffusive lattice Boltzmann methods. SIAM J. Sci. Comp. 32, 1327-1354 (1995)

- Succi, S., Benzi, R., Higuera, F.: The lattice Boltzmann equation – a new tool for computational fluid dynamics. Physica D47, 219-230 (1991)

- Noble, D.R., Georgiadis, J.G, Buckius, R.O.: Comparison of accuracy and performance for lattice Boltzmann and finite difference simulation of steady viscous flow. Int. J. Numer. Meth. Fluids 23, 1-18 (1996)

- Sterling, J.D., Chen, S.: Stability analysis of lattice Boltzmann methods. J. Comp. Phys. 123, 196-206 (1996)

- Cao, N., Chen, S., Jin, S., Martinez, D.: Physical symmetry and lattice symmetry in lattice Boltzmann method. Phys. Rev. E 55, R21-24 (1997)

- Koelman, J.M.V.A.: A simple lattice Boltzmann scheme for Navier-Stokes fluid flow. Europhys. Lett. 15, 603-607 (1991)

- Nannelli, F., Succi, S.: The lattice Boltzmann equation on irregular lattices. J. Stat. Phys. 68, 401-407 (1992)

- Amati, G., Succi, S., Benzi, R.: Turbulence channel flow simulation using a coarse-grained extension of the lattice Boltzmann method. Fluid Dyn. Res. 19, 289-302 (1997)

- He, X., Luo, L.S., Dembo, M.: Some progress in lattice Boltzmann method, Part I nonuniform mesh grids. J. Comp. Phys. 129, 357-363 (1996)

- Benzi, R., Succi, S., Vergassola, M.: The lattice Boltzmann equation - theory and application. Phys. Rep. 222, 145-97 (1992)

- Succi, S., Benzi, R., Massaioli, F.: A review of the lattice Boltzmann method. International Journal of Modern Physics C4 (2), 409-415 (1993)

- Rothman, D.H., Zaleski, S.: Lattice gas models of phase separation: interfaces, phase transitions and multiphase flow. Rev. Mod. Phys. 66, 1417-79 (1994)

- Chen, S., Dawso, S.P., Doolen, G.D., Janecky, D.R., Lawniczak, A.: Lattice methods and their applications to reacting systems. Comp. Chem. Eng. 19, 617-646 (1995)

- Qian, Y.H., Succi, S., Orszag, S.A.: Recent advances in lattice Boltzmann computing. Annu. Rev. Comp. Phys. 3, 195-242 (1995)

- Premnath, K., McCracken, M., Abraham, J.: A Review of Lattice Boltzmann Methods for Multiphase Flows Relevant to Engine Sprays. SAE Technical Paper 2005-01-0996, (2005)

- Aidum, C.K., Clausen, J.R.: Lattice Boltzmann method for complex flows. Annual Review of Fluid Mechanics 42, 439-472 (2010)

- Seta, T.: Progress in Lattice Boltzmann Method. Japanese Journal of Multiphase flow 24(4), 427-434 (2010)

- Yin, X., Le, G., Zhang, J.: Mass and momentum transfer across solid-fluid boundaries in the lattice-Boltzmann method. Phys. Rev. E Stat. Nonlin. Soft. Matter. Phys. 86(2 Pt 2): 026701 (2012)

- Jahanshaloo, L., Pouryazdanpanah, E., Sidik, N. A. C.: A Review on the application of the lattice Boltzmann method for turbulent flow simulation, Numerical Heat Transfer, Part A: Applications. An International Journal of Computation and Methodology 64 (11), 938-953 (2013)

- Djenidi, L.: The lattice Boltzmann method and the problem of turbulence. AIP Conference Proceedings 1648, 030003 (2015)

- Che Sidik, N.A., Aisyah Razali, S.: A Review on Lattice Boltzmann Method for Numerical Prediction of Nanofluid Flow. International Review of Mechanical Engineering 7(7), 1269-1274 (2013)

- Che Sidik, N.A, Aisyah Razali, S.: Lattice Boltzmann method for convective heat transfer of nanofluids, Renewable and Sustainable Energy Reviews 38(C), pages 864-875, Elsevier, London (2014)

- Perumal, D.A., Dass, A.K.: A Review on the development of lattice Boltzmann computation of macro fluid flows and heat transfer. Alexandria Engineering Journal 54 (4), 955-971 (2015)

- Succi, S., Moradi, N., Greiner, A., Melchionna, S.: Lattice Boltzmann modeling of water-like fluids. Front. Phys. 16 (2014)

- Wang, J., Chen, L., Kang, Q., Rahman, S.S.: The lattice Boltzmann method for isothermal micro-gaseous flow and its application in shale gas flow: a review. International Journal of Heat and Mass Transfer 95, 94-108 (2016)

- Yoon, H., Kang, Q., Valocchi, A.J.: Lattice Boltzmann-Based Approaches for Pore-Scale Reactive Transport. Reviews in Mineralogy and Geochemistry 80 (1), 393–431 (2015)

- Liu, H., Kang, Q., Leonardi, C.R., Schmieschek, S.M.P., Narvaez Salazar, A.E., Jones, B.D., Williams, J.R., Valocchi, A.J., Harting, J.D.R.: Multiphase lattice Boltzmann simulations for porous media application: a review. Computational Geosciences 20(4), 777-805 (2016)

- Sharma, K.V., Straka, R., Tavares, F.W.: Lattice Boltzmann method for industrial applications. Ind. Eng. Chem. Res. 58, 16205-16234 (2019)

- W. Shao, J. Li, Review of Lattice Boltzmann Method Applied to Computational Aeroacoustics. Archives of Acoustics 44(2), 215–238 (2019)

- Carenza, L.N., Gonnella, G., Lamura, A., Negro, G., Tiribocchi, A.: Lattice Boltzmann methods and active fluids. European Physical Journal E 42, 1-83 (2019)

- Wang, H., Yuan, X., Liang, H., Chai, Z., Shi, B.: A brief review of the phase-field-based lattice Boltzmann method for multiphase flows. Capillarity 2(3), 33-52 (2019)

- Li, L., Lu, J., Fang, H., Yin, Z., Wang, T., Wang, R., Fan, X., Zhao, L., Tan, D., Wan, Y.: Lattice Boltzmann method for fluid-thermal systems: status, hotspots, trends and outlook. IEEE Access 8, 27646-27675 (2020)

- Gutowitz, H. (ed): Cellular automata: theory and experiment; proceeding s of a workshop sponsored by the centre for nonlinear studies. Los Alamos National Laboratory, Los Alamos, NM87545, USA, September 9-12, (1989).

- Wolf-Gladrow, D.A.: Lattice-gas cellular automata and lattice Boltzmann models, an introduction. Springer, Berlin Heidelberg, New York (2000)

- Succi, S.: Lattice Boltzmann equation for complex states of flowing matter. Oxford University Press, Oxford (2018)

- Sukop, M., Thorne, D.T.: Lattice Boltzmann Modelling: an introduction for geoscientists and engineers (1st edn). Springer, Berlin Heidelberg, New York (2006)

- Mohamad, A.A.: Lattice Boltzmann method: fundamentals and engineering applications with computer codes (1st & 2nd edns). Springer, Berlin Heidelberg, New York (2011 / 2019)

- Mohamad, A.A. (author), Yang, D. (translator): Lattice Boltzmann method: fundamentals and engineering applications with computer codes (in Chinese). Electronic Industry Press, China (2015)

- Guo, Z., Shu, C.: Lattice Boltzmann method and its applications in engineering (Advances in Computational Fluid Dynamics). World Scientific Publishing Company, London (2013)

- Stoer, J., Bulirsch, B.: Introduction to numerical analysis. Springer, Berlin Heidelberg, New York (1992).

- Huang, H., Sukop, M., Lu, X.: Multiphase lattice Boltzmann methods: theory and application, John Wiley & Sons, Chichester (2015)

- Kruger, T., Kusumaatmaja, H., Kuzmin, A., Shardt, O., Silva, G., Viggen, E.M.: The lattice Boltzmann method, principles and practice. Springer, Berlin Heidelberg, New York (2017)

- Montessori, A., Falcucci, G.: Lattice Boltzmann modeling of complex flows in engineering applications. Morgan & Claypool Publishers, San Rafael, CA, 94901, USA (2018)

- Akker, H. (Ed.): Lattice Boltzmann modeling for chemical engineering, Volume 55. Academic Press, London, New York (2020)

- Bolton, D.C., Schwartz, B., Shreedharan, S.: Lattice Boltzmann methods, pp1-55. appears at webpage https://www.ems.psu.edu (2020)

- He, Y., Wang, Y., Li, Q.: Lattice Boltzmann method: Theory and Applications (in Chinese). Science Press, Beijing (2009)

- Guo, Z., Zheng, Z.: Theory and applications of lattice Boltzmann method (in Chinese). Science Press, Beijing (2009)

- Wang, H., Li, X.: Applications of lattice Boltzmann method in partial differential equations of soliton waves (in Chinese). Science Press, Beijing (2017)

- Begum, R., Basit, M.A.: Lattice Boltzmann Method and its Applications to Fluid Flow Problems. Euro. J. Sci. Research 22, 216-231 (2008)

- Wagner, A.J.: A practical introduction to the lattice Boltzmann method. Department of Physics, North Dakota State University, Fargo, March 2008, at http://www.physics.ndsu.nodak.edu/wagner/LB.html.

- Chen, L.: Lattice Boltzmann for flow and transport phenomena. appears at webpage  https://gr.xjtu.edu.cn/web/lichennht08 (2018)

- Swift, M.R., Osborn, W.R., Yeomans, J.M.: Lattice Boltzmann simulation of nonideal fluids. Phys. Rev. Lett. 75, 830-833 (1995)

- Swift, M.R., Orlandini, S.E., Osborn, W.R., Yeomans, J.M.: Lattice Boltzmann simulation of liquid-gas and binary-fluid systems. Phys. Rev. E 54, 5041-5052 (1996)

- Nadiga, B.T., Zaleski, S.: Investigations of a two-phase fluid model. Eur. J. Mech. B/fluids 15, 885-896 (1996)

- He, X., Chen, S., Doolen, G.D.: A novel model for the lattice Boltzmann method in compressible limit. Journal of Computational Physics, Elsevier 146 (1), 282-300 (1998)

- Jacob, J., Malaspinas, O., Sagaut, P.: A new hybrid recursive regularized Bhatnagar-Cross-Krook collision model for lattice Boltzmann method-based large eddy simulation. J. Turbul. 19(11), 1051-76 (2018)

- Feng, Y., Boivin, P., Jacob, J., Sagaut, P.: Hybrid recursive regularized thermal lattice Boltzmann model for high subsonic compressible flows. J. Comput. Phys. 394, 82-99 (2019)

- Renard, F., Feng, Y., Boussuge, J.-F., Sagaut, P.: Improved compressible hybrid lattice Boltzmann method on standard lattice for subsonic and supersonic flows. Computers and Fluids 219, 104867 (2021)

- Shan, X.: Central-moment-based Galilean-invariant multiple-relation-time collision model. Phys. Rev. E 100(4), 043308 (2019)

- Wilde, D., Krämer, A., Küllmer, K., Foysi, H., Reith, D.: Multistep lattice Boltzmann methods: theory and applications. Int. J. Numer. Methods Fluids 90(3), 156-169 (2019)

- Saadat, M.H., Bösch, F., Karlin, I.V.: Lattice Boltzmann model for compressible flows on standard lattices: variable Prandtl number and adiabatic exponent. Phys. Rev. E 99(1), 013306 (2019)

- Feng, Y.-L., Tao, W.-Q., Sagaut, P.: Regularized thermal lattice Boltzmann method for natural convection with large temperature differences. Int. J. Heat Mass Transf. 125, 1379-91 (2018)

- Wissocq, G., Sagaut, P., Boussuge, J.F.: An extended spectral analysis of the lattice Boltzmann method: model interactions and stability issues. J. Comut. Phys. 380, 311-33 (2019)

- Fung, Y.C.: A first course in continuum mechanics (3rd edn). Prentice-Hall, London (1977)

- Xing, J.T.: Fluid-solid interaction dynamics, theory, variational principles, numerical methods, and applications. Academic Press, Elsevier, London (2019)

- Xing, J.T.: Fluid-solid interaction dynamics, theory, variational principles, numerical methods, and applications. Higher Education Press, Beijing (2021)

- Xing, J.T., Price, W. G.: A power flow analysis based on continuum dynamics. Proc. R. Soc. Lond. A 455, 401-435 (1999)

- Xing, J.T., Price, W. G.: The energy flow equation of continuum dynamics. In IUTAM Symposium on Statistical Energy Analysis, (eds. F. Fahy & W. G. Price), pp. 83-94. Kluwer academic publishers, Netherlands (1999)

- Xing, J.T.: Energy flow theory of nonlinear dynamical systems with applications, Springer, Berlin Heidelberg, New York (2015)

- Xing, J.T.: Generalised energy conservation law of chaotic phenomena. Acta Mech Sin 35(6), 1257-1268 (2019) http://doi.org/10.1007/s10409-019-00886-7

- Qin, L., Qin, H., Xing, J.T.: Energy flow characteristics of friction-induced nonlinear vibrations in a water-lubricated bearing-shat coupled system. Acta Mech Sin 37(4), 679-704 (2020) http://doi.org/10.1007/s10409-020-01047-x

- Zhang, F.: The Schur complement and its applications, Springer, Berlin Heidelberg, New York (2005)